S-403 (Inland ECDIS and Inland ECS Interoperability

Specification)

Draft 0 of Edition

Permission N°10/2024 Material from IHO publication S-98 S-100 ECDIS and Interoperability Specification Edition 2.0.0 – October 2025 is reproduced with the permission of the IHO Secretariat (Permission No 11/2026) acting for the International Hydrographic Organization (IHO), which does not accept responsibility for the correctness of the material as reproduced: in case of doubt, the IHO’s authentic text shall prevail. The incorporation of material sourced from IHO shall not be construed as constituting an endorsement by IHO of this product.

# CONTENTS

CONTENTS	iii

1	Introduction	8

2	References	9

2.1	Normative references	9

2.2	Informative references	10

3	Abbreviations and Notation	10

3.1	Abbreviations	10

3.2	Notation	10

3.3	Normative vs Informative within this document.	11

4	Background - System Concepts and Limitations	11

4.1	The concepts of ENC, ENDS and System Database	11

4.2	ECDIS concept, limitations, and challenges	11

4.2.1	Depth Related Functionality	13

4.3	Integrated Navigation System (INS) concept, limitations and challenges	13

5	User Interface Design	14

5.1	General principles	14

6	Data Layers	14

6.1	Data products and information layers	14

6.1.1	Basic product specifications and layers	14

6.1.2	Projected Data	15

6.1.3	Other data products	15

6.1.4	Data layers in Display Base and Standard Display	15

6.2	Mitigation of data overload	15

7	Portrayal Process, Catalogue Elements, and Alerting	15

7.1	Overview of the portrayal process	16

7.2	Elements of S-100 Portrayal	16

7.2.1	Pixmaps	16

7.2.2	Colour coding scheme	16

7.2.3	Symbols	16

7.2.4	Area fills	16

7.2.5	Line styles	16

7.2.6	Portrayal of Shared Edges.	17

7.2.7	Text	17

7.2.8	Style sheets	18

7.2.9	Display planes	18

7.2.10	Display (drawing) priorities	18

7.2.11	Viewing groups	18

7.2.12	Viewing group layers	18

7.2.13	Display modes	18

7.2.14	Rules	18

7.2.15	Context	18

7.2.16	Validity times	18

7.3	The Alert Catalogue	19

8	Display Organisation and Operation	19

8.1	Display of non-S-100 information	19

8.1.1	Distinguishing between official S-100 data and additional data	19

8.1.2	Supplemental display items	20

8.2	Priority of information	20

8.2.1	Priority layers	20

8.2.2	Radar priority	21

8.3	Displaying ECDIS updates	21

8.4	Display functions	21

9	General Rules for Symbols and Text	21

9.1	Symbol Specifications	21

9.2	Reproduction of lines, symbols and text	22

9.2.1	Introduction	22

9.2.2	Minimum Requirement for size and resolution	22

9.2.3	Zooming	22

9.3	Common text and information attributes	22

10	Text and Graphics	23

10.1	Arrangement of viewing group layers.	23

11	Display Elements	23

11.1	Elements related to data and display scales	23

11.1.1	ENC scale	24

11.2	Display of Non-ENC data layers	24

11.2.1	Restrictions on display of Non-ENC data layers	24

11.3	Overscale	25

11.3.1	Chart Scale boundary	25

11.3.2	Overscale Pattern	25

11.3.3	Larger scale data available	26

11.4	Graphical indexes	26

11.4.1	Graphical indexes of ENCs	26

11.4.2	Graphical indexes of other S-100 products	27

11.5	Limits of data	27

11.5.1	ENC No data areas	27

11.5.2	Limit of official S-101 data and non official S-101 data	27

11.5.3	Limits of other S-100 product data	27

11.6	Special ECDIS chart symbols to identify unsafe depths	27

11.6.1	Safety Contour	27

11.7	Date-dependent features	28

11.7.1	Display of date-dependent features by mariner-selected date	28

11.7.2	Indication of date adjustment	28

11.8	Decluttering the screen	28

11.9	IMO presentation elements	28

11.9.1	Scale bar and latitude scale	29

11.9.2	North arrow	29

11.9.3	Graticule	29

11.9.4	Display mode	30

11.9.5	Black level adjustment symbol	30

11.9.6	Detection and notification of navigational hazards	30

11.9.7	Detection of areas for which special conditions exist	31

11.9.8	Definition of safety contour	31

11.9.9	Detection of safety contour	31

11.9.10	Indications related to ENC accuracy	32

11.9.10.1	Check area extension value – S-101 only	32

11.9.10.2	Check area extension value – S-102 depth substitution	33

11.9.10.3	Multiple QualityOfBathymetricData Features	33

11.9.11	Enhanced Safety Contour and Water Level Adjustment.	33

11.10	Other specified display features	34

11.10.1	Legend	34

11.10.2	Legend details	34

11.10.3	ECDIS Legend	34

11.11	Displaying manual and automatic updates and added information	36

11.11.1	Manual updates	36

11.11.2	Additional Mariner’s Information	36

11.11.3	Automatic Updates	37

11.11.4	Identifying automatic chart corrections on demand	37

11.11.5	Update Information	37

11.11.6	Non-official Added Chart Information	37

11.12	Display and Management of Navigational Warnings	38

11.12.1	Requirements of the Graphical User Interface	38

11.12.2	Classifying Navigational Warning information	38

11.12.3	Cancelled datasets	39

11.12.4	Retention of datasets	39

11.12.5	Portrayal of feature types	39

12	Coverages and Time Series for Gridded Data (S-102, S-104 and S-111)	39

12.1	Thinning	39

12.2	Temporal variation	40

13	Colours	40

13.1	Colour palettes implemented by the portrayal catalogue	40

14	Cursor Pick Reports and Displays in Interface Panels	40

14.1	Cursor pick rules	40

14.2	Display of dataset support files	41

14.3	Pick report organisation.	41

14.3.1	Sorting order of results	41

14.3.2	Hover-over function	41

14.4	Tidal stream data panels	42

15	Alerts and Indications	42

15.1	Multiple spatial objects	42

16	Use of Context Parameters	43

17	Dual-fuel systems	43

17.1	Concurrent applicability of S-52 and S-57	43

18	Specifications for the display screen	44

18.1	Physical display requirements	44

18.2	Colour reproduction	44

18.3	Display requirements for colours	44

18.3.1	General	44

18.3.2	Display calibration and verification	44

18.3.3	Colour control, contrast and brightness controls	45

18.3.4	Effect of controls	45

18.3.5	Use of the controls	45

18.3.6	Setting the controls for route monitoring	45

18.4	Colour display capability	46

18.4.1	Colour conversion tolerances and tests	46

18.5	ECDIS Chart 1 and Colour (Differentiation) Test Diagram	48

18.6	Use of ECDIS Chart 1 and Colour Test Diagram	48

18.6.1	Specification for ECDIS Chart 1 and the Colour Test Diagram	48

18.6.2	The Colour Test Diagram	48

18.6.3	Relationship to S-57 Chart 1	50

18.6.4	The Black-adjust symbol	50

19	Dataset Management	50

19.1	Multiple Feature and Portrayal Catalogues	51

19.2	Portrayal and feature catalogue compatibility	52

19.3	Dataset overlaps and gaps	52

19.3.1	Overlaps in ENC coverage	52

19.3.2	Overlaps and gaps between other data products	52

19.4	Updating Datasets on ECDIS	52

19.4.1	Introduction	52

19.4.2	General requirements	53

19.4.3	Automatic Update	53

19.5	New editions, re-issues, cancellations and updates of datasets	54

20	Dataset Support Files	54

20.1	Exchange set delivery	54

20.2	Management of automatic updates to Dataset Support files	55

20.3	Dataset Support File formats.	55

Appendix A – Manual Updates	57

A-1	Introduction	57

A-2	Attribution of Manual Update Features	59

Appendix B: S-100 Data Import Error Codes and Explanations	60

B-1.	Introduction.	60

B-2.	Error code descriptions.	61

B-3.	Exchange Set Installation on ECDIS	63

Appendix C  -  ECDIS Update Status Reports	67

C-1	Purpose	67

Appendix D – Enhanced Safety Contour and Water Level Adjustment	74

D-1	Enhanced Safety Contour	74

D-1.1	Provision of Sun Illumination	75

D-1.2	Control over the use of S-102	75

Data Constraints	75

D-1.3	75

D-1.4	Enhanced Safety Contour Selection and Implementation	76

D-1.5	Vertical Uncertainty in the calculation of depth values.	78

D-2	Water Level Adjustment (WLA)	80

D-2.1	Scope	80

D-2.2	Constraints on input data	80

D-2.3	User Inputs	81

D-2.4	Implementation - general	81

D-2.5	Implementation of WLA Option 1 – WLA for a single datetime instant	83

D-2.6	Implementation of WLA Option 2 – WLA for a datetime range specified by the user as a time period	84

D-2.7	Implementation of WLA Option 3 – linking of WLA to a defined route with planned waypoints and times	85

D-3	Treatment of depth and water level related S-101 features	90

D-3.1	Substitution and adjustment of depth values	90

D-3.2	Areas covered by S-102 only or by both S-102 and S-104	90

Adjustment of vertical clearance values in areas covered by both S-102 and S-104	92

D-3.3	92

D-3.4	Alerts and indication details	93

D-3.5	Legend details	93

Appendix E – Dataset Loading and Display (Rendering) Algorithms	95

E-1.	Introduction	95

E-1.1.1.	Prerequisites	95

Appendix F – GML Coverage Polygons	101

F-1	GML Coverage Polygons	101

F-1.1	Introduction	101

F-1.2	Specification	101

Appendix G - Thinning Algorithms	102

G-1	Introduction	102

G-1.1	Regularly gridded data	102

Appendix H- Import and Use of projected data on S-100 ECDIS	106

H-1	Introduction	106

H-1.1	Conversion from UTM to WGS 84	106

H-1.2	Conversion from UPS North to WGS 84	108

H-1.3	Conversion from UPS South to WGS 84	109

H-1.4	References [Informative]	110

Document History

Changes to this Specification are coordinated by the Inland ENC Harmonization Group (IEHG). New editions will be made available via the S-100 registry. Maintenance of the Specification shall conform to IHO Resolution 2/2007 (as amended).

| Version Number | Date | Approved By | Purpose |
| --- | --- | --- | --- |
| 0.0.1 |  |  |  |

# Introduction

This document contains requirements and guidance for the implementation of harmonised portrayal and other user interaction functions for S-401 Inland Electronic Navigational Chart (IENC) and other S-100 based data products in an Inland Electronic Chart Display and Information System (Inland ECDIS) or Inland Electronic Chart System (ECS). It describes how S-100 products are to be used and displayed simultaneously on the navigation screen. It does not address the portrayal processes or architectures, which are addressed in S-100 Part 9. It is based on the general principles described in S-100 Part 16A.

This specification does not revoke nor does it supersede S-52 based requirements for the display of S-57 based information on Inland ECDIS. It assumes Inland ECDIS and Inland ECS will support both S-52/S-57 based Inland ENCs and S-401 Inland ENCs for the foreseeable future. Such systems should continue to follow the S-52/S-57 based standards (e.g. the European Inland ECDIS standard) for S-57 based information (editions 2.1 to 2.6 of the Product Specification for Inland ENCs).

Inland ECDIS and Inland ECS presentation of Inland ENCs and user interactions are determined by the following standards:

- IHO standards provide the framework for data content, primarily in S-100, as well as standards for portrayal of hydrographic data. S-100 also provides an abstract specification for visual interoperability; for maritime ECDIS, interoperability requirements are specified in S-98. IHO standards additionally provide methods of test and required test results for portrayal and interoperability, these are in S-64 (for S-57) and S-164 (for S-100).

- IEC standards describe methods and required results for equipment and system testing of maritime ECDIS.

- Data product specifications describe the content, data formats, symbols, portrayal rules, packaging, and delivery, of individual data products. For Inland ECDIS and Inland ECS, the basic data product is S-401 (Inland Electronic Navigational Charts). Other products describe additional information relevant to navigation, for example, bathymetry, currents, water levels, regulated areas and services .

- Hardware and type approval requirements for Inland ECDIS and ECS can be defined in regulations of the various regions (e.g. the European Inland ECDIS standard).

The standards that are current at the time of writing of this specification are listed in the References section. More detailed information about the various standards is provided in S-100 Part 16a.

Application developers should obtain an up-to-date set of applicable standards and specifications from the relevant organisations. Developers must conform to the mandatory requirements of the particular standards which apply to an application or system. In case of a conflict between this specification and a mandatory requirement in an applicable standard, the requirement in the standard supersedes the guidance in this specification. Manufacturers, type-approval authorities, and above all boatmasters, are always encouraged to contact the IEHG to provide feedback that they may have about the Inland ECDIS or Inland ECS display, in order that this specification can be kept effective and up to date.

This document contains the main requirements and guidance for development of S-100 based Inland ECDIS and Inland ECS. Boatmasters and other users will receive different S-100-based data products, each providing one or more information layers, and will often need to view some of the information layers simultaneously on a S-100 compatible Inland ECDIS or Inland ECS as well as in other vessel and shore-based scenarios.

IEHG Standards are maintained under a formal, ongoing maintenance regime and are subject to periodic review and revision. They are intended to evolve in response to operational experience, technological developments, and user requirements.

Consequently, future editions of an IEHG Standard may add, modify, or remove requirements. Provisions identified as optional, conditional, or informative in one edition may be reclassified as mandatory in later editions, and vice versa. Conformance claims shall therefore always reference a specific edition of the Standard, and compliance with earlier editions does not imply compliance with subsequent ones.

Implementers are advised to ensure they are using the current edition of the Standard and to consider the potential impact of future revisions in system design and implementation.

This document also contains various clauses which reference constraints within data which is intended for use in S-100 Inland ECDIS and Inland ECS and its supporting infrastructure. These are reproduced in Annex A: Data Constraints. They are intended for data producers to correctly encode data intended for use on Inland ECDIS and Inland ECS. Annex A thus contains no requirements for Inland ECDIS or Inland ECS over and above those contained in the main document of S-403.

# References

## Normative references

IEC 60945	Maritime navigation and radiocommunication equipment and systems – General requirements – Methods of testing and required test results, International Electrotechnical Commission (IEC), Fourth Edition, 2002.

IEC 61174	Maritime navigation and radiocommunication equipment and systems – Electronic chart display and information system (ECDIS) – Operational and performance requirements, methods of testing and required test results, International Electrotechnical Commission (IEC)

IEC 62288	Maritime navigation and radiocommunication equipment and systems – Presentation of navigation-related information on shipborne navigational displays – General requirements, methods of testing and required test results, International Electrotechnical Commission (IEC), Edition 3.0, 2021.

IEC 63173-2	Maritime navigation and radiocommunication equipment and systems - Data interfaces - Part 2: Secure communication between ship and shore (SECOM)

MSC.191(79)	Amendments to the Performance Standards for the Presentation of Navigation-Related Information on Shipborne Navigational Displays, IMO Resolution MSC.191(79), 2004. As amended by MSC 466(101).

MSC.252(83)	Performance Standards for Integrated Navigation Systems (INS), IMO Resolution 252(83), 2007.

MSC.302(87)	Adoption of Performance Standards for Bridge Alert Management, IMO Resolution 302(87), 2010.

MSC.466(101)	Amendments to the Performance Standards for the Presentation of Navigation-Related Information on Shipborne Navigational Displays, Resolution MSC.191(79), 2019.

MSC.530(106)/Rev.1	Performance Standards for Electronic Chart Display and Information Systems (ECDIS), Resolution MSC.530(106)/Rev.1, 2024.

MSC.1/Circ.1609	Guidelines for the Standardization of User Interface Design for Navigation Equipment, IMO MSC.1/Circ 1609, 2019.

S-97	IHO Guidelines for Creating S-100 Product Specifications, IHO Publication S-97 1.1.0 (June 2020).

S-100	Universal Hydrographic Data Model, IHO Publication S-100, Edition 5.2 (2025).

SN.1/Circ.243/Rev. 2	Guidelines for the Presentation of Navigational-Related Symbols, Terms and Abbreviations, IMO SN.1/Circ.243/Rev.2, 2019.

Product Specification for Inland ENCs, editions 2.1 to 2.6, https://editions.openecdis.org/

All undated or unnumbered references refer to the latest revision or amendment.

## Informative references

ISO 19117	Geographic Information – Portrayal, ISO Standard 19117 Edition 2, 2012.

S-52	Specifications for Chart Content and Display Aspects of ECDIS, IHO Publication S-52, Edition 6.1.1, June 2015.

European Inland ECDIS Standard: Part I and Part V of the European Standard for River Information Services (ES-RIS), https://www.cesni.eu/en/

# Abbreviations and Notation

## Abbreviations

AIS	Automatic Identification System

CRS 	Coordinate Reference System

CSS	Cascading Style Sheets

ECDIS	Electronic Chart Display and Information System

ECS	Electronic Chart System

ENC	Electronic Navigational Chart

ENDS	Electronic Navigational Data Service

GML	Geographic Markup Language

HO	Hydrographic Office

IEC	International Electrotechnical Commission

IENC	Inland Electronic Navigational Chart

IHO	International Hydrographic Organization

IMO	International Maritime Organization

BSVS	Boatmaster Selected Viewing Scale

SD	System Database

SVG	Scalable Vector Graphics

XML	eXtensible Markup Language

XSLT	eXtensible Stylesheet Language Transformations

## Notation

The following notation is used throughout the document to describe various portrayal aspects.

(Name of feature or information type)

Features and information types are referenced by their names in camel-case or space-separated names in initial capitals (e.g “LandArea” or “Land Area”). These names will be in bold text to differentiate them from other text, e.g “LandArea”

(Name of attribute)

Attributes are referenced by their names in camel-case with initial letters in lower case, or space-separated names in all lower case (e.g “visuallyConspicuous” or “visually conspicuous”). These names will be in italics, e.g. “visuallyConspicuous”

(Portrayal register item)

Items from the IHO portrayal registry are referenced by their registered name and item type (“line style SCLBDY51”). The portrayal registry is a component of the IHO Geospatial Information Registry (https://registry.iho.int).

Other items of a technical nature, for example context parameters, are referenced by their names in italics.

## Normative vs Informative within this document.

This document uses the word “must” to describe how mandatory functionality for S-100 Inland ECDIS and Inland ECS must be implemented. Some parts of the S-100 framework are also described in overview to provide useful context for implementers. Where this is the case clauses are marked as “Informative” to clarify.

Non-mandatory requirements or recommendations to implementers are indicated using the word “should”.

The word “may” expresses a permissible practice for action. It does not express a requirement.

Mandatory functionality is achieved by the implementation of the relevant parts of S-100 and S-403. In all cases the content of the published, operational IEHG product specifications are also normative.

# Background - System Concepts and Limitations

## Inland ECDIS and Inland ECS concept, limitations, and challenges

Definition of an Inland Electronic Chart Display and Information System (Inland ECDIS):

Inland Electronic Chart Display and Information System (Inland ECDIS) means a navigation information system which with adequate backup arrangements displaying selected information from a system database with positional information from navigation sensors to assist the boatmaster in route planning and route monitoring on inland waterways and, if required, display additional navigation-related information.

While a type approved ECDIS is allowed to display S-401 IENCs when the vessel is outside of SOLAS regulated areas, it is not required by S-98 that every maritime ECDIS needs to be able to display IENCs correctly. The term “Inland ECDIS” is used for applications that meet the requirements of S-403 and the applicable additional regional requirements. An Inland ECDIS may also be approved as a maritime ECDIS in accordance with S-98 and the IMO standards, but it is not required. An Inland ECDIS can also be used as an ECS in SOLAS areas if it is able to display S-101 ENCs, but is not type approved as maritime ECDIS

ECS are allowed to display IENCs, but it is not required by maritime standards. The term “Inland ECS” is used for an ECS that meets the requirements of S-403 and the applicable additional regional requirements.

The following list explains more of the Inland ECDIS and Inland ECS concept in detail and provides information about certain limitations.

- IENC is an integral part of Inland ECDIS and Inland ECS and therefore is defined as the base layer for the portrayal harmonisation framework. Additional layers are classified as two main types, additional information to that of the IENC or enhanced information to that of the IENC. Additional information is information that is not contained in the IENC, while enhanced information are layers that contain improved, detailed or higher resolution information than the IENC.
S-101 ENC is an alternative base layer. For areas with S-101 ENCs S-98 has to be applied.

- ECDIS, used together with up to date official data, may be accepted as complying with the nautical chart and nautical publications carriage requirement required by regulations V/19 and V/27 of the 1974 SOLAS Convention amended in 2009. Electronic chart systems not meeting these ECDIS specifications of IHO and IMO are known as ECS (Electronic Chart Systems).

- Chart information may be used in conjunction with a radar overlay on Inland ECDIS. Tracked radar targets and targets reported by AIS (Automatic Identification System) can be integrated into the Inland ECDIS and Inland ECS display, as well as other navigational information which may be added to the Inland ECDIS or Inland ECS display. With the advent of S-100, additional types of data may also be occasionally added to the Inland ECDIS and Inland ECS display, especially high-density bathymetry, surface current and water levels data or predictions, and under keel clearance area data.

- Colours and symbols are defined in the S-401 portrayal catalogue.

- Inland ECDIS and Inland ECS combine chart and real-time navigational positioning information. Modern navigation systems (i.e. GNSS) may offer a more accurate positioning than was available to position some of the surveys from which the digital chart data IENC was derived. Further, other products may not be encoded to the same accuracy or precision as IENC data or ship’s positioning, therefore there is a potentially greater reliance on metadata covering data quality and precision in a digital navigation context

- In the initial period, S-100 compatible Inland ECDIS and Inland ECS will be “dual-fuel” in that they will have both S-57 based and S-401 IENCs (see: Section 18). It is possible that there will be situations where the navigation screen will display both S-57 based and S-401 IENCs. The user experience aspects of such situations are discussed in Section 18.

- Inland ECDIS and Inland ECS shall also be capable to display S-101 ENCs.

The versatility of Inland ECDIS and Inland ECS poses several challenges for Inland ECDIS and Inland ECS display design. These challenges are discussed below.

The diversity of information displayed on an Inland ECDIS or Inland ECS may include:

- physical chart information, for example, coastline, depth contours, buoys;

- traffic routeing; specified areas; cautions; etc.;

- supplementary information from light list, etc.;

- boatmaster's notes; additional local chart information; manufacturer's information;

- chartwork such as planned route; electronic bearing lines and range rings; etc.;

- own ship's position and velocity vector; ship's heading and rate of turn; past track;

- fix accuracy, or position check from secondary positioning system;

- possibly, vessel handling options, based on vessels’s characteristics, alphanumeric navigation information (vessel’s latitude, longitude, heading, course, etc.);

- information from radar and other sensors;

- information from AIS;

- navigational indications and alerts generated by Inland ECDIS or Inland ECS;

- possibly, telemetered information from shore authorities (traffic, real-time tides etc.);

- possibly, ice information;

- reminders (for example, time to contact pilot station);

- possibly, messages from other displays.

The flexibility of portrayal may include:

- displaying/removing various types of chart and non-chart information,

- selecting standard chart display or a thinned out display, and either paper chart symbols or simplified symbols,

- using cursor interrogation for further detail,

- overlaying/removing radar video or radar target information (in order to: confirm ship’s positioning; aid radar interpretation; show the entire navigation situation on one screen),

- overlaying/removing various other sensor information, or information telemetered from shore,

- changing the scale or orientation of the display,

- selecting true motion or relative motion,

- changing screen layout with windowed displays, text information in the margins, etc.,

- possibility of pull-down menus and other operator interaction devices being alongside the operational navigation display and so interacting with it,

- giving navigation and chart warnings such as “too close approach to safety contour”; “about to enter prohibited area”; “overscale display”; “more detailed (larger scale) data available”; etc.,

- possibly, a diagrammatic representation of a computer evaluation of grounding danger,

- possibly, a diagrammatic representation of the immediate vicinity of the ship to aid in close quarters manoeuvring,

### Depth Related Functionality

S-100 Inland ECDIS allows water levels to be automatically adjusted using an interoperable combination of S-401, S-102 and S-104 data, both for monitoring and planning purposes. This is described fully in appendix D of this document.

# User Interface Design

## General principles

The guidelines in IMO MSC.1/Circ.1609 (Guidelines for the Standardization of User Interface Design for Navigational Equipment) apply to the design of user interfaces for navigation systems such as ECDIS and have therefore also been a basis for S-403. The general principles are described in MSC.1/Circ.1609 and are summarised in S-100 Part 16A.

# Data Layers

## Data products and information layers

### Basic product specifications and layers

The Inland ECDIS and Inland ECS must be able to process and display at least the following data products:

- S-401 IENC data as the base layer, alongside S-57 based IENC data (editions 2.1 to 2.6) during the transition period;

- S-102 Bathymetric Surface, only for Enhanced Safety Contour and Water Level Adjustment (Appendix D);

- S-104 Water Level information for Surface Navigation, only for Water Level Adjustment (Appendix D);

The IENC layer (S-401 and S-57) must be permanently on. The boatmaster must be able to switch the graphical display and processing of other S-100 products on and off. Graphical display of S-401 is defined in Appendix E. S-102 and S-104 processing is contained in Appendix D.

### Projected Data

Inland ECDIS and Inland ECS must support the import of data products using unprojected geographic latitude and longitude values. Additionally, Inland ECDIS and Inland ECS must support the import of S-102 and S-104 data products using coordinates which are projected using either UTM or UPS projections. Formulas for conversion of such coordinates back to unprojected latitude and longitude values are contained in appendix H of this document.

### Other data products

The intention is that in future editions of this document, when operational product specifications are published, navigation displays such as the Inland ECDIS or Inland ECS main screen will allow the boatmaster to process the following data products:

- .

### Data layers in Display Base and Standard Display

Display base means the chart content which cannot be removed from the display. It is not intended to be sufficient for safe navigation. Standard display is the display mode intended to be used as a minimum during route planning and route monitoring. Inland ECDIS and Inland ECS are required to present the standard display at any time by a single operator action.

When display base or standard display is selected:

- User selector settings must be preserved;

- A combination of S-401, S-102 and S-104 is used to develop the safety contour (if already selected by the user) in accordance with Appendix D;

## Mitigation of data overload

To prevent the simultaneous presentation of multiple data layers from cluttering the presentation and hiding critical information, the Inland ECDIS and Inland ECS should implement good practice in user interface design, see also Section 8.

Detection of and response to possible data overload should be guided by the role a particular system is playing at the moment (e.g., a display being used for planning may be able to treat potential overloading in a laxer manner than navigation displays being used for route monitoring and collision avoidance).

Selection of standard display, available via a single operator action, provides an additional mitigation to data overload.

# Portrayal Process, Catalogue Elements, and Alerting

This section provides a brief, informative summary of the S-100 portrayal process and the main elements of Portrayal Catalogues. More details about the process and definitions of Portrayal Catalogue elements are available in S-100 Parts 9 and 9a.

Later clauses of this document describe how the portrayal elements should be used in an Inland ECDIS and Inland ECS implementation.

## Overview of the portrayal process

The system has feature data within its internal database that needs to be portrayed. The System Portrayal Engine transforms the feature data into drawing instructions according to the portrayal rules defined in the Portrayal Catalogue. The drawing instructions are intermediate data used by the rendering engine to produce the portrayal output. Drawing instructions include such things as references to symbol definitions, priority and filtering information. The symbol definitions contain the details of all graphical elements used for the portrayal. The drawing instructions are processed by the rendering engine to produce the final display according to the output device.

S-100 defines two different portrayal mechanisms, one based on XSLT templates and the other based on Lua programming. The basic portrayal process is the same for both, and is described in S-100 Part 9. For the S-100 product specifications listed in 6.1.1, the Inland ECDIS and Inland ECS must at least support Lua based portrayal catalogues in accordance with Part 9a and Part 13

## Elements of S-100 Portrayal

Much of the content of this section is included for information as it is described definitively in S-100 Part 9 and Part 9a.

### Pixmaps

Pixmaps can be used for defining pattern fills for areas that are not sufficiently described to be symbolised, or for which no symbol exists in the IHO Geospatial Information (GI) Registry.

### Colour coding scheme

The Portrayal Catalogue uses a colour scheme, which classifies colours by their usage. Each colour usage is represented by a colour token. Each colour token corresponds to a colour definition given in CIE and/or sRGB coordinates in one of a set of colours; each set is intended for a different bridge lighting condition. Each such set of colours is referred to as a “palette” and is generally identified by a descriptive name, such as “Day”, “Dusk” and “Night”. A particular colour token is often assigned different CIE/sRGB coordinates in different palettes (especially day/night/dusk palettes).

A “colour profile”, as the term is used in S-100, is a collection of tokens within a single XML file. An Inland ECDIS or Inland ECS makes palettes for day, night, and dusk conditions available on the system.

Symbols, fill styles and line styles refer to the colour tokens as part of the symbol definition.

### Symbols

The Portrayal Catalogue provides a set of symbols. These symbols are encoded as SVG files which conform to the S-100 SVG profile

### Area fills

The Portrayal Catalogue offers various ways to fill areas. They can be filled with an opaque colour; with a colour shown with some transparency; or with a pattern of symbols (fill pattern) or with a centred symbol. Fill patterns and centred symbols are a solution for the symbolization of areas in special situations.

Area boundaries should not be visible on the edge of the display window when the display window is enclosed by an area. However, if the boundary is actually on the edge of the display window, it should be visible.

### Line styles

The Portrayal Catalogue uses two types of line styles: simple line styles and complex line styles. Simple line styles are solid, dashed or dotted lines with varying colour and thickness (width or stroke width). Complex (or “composite”) line styles are composed of repeating line patterns.

Simple line styles are generally described by continuity, width, and colour. The full specification of a simple line style may also include other elements such as dash interval, cap and join types (see S-100 clauses 9-12.4 and 9a-11.2.2.3), defaults for which may be set in the Portrayal Catalogue. Complex line styles consist of additional elements, described in S-100 9-12.4 and 9a-11.2.2.3.

Complex linestyles may be one-sided (symbols, text, etc., which are part of the line extend to only one side of the line) or two-sided (symbols, text, etc. extend on both sides of the line).

### Portrayal of Shared Edges.

Shared edges portrayed with dash patterns may be presented incorrectly to the boatmaster when the Inland ECDIS or Inland ECS draws the edge in both directions. This issue can occur along the shared edges of features which abut, such as restricted areas or anchorages where adjacent edges are drawn in opposite directions.

Rendering the dash pattern in both directions can alter the pattern from what is intended, and in some cases may result in a solid line.

In the figure below, note the edges under the green highlight:

| Incorrect presentation | Correct presentation |
| --- | --- |

This issue can also affect complex line styles, for example:

| Incorrect presentation | Correct presentation. |
| --- | --- |

### Text

S-401 and other Product Specifications may utilize a cartographic feature called TextPlacement that is used in association with a feature to optimise placement of labels (usually, either feature name or light characteristic description string).

There are three types of text instructions possible in portrayal catalogues:

- Text relative to a point;

- Text that will be drawn along a line; and

- Text placed relative to an area uses a local coordinate reference system. Note that this can cause the text to be drawn at multiple locations.

However, explicit cartographic placement along curves or relative to area/local CRS cannot be explicitly encoded in S-401 datasets, though it can be encoded in portrayal rules in a Portrayal Catalogue (S-100 Part 9a, clause 9a-11.2).

### Style sheets

Cascading Style Sheets (CSS) files are used to provide different sets of stroke and fill style instructions to be applied to symbols. This mechanism allows changing colours and line weights used in the symbols by swapping CSS files according to the desired colour scheme.

In principle, any style attribute can be set in a CSS file, but the CSS files in IHO Portrayal Catalogues are restricted by S-100 9-B-4.4 to only two use cases: setting stroke and fill colours, altering the visibility of elements which are not normally displayed (for example, the pivot point).

### Display planes

Display planes are used to split the output of the portrayal functions into lists. An example of this is the separation of chart information drawn under a radar image and chart information drawn over a radar image.

### Display (drawing) priorities

Display priorities are also called “drawing priorities” in S-100 Parts 9 and 9a.

Display priorities control the order in which the output of the portrayal functions is processed by the rendering engine within a display plane. Priorities with smaller numerical values will be processed first.

The display priority applies irrespective of whether a feature is a point, curve or surface. If the display priority is equal among features, curve features have to be drawn on top of surface features whereas point features are drawn on top of both. If the display priority is still equal among features of the same type of geometry (curve, surface or point) the given sequence in the data structure of the System Database, or some other neutral criterion, should be used for a decision as to which feature is drawn on top.

### Viewing groups

The viewing group controls the content of the display. It provides an on/off switch in the Portrayal Catalogue for all drawing instructions assigned to the corresponding viewing group.

### Viewing group layers

A viewing group layer defines a collection of viewing groups whose visibility can be simultaneously toggled on/off by an application.

### Display modes

A display mode defines a collection of viewing group layers which can be simultaneously toggled on/off by an application.

### Rules

A portrayal rule describes the transformation of feature data into drawing instructions.

### Context

The context is a set of parameters whose values alter the portrayal processing. These parameters can be used to provide contextual information to the execution of rules in portrayal processing. Examples are settings of depth values for the safety depth, shallow and deep contours.

### Validity times

Drawing instructions can be designated as being valid only during a specified interval, which may be open-ended at either beginning or end, or a single time point (with beginning and end the same). Intervals may be specified in terms of date, time (of day), or date-time endpoints.

The start and end instants are defined by their date/time component of the smallest granularity. For example, if the end instant is specified a date without a time of day, and the interval is “right-closed”, the end instant is midnight at the end of the specified day (240000 in ISO 8601 terms). This is consistent with the S-100 treatment of dateStart and dateEnd attributes (see S-100 Part 3 clause 3-8-3 (Interpretation of models of time intervals and period), but note that the open/closed nature of the interval affects the interpretation for Part 9 time intervals — if the same interval is specified as “right-open” the start time point is midnight at the beginning of the specified day (000000 in ISO 8601 terms).

# Display Organisation and Operation

## Display of non-S-100 information

This section describes the display of information that is not from official S-100-based data products, S-57, radar, or AIS.

### Distinguishing between official S-100 data and additional data

This specification requires that Inland ECDIS and Inland ECS must distinguish between official S-100 data and additional data from users (boatmasters) and manufacturers. The colour and symbol usage for boatmasters’ and manufacturers’ data in the IHO GI Registry and the Portrayal Catalogues for S-100 data products are designed to implement this while ensuring the display remains clear and uncluttered.

The official status of S-100 data delivered to the Inland ECDIS and Inland ECS is defined by the value of a role. This is encoded in the ST field of the X.509 certificate referenced in the mandatory digital signature accompanying content in the exchange set catalogue. Digital signatures are mandatory for any S-100 data imported into the Inland ECDIS and Inland ECS, whether official or not. The mandatory fields within every authenticating certificate are defined as:

- C (Country) = ISO Country Code of state making request

- ST (State or Province) = A code reflecting the role of the signing entity (below)

- O (Organisation) = member state organisation name (text) or other organisation name

- CN (Common Name) = IHO data producer code integer and alpha code (as part of a colon separated MRN), e.g. urn:mrn:iho:00AA:01810 or urn:mrn:iho:GB00:00540. For digitally signed datasets, the CN data producer integer and alpha code elements in the MRN of at least one of the authenticating certificates must match those contained in the dataset and its CATALOG.XML entry.

For Inland ECDIS and Inland ECS, the ST code holding the role determines whether a dataset or supporting resource is non official or official. The following codes are supported by the IHO data protection scheme:

- For S-100 datasets or supporting resources the following roles define official data.

  - DATA_PRODUCER Data Producers - producing data content for live navigation under SOLAS. This data is "official"

  - DATA_AGGREGATOR RENCs/Aggregators - validate, distribute and (sometimes) digitally sign data on behalf of their members. These organisations do not create data content, except for generation of S-128 datasets but can sign data as “official”

- Catalogues

  - SCHEME_ADMINISTRATOR The Scheme Administrator, the IHO. Only digitally signed catalogues with a certificate authenticated by the scheme administrator may be loaded onto the Inland ECDIS and Inland ECS.

ST field values using the role OTHER_DATA_PRODUCER (or any other value not included in the list above, or those not conforming to the CN MRN format above) represent data or catalogue content which is non official.

### Supplemental display items

The following information must be shown on demand

- Positional data and time;

- Legend (See 11.10.1);

- Feature description and associated attributes (result of "cursor query") in human readable language;

- Textual information from IENC, for example, dataset name, compilation date, date of issue;

- Record of updates for all data products;

- Black-adjust symbol for contrast adjustment;

- Colour differentiation diagram (available from IHO S-164);

- List of categories which are removed from Standard Display;

- Dataset Edition and update numbers (if applicable) of S-401 and other Products in use;

- Edition numbers of Presentation Libraries in use;

- Edition numbers of feature and portrayal catalogues in use.

## Priority of information

### Priority layers

System Database information contains three categories that determine what data is to be on the display: Display Base (always present on the display); Standard Display (the default display); and all other information (displayed on demand).

There are 9 basic priority layers for the drawing sequence of the data on the display, ordered from higher to lower priorities in the list below:

- Inland ECDIS and Inland ECS visual alerts/indications (for example caution, overscale);

- Official-data: Points/Curves and Surfaces + official updates;

- Manual input and Navigational Warnings;

- Official-caution (IENC and other cautions);

- Official-colour-fill area data;

- Official on demand data (for example, water levels, surface currents, under keel clearance);

- Radar and AIS information;

- Boatmaster’s data: Points/lines and areas;

- Boatmaster’s colour-fill area data.

This list is not intended to indicate a drawing sequence, but to specify that the information content of category n+1 must not obscure the information content of category n, or any higher-priority category (n-1 etc.).

### Radar priority

Radar priority for a given display plane is described by the order attribute within the Portrayal Catalogue.

The RadarOverlay Portrayal Catalogue context parameters must be updated whenever the radar image is toggled on or off.

When present, the radar image should be written over Portrayal Catalogue display planes with a negative order attribute; and below display planes with a positive order attribute.

In order to adjust the ship’s position, the Inland ECDIS and Inland ECS may incorporate the capability of temporarily changing the radar image priority during the adjustment.

## Displaying updates in Inland ECDIS and Inland ECS

The strategy for displaying updates in Inland ECDIS and Inland ECS is derived from the following requirements:

Inland ECDIS and Inland ECS shall be capable of accepting official updates provided in conformity with S-100 standards. These updates must be automatically applied to the system database. By whatever means updates are received, the implementation procedure must not interfere with the display in use.

Inland ECDIS and Inland ECS shall also be capable of accepting updates entered manually with simple means for verification prior to the final acceptance of the data. They must be distinguishable on the display from official data and its official updates and not affect display legibility. (The requirements for Manual Updates are given in clauses 11.11.1.)

Inland ECDIS and Inland ECS shall allow the boatmaster to display updates in order to review their contents and to ascertain that they have been included in the system database.(The requirements for displaying updates for review are given in clause 11.11.5.).

## Display functions

The Portrayal Catalogue defines a number of Context Parameters. These must all be implemented and exposed to the end user through the user interface. Many of these may have already been implemented for existing (S-57) Inland ECDIS and Inland ECS, for example Four Depth Shades, Contour Labels and viewing of Accuracy patterns. Implementation should therefore be harmonised to provide a harmonised user interface.

# General Rules for Symbols and Text

## Symbol Specifications

All symbols are specified in the Portrayal Catalogue for the respective Product Specifications and are defined according to S-100 Part 9 appendix B.

Some feature types do not have a symbol. Such "no symbol" features may be picked up by cursor interrogation of the area.

## Reproduction of lines, symbols and text

### Introduction

IEC 62288 sets requirements for size of operational information other than chart information in terms of the physical size of symbols and the number of pixels required to present them. These parameters are dependent on a minimum angle subtended from the eye and the nominal viewing distance specified by the manufacturer.

Requirements for additional symbols not yet available in the IEC 62288 are defined in IEC 61174. The Portrayal Catalogue sets requirements for size of chart symbols

### Minimum Requirement for size and resolution

The minimum sizes for all symbols must be as defined in the Portrayal Catalogue.

Enough "picture units" (pixels) must be used to draw small features and symbols clearly and allow viewers to distinguish similar symbols. Symbols must therefore always be drawn with at least the same number of pixels as are required to draw the symbol at the size defined in the Portrayal Catalogue for the minimum resolution and minimum chart display area.

NOTE (informative): This requirement means the minimum height in pixels of a symbol is: (symbol height in mm) divided by the "pixel size" for the minimum size chart display.

The CHKSYM01 symbol may be used for comparison; this symbol must measure 5mm×5mm when displayed at its nominal size (that is, scaled to 100%).

Since the CHKSYM01 symbol is quite small for the purpose of on-site system configuration by ordinary users, manufacturers may use an enlarged version or an equivalent method for that purpose.

### Zooming

When the display scale is changed the symbol and text size must remain constant

## Common text and information attributes

Several Product Specifications use common structures for feature names (featureName) and information attributes (information.text)

To support dataset portrayal using languages other than English, the Inland ECDIS and Inland ECS must expose to the boatmaster two context parameters;

National Language (applies for S-57 based IENC only)

This Boolean value should be used to show the national attribute values present in S-57 IENC data.

Preferred Language (does not apply to S-57 based IENC)

This context parameter contains an ordered list 0 or more preferred language values. The following procedure must be used when displaying the “feature name” attributes.

Feature names: The attribute “feature name” (featureName) is a complex attribute consisting of the mandatory sub-attribute “name” (the actual name of the feature) and sub-attributes “language” and “name usage”. The sub-attribute “name usage” is an enumeration which indicates if a name should be displayed when preferred language is not selected (1), or may be displayed when preferred language is selected (2).

If selected, feature name values must be displayed based on the user defined language preference values, the first value contained in the context parameter list should be displayed. If no feature names correspond to the language preferences, then the feature name with name usage= 1 must be displayed.

In the following example Island Group feature below when the user has set a language preferences as follows;

- sms

- smn

Then the name Lâu'ŋŋsuõllu is displayed, because it contains a name with the language “sms”, the highest priority in the language preference list. With no language preference set, the name “Turvesaaret” must be displayed because it has nameUsage = 1.

IslandGroup

featureName

language: fin

name: Turvesaaret

nameUsage: 1

featureName

language: smn

name: Lavŋesuolluuh

nameUsage: 2

featureName

language: sms

name: Lâu'ŋŋsuõllu

nameUsage: 2

# Text and Graphics

## Arrangement of viewing group layers.

Viewing group layers should be grouped together when one (the child) is wholly included (a subset) of another (the parent). The table below shows an example of grouping of “Other text” viewing group layers.

| Viewing Group Layer | Name of Viewing group layer in the Inland ECDIS and Inland ECS |
| --- | --- |
| 1 | Important text |
| 2 | Other Text |
| 2.1 | Names |
| 2.2 | Light description |
| 2.3 | All other |

# Display Elements

This section describes additional display elements and special symbols and considerations for IENC data, including chart furniture, contours, boatmaster’s features, and presentation of updates.

Cursor pick reports and information displays in off-graphic panels are described in clause 14.

## Elements related to data and display scales

IENC Features are contained within DataCoverage features and aggregated into datasets. Each DataCoverage feature contains attributes optimumDisplayScale, minimumDisplayScale and maximumDisplayScale. The algorithm which must be used for the selection and rendering of IENC features is contained in Appendix E – Dataset Loading and Display (Rendering) Algorithms.

To ensure seamless presentation all enabled features of same display priority from all datasets must be drawn so as to prevent clipping at the edges of datasets.

### IENC scale

The term “scale” is used to describe either:

- Display Scale, or (equivalently) Boatmaster Selected Viewing Scale (BSVS)

- The scale of the source data used to compile the IENC.

- Data Scale (the values used for the DataCoverage attributes minimumDisplayScale, maximumDisplayScale, optimumDisplayScale. This may not be the same as the scale of the source data used to compile the IENC. These values define the range of scales at which IENC coverage is intended to be displayed.

The S-401 IENC values used for the Data Scale attributes minimumDisplayScale and optimumDisplayScale can be  taken from the table below:

Scale

NULL

1:10,000,000

1:3,500,000

1:1,500,000

1:700,000

1:350,000

1:180,000

1:90,000

1:45,000

1:22,000

1:12,000

1:8,000

1:4,000

1:3,000

1:2,000

1:1,000

1:500

1:200

Table 15.5 - minimumDisplayScale and optimumDisplayScale values

The ordering and selection of Data Coverage features by Data Scale, prior to S-401 portrayal is defined fully in in Appendix E – Dataset Loading and Display (Rendering) Algorithms.

## Display of Non-IENC data layers

Non-IENC data layers (as listed in clause 6.1.1) may not contain DataCoverage features. Where the dataset discovery metadata entries for non-IENC datasets contain values for minimumDisplayScale and optimumDisplayScale these values define the range of display scales for which a non-IENC dataset is intended to be displayed.

Where the dataset discovery metadata do not contain values for minimumDisplayScale and optimumDisplayScale and the user has requested data to be displayed/used, then the Inland ECDIS and Inland ECS must show it by default but may drop display in the interests of system resources.

There is no algorithm defined for the selection and rendering of non-IENC data.

### Restrictions on display of Non-IENC data layers

Spatial overlap between non-IENC datasets is only permitted when their display scale ranges do not overlap. In the case of such overlaps the user must be able to choose which data provider is used, or a selection based on maximum available optimumDisplayScale values, or optimumDisplayScale values closest to the BSVS

## Overscale

Data is overscale when displayed at a scale larger than its optimumDisplayScale.

#### Overscale Indication

The overscale indication is intended to remind the mariner that chart errors are magnified as the display scale is increased. A 1 mm error at a display scale of 1:20,000 becomes a 1.3 mm error at a display scale of 1:15,000 and a 2 mm error at 1:10,000. The overscale indication is specific to IENC portrayal.

The overscale factor must be calculated as :

[denominator of the optimumDisplayScale] / [denominator of the mariner’s selected viewing scale]

and expressed as, for example "X1.3", or "X2" (using the figures in the example above.)

When the overscale factor is > 1 it must be indicated on the same screen as the chart display, and treated as display base. Use colour SCLBR.

This overscale indication must be shown whenever the overscale factor at the vessel location (when the vessel is on screen), or at the centre of the screen (when the vessel is not on screen), exceeds 1.

### Chart Scale boundary

Where the Inland ECDIS or Inland ECS simultaneously presents Data Coverage features with different Optimum Display Scales, the “chart scale boundary” must be shown between regions where there are differences in optimumDisplayScale of more than three steps. The steps are the defined scale bands in Appendix E-1.2.

The "chart scale boundaries", where the optimumDisplayScale of the data changes by more than three steps, must be symbolised on the Inland ECDIS and Inland ECS display by a solid 0.32mm wide line using colour token CHGRD. Alternatively, linestyle SCLBDY51 may be used. The display priority is 9; over-radar; standard display; viewing group 21030.

When scale boundaries of smaller scale Data Coverage areas overlap larger scale Data Coverage areas, that portion of the scale boundary which intersects the larger scale Data Coverage area should not be visible.

### Overscale Pattern

Inland ECDIS and Inland ECS displays all chart data at the same (boatmaster selected) viewing scale. In order to avoid leaving parts of the display blank, the display may be rendered using data from multiple datasets using the algorithm defined in  Appendix E – Dataset Loading and Display (Rendering) Algorithms. These datasets may contain Data Coverage areas with different optimumDisplayScale.

The area fill OVERSC01 must be used to indicate Data Coverage areas displayed larger than the maximumDisplayScale, when the area has been selected for display by the Inland ECDIS or Inland ECS in order to avoid leaving that portion of the display blank. These Data Coverage areas are those selected after Row 3.b, is reached in the SelectDataCoverages function in clause E-1.3 of Appendix E – Dataset Loading and Display (Rendering) Algorithms

For example, an overscale situation arises when the ship approaches a scale boundary from a larger to a smaller scale IENC, typically when leaving harbour. In combining data from the large scale and the small scale IENCs to generate a display at the larger scale, the Inland ECDIS and Inland ECS will have "grossly enlarged" the small scale data.

The "grossly overscale" part of the display must be identified with area fill OVERSC01, as illustrated in Figure 1.

Figure 1 shows a display (at MSVS = 1:12,500) with two IENCs with the scale values contained in the table.

The diagram shows the optimumDisplayScale value of the larger scale IENC and the maximumDisplayScale value of the smaller scale IENC.

| Small Scale IENC | optimumDisplayScale | maximumDisplayScale |
| --- | --- | --- |
|  | 1:180,000 | 1:50,000 |
| Large Scale IENC |  |  |
|  | 1:12,000 | 1:4,000 |

`

Figure 1: Illustration of overscale display (BSVS=1:12,500)

In this clause, "grossly enlarged" and "grossly overscale" means that the display scale is enlarged/overscale with respect to the maximumDisplayScale. In the example, at the left edge of the display, the BSVS of scale of 1:12,500 is larger than the maximumDisplayScale of 1:50,000, and so the overscale pattern is required.

Note that in this situation the OVERSC01 area fill should only be shown on the area compiled from the smaller scale IENC. If the area from the larger scale IENC is also overscale, this should be indicated only by the "overscale indication". The OVERSC01 area fill should not be shown on the part of the display taken from the larger scale IENC

For example if the display scale (MSVS) 1:3,500 is selected, the area of optimumDisplayScale 1:12,000 would have an overscale indication of X3.4 but would have no OVERSC01 area fill.

### Larger scale data available

The Inland ECDIS and Inland ECS must provide an indication when an IENC with a larger optimumDisplayScale than that provided by the display is available at own vessels’s position.

## Graphical indexes

### Graphical indexes of IENCs

The system must be capable of displaying a graphical index of IENCs.

All datasets installed on the system must be included in the graphical index.

Additionally:

- IENC Discovery metadata from Exchange Catalogue files in Exchange Sets

The description of dataset extents in S-401 IENC discovery metadata in exchange catalogue files uses a fixed format, described in Appendix F.

It must be possible to individually select the display of the graphical index entries for the installed S-57 based and S-401 IENCs. The display of each data product must be differentiated.

### Graphical indexes of other S-100 products

The Inland ECDIS and Inland ECS must additionally implement a capability for displaying graphical indexes of S-102 and S-104 data products, and may implement a graphical index of other non-IENC data products

If the system displays graphical indexes for different products simultaneously it must be possible to select the display of each supported data product. The display of each supported data product must be differentiated.

## Limits of data

### IENC No data areas

The Inland ECDIS and Inland ECS must fill any areas in which there is no installed IENC coverage using area pattern NODATA03, using colour token NODTA. These areas are defined as no data areas.

The indication "No IENC Available" must be shown if the display includes areas in which there are waters with no IENCs installed.

An indication that the boatmaster must refer to the official chart must also be given whenever line style NONHODAT appears on the display, or whenever the display is comprised of other than IENC data as specified by clause 8.1.1.

### Limit of official S-401 data and non official S-401 data

The limit of official S-401 data on the graphical index defines the limit of official IENC coverage. Clause  8.1.1 describes how the Inland ECDIS and Inland ECS determines that the data is official.

The appearance of the “No data” colour (colour token NODTA) and NODATA03 area fill (see 12.5.1) will indicate the end of official data.

If non-official chart data is shown on the Inland ECDIS and Inland ECS display, its boundary must be demarcated by the line style NONHODAT. Note that the NONHODAT line style is a “one-sided” line and the boundary of the non official data must be drawn according to S-401 rules to ensure that the diagonal stroke of the line is on the non official data side of the line. The non official data boundary indicated by the NONHODAT line style serves to separate official IENC data from non official chart information.

### Limits of other S-100 product data

For all other supported data products, it must be possible for the boatmaster to be able to select for display “non official data” boundaries. Clause 8.1.1 describes how the Inland ECDIS and Inland ECS determines that data is official. The display for each supported data product must be differentiated.

## Special Inland ECDIS and Inland ECS chart symbols to identify unsafe depths

The Inland ECDIS and Inland ECS highlights four concepts that are important for safe navigation. These are the safety contour, depth shades, the safety depth and isolated dangers. Depth shades, safety depth and isolated dangers are implemented in the portrayal catalogue.

### Safety Contour

The own-ship safety contour is double-coded by a thick line and a prominent change in depth shade.

If the safety contour selected by the boatmaster is not available in the System Database, the Inland ECDIS and Inland ECS must default to next deeper contour and inform the boatmaster. If the ship moves onto a new chart, and the safety contour previously in use is no longer available, the Inland ECDIS and Inland ECS must again select the next deeper contour, and inform the boatmaster.

If the boatmaster does not select a safety contour, the value must default to 30 metres.

## Date-dependent features

There are a number of features within the inland waterway environment which are seasonal, such as racing buoys. These features are only to be displayed over a certain period, S-401 uses the complex attribute periodicDateRange with the sub attributes dateStart and dateEnd to indicate the periodic nature of the feature. Other features, such as traffic separation schemes, use the complex attribute fixedDateRange with the sub attributes dateStart and dateEnd to indicate their introduction or removal. In order for the boatmaster to receive important changes to traffic separation schemes before the event, authorities are required to provide updates or new editions containing the alterations at least one month before they come into force. Any S-401 feature with one of the above complex attributes must not be displayed outside its effective dates unless requested by the boatmaster.

### Display of date-dependent features by boatmaster-selected date

To provide the boatmaster with effective route planning capabilities and for the look-ahead function during route monitoring Inland ECDIS and Inland ECS must display date dependent chart data based on a boatmaster selected date or date range (start viewing date and end viewing date).

During route planning and monitoring the boatmaster must be able to select a date or date range to display all date dependent chart features.

EXAMPLE: A new traffic separation scheme is coming into effect on 01.01.2013, it has been encoded by the IENC producer using the attribute dateStart. The current date is 12.12.2012 and the boatmaster is planning a route that will cross this area over the effective start period.

In this case the Inland ECDIS and Inland ECS must be capable of providing the boatmaster the ability to set the date the vessel will be in the area (e.g. 02.01.2013 in the above example) and the system should show the new traffic scheme.

### Indication of date adjustment

When viewing date or date range do not include the current date, the boatmaster must be informed by a permanent indication on the chart display that the date has been adjusted. The indication must begin with the text “Display Not Current Time” and the selected date or date range must at least be available on demand.

The use of one of the following formats is recommended:

- Display Not Current Time – Display is based on date dd mmm yyyy

- Display Not Current Time – Display is based on viewing date range from dd mmm yyyy to dd mmm yyyy

## Decluttering the screen

To reduce screen clutter some features within IENC carry the attribute scaleMinimum to specify the smallest display scale at which they are to be portrayed. At display scales smaller than scaleMinimum the drawing instructions output from the portrayal engine must not be drawn.

For example, the drawing instructions for a feature with a scaleMinimum value of 49999, (indicating a display scale of 1:49,999), must not be drawn on an Inland ECDIS and Inland ECS with a display scale of 1:50,000.

Inland  ECDIS and Inland ECS must provide an indication if information at own vessel's position is not displayed because of applying scale minimum for display.

## Other presentation elements

In some cases, S-401 does not provide a symbology instruction in the Portrayal Catalogue that specifies how to present a specific feature on the Inland ECDIS and Inland ECS screen. The reason is that such a feature is not defined as an S-401 feature type or it would be illogical to include it in the boatmasters' navigational feature types.

The following presentation instructions are therefore provided to assist the manufacturer.

### Scale bar and latitude scale

An indication of scale and range must be part of the Display Base. The display scale determines which must be used:

- Case 1: for display scales larger than 1:80,000: always display the 1 km scale bar provided in the Portrayal Catalogue

- Case 2: for display scales at 1:80,000 or smaller: always display the 10 km latitude scale provided in the Portrayal Catalogue.

The scale bar or latitude scale must always be drawn vertically at the left side of the chart display, just clear of the border of the display. The symbols and drawing parameters are described in Table 1.

The placement must be 3mm in from the border of the display. The symbol must be properly sized to represent 1 km at the scale of the display (for Case 1) or 10 km at the scale of the display (for Case 2).

| Scale range | Symbol | Drawing priority | Display plane | Display category | Viewing group |
| --- | --- | --- | --- | --- | --- |
| 1:79,999 and larger scale | SCALEB10 | 27 | OverRadar | Display Base | 11030 |
| 1:80,000 and smaller scale | SCALEB11 |  |  |  |  |

Table 1 - Scale bar presentation parameters

EXAMPLE 1: For display scales larger than 1:80,000 (for example, a scale of 1:50,000) draw symbol 'SCALEB10' on the left side of the chart display, bottom justified and 3mm in from the border of the display. Make sure the symbol should be sized to represent 1 km at the scale of the display.

EXAMPLE 2: For display scales of 1:80,000 or smaller (for example, 1:250,000) use symbol 'SCALEB11', similarly located, and scaled to represent 10 km at the scale of the display.

IEC define a requirement for indicating the location at which the scale is calculated under certain circumstances.

### North arrow

A north arrow must be provided as part of the Display Base. The north arrow must always be shown at the top left corner of the chart display, just clear of the scale bar or latitude scale. Other requirements for the north arrow are defined in IEC 61174.

The symbol 'NORTHAR1' must be used to indicate true north. It must be placed in the top left corner of the chart display, on the inner side of the scalebar. It must be rotated to true north if the display is other than north up, and clear of the scalebar even if the latter extends the full height of the display.

| Symbol | Drawing priority | Display plane | Display category | Viewing group |
| --- | --- | --- | --- | --- |
| NORTHAR1 | 27 | OverRadar | Display Base | 11040 |

Table 2 – North Arrow presentation parameters

### Graticule

If the Inland ECDIS or Inland ECS shows a graticule (listed in “other information” the lines must use the colour token CHBLK.

### Display mode

The Inland ECDIS and Inland ECS manufacturers must provide the indication of display mode.

### Black level adjustment symbol

Unless the brightness and contrast controls of the monitor are properly adjusted there is a danger that information may be lost from the chart display, particularly at night. Symbol BLKADJ is provided for checking correct adjustment and for re-adjusting as necessary. The BLKADJ symbol is provided as a part of ECDIS Chart 1, see clause 19.5

### Detection and notification of navigational hazards

A graphical indication should also be given if the boatmaster plans a route closer than a user-specified distance from a user-selectable category of point objects, such as a fixed or floating aid to navigation or isolated danger.

Inland ECDIS and Inland ECS should give a warning or caution or indication as selected by the boatmaster and related graphical indication if, continuing on its present course and speed, over a specified time or distance set by the boatmaster, own ship will pass closer than a user-specified distance from a user-selectable category of danger (e.g. obstruction, wreck, rock) that is shallower than the boatmaster's safety contour or a user-selectable category of aid to navigation.

A graphical indication should be given if the current or the next leg of the selected route goes closer than a user-specified distance from the boundary of a user-selectable category of prohibited area or a geographic area for which special conditions exist.

The Inland ECDIS and Inland ECS must implement support for the Alert and Indications Catalogue which may be provided within each product’s Portrayal Catalogue.

The safety contour value is set by the user; in the absence of a user setting, its default value must be 30 metres.

When route planning, the Inland ECDIS and Inland ECS must provide a graphical indication of the intersection of:

- the area bounded by a user-specified distance from the route and

- the spatial components associated with alert instructions output by the portrayal.

When route monitoring, the Inland ECDIS and Inland ECS must provide a graphical indication of the intersection of:

- the area bounded by a user-specified distance from the current and next legs of the route (as a minimum) and

- the spatial components associated with alert instructions output by the portrayal.

When route monitoring, the Inland ECDIS and Inland ECS must provide an additional graphical indication of the intersection of:

- the area bounded by a user-specified distance from a vector extending from own ship in the direction of own ship’s course for a user-specified time or distance and the spatial components associated with alert instructions output by the portrayal.

Figure 2 depicts indication highlights for points, curves, and surfaces.

Figure 2 - Examples of indication highlights

### Detection of areas for which special conditions exist

A graphical indication should be given if the boatmaster plans a route closer than a user-specified distance from the boundary of a user-selectable category of prohibited area or geographic area for which special conditions exist (see appendix 4).

Inland ECDIS and Inland ECS should give a warning or caution, or indication, as selected by the boatmaster, and related graphical indication if, within a specified time or distance set by the boatmaster, own ship will pass closer than a user-selected distance from the boundary of a user-selectable category of prohibited area or of a geographical area for which special conditions exist (see appendix 4).

The Inland ECDIS and Inland ECS must implement support for Alert and Indications Catalogues.

### Definition of safety contour

The safety contour is defined as the boundary between safe and unsafe areas of the display.

### Detection of safety contour

A graphical indication is required if the boatmaster plans a route closer than a user-specified distance from own ship's safety contour.

It should be possible to select that Inland ECDIS and Inland ECS gives an alarm and related graphical indication if, within a specified time or distance set by the boatmaster, own ship will pass closer than a user-selected distance from the safety contour.

Clause 11.4.7 Route Monitoring states:

A graphical indication should be given if the current or the next leg of the selected route passes closer than a user-specified distance from the safety contour.

The Inland ECDIS and Inland ECS must implement support for S-100 Alert Catalogues as specified in S-100 9-13.4

The point, curve or surface must be graphically indicated using the presentation named as “DNGHLT” in the S-401 Portrayal Catalogue, as depicted in Figure 3.

Figure 3 - Examples of danger highlights in ECDIS

### Indications related to IENC accuracy

It should be possible for the boatmaster to select that the indications of 11.3.6 and 11.3.7

take into account accuracy information of relevant hydrographic information, as defined by IEHG

standards.

The selector used to take into account accuracy information must be called “Uncertainty”. The OEM may optionally split the uncertainty selector into separate horizontal and vertical components.

It should be possible for the boatmaster to select that the indications of 11.4.3, 11.4.4, 11.4.6, 11.4.7 and 11.4.8 take into account accuracy information of relevant hydrographic information, as defined by IEHG standards.

When the boatmaster has chosen to take accuracy of hydrographic information into account the Inland ECDIS and Inland ECS must use a zoneOfConfidence attribute to extend the check area in a horizontal direction for route plans (closer than a user-specified distance) and for own ship (within a specified time or distance set by the boatmaster, own ship will pass closer than a user-selected distance).

### Check area extension value – S-401 only

When the Inland ECDIS or Inland ECS is using S-401 without any S-102 depth substitution or S-104 depth adjustment the S-401 QualityOfBathymetricData feature contains a zoneOfConfidence complex attribute. This provides either a horizontalPositionUncertainty sub-attribute. If no horizontalPositionUncertainty value is available then the amount of extension must be taken from the sub-attribute categoryOfZoneOfConfidence according to the following table of default values:

| categoryOfZoneOfConfidence value. | Positional Accuracy |
| --- | --- |
| A1 | 5m |
| A2 | 20m |
| B | 50m |
| C | 500m |
| D | 500m |
| U | 500m |

Table 3: categoryOfZoneOfConfidence and Accuracy values.

### Check area extension value – S-102 depth substitution

When the depth information in the S-401 is substituted by S-102 the zoneOfConfidence.horizontalPositionUncertainty.uncertaintyFixed in the QualityOfBathymetryCoverage feature attribute table (S-100 Part 10c, 9.6.2) from the S-102 must be used instead of the S-401 feature information as defined in the last clause if it is set. If this value is not available then the S-401 value defined in the previous clause must be used instead.

### Multiple QualityOfBathymetricData Features

For the S-401 case, when multiple QualityOfBathymetricData features with different categoryOfZoneOfConfidence values underlay the check area, then the value of extension changes at the point where the original non-extended check area crosses the boundary of the QualityOfBathymetricData areas with different categoryOfZoneOfConfidence values, see the example below:

Fig 10. Example Extension of check-area by multiple underlaying QualityOfBathymetricData areas with different values of categoryOfZoneOfConfidence

### Enhanced Safety Contour and Water Level Adjustment.

The term “Enhanced Safety Contour” means a safety contour created from bathymetric grid data using the value set by the user. With IENCs the user sets a value for the safety contour, but if a contour of that value is not found in the IENC then the safety contour defaults to the next deepest available contour which can be substantially deeper than the value requested by the user.

Bathymetric grid data allows the user the ability to define a value and for the system to delineate areas of safe and unsafe water based on that value. The areas defined can then be used for the definition of safety contour and attendant alerts/indications. When this is done contextual IENC features with depth information are also substituted from the bathymetric grid to present the user with a harmonised picture.

The combination of S-401 with S-102 and S-104 together enable Water Level Adjustment (WLA), allowing the water level data contained in S-104 to complement the S-102 and S-401 chart data. As with depth information, WLA processes adjust attribute values in the IENC data to present the user with a harmonised picture.

Appendix D defines the detailed mechanisms for Enhanced Safety Contour and Water Level Adjustment. The Inland ECDIS and Inland ECS must support the Enhanced Safety Contour and Water Level Adjustment features.

## Other specified display features

### Legend

A standard legend of general information relating to the area displayed, applicable to the position selected by the mariner, must be shown on a graphic or text display. This legend must contain at minimum (and, as applicable):

- Units for Depth;

- Units for Height;

- Scale of display;

- Data quality indicator;

- Sounding/vertical datum;

- Horizontal datum;

- The value of the safety depth selected by the boatmaster;

- The value of the safety contour selected by the boatmaster;

- The value of the safety contour displayed (which may be different from that selected by the boatmaster);

- Method of Water Level Adjustment (if enabled);

- Magnetic variation;

- Date and number of last update(s) affecting the datasets currently in use;

- Edition number and date of issue of the datasets currently in use;

- Display projection.

Since attempting to display all the above items for all displayed data products may lead to an unduly large legend, manufacturers may suppress information from data products other than IENCs. If this is done, the suppressed information should be easily accessible, e.g. by a temporary expansion of the legend activated by clicking on a target in the legend. (Note that some of the items will be the same for all data products - in particular, units and datums should be the same for all products, or converted to present the same display result, in order to reduce the chances of user error.)

### Legend details

The legend must additionally indicate the vertical datum of any S-102 and S-104 used for Enhanced Safety Contour and Water Level Adjustment on the display, as specified in Appendix D-1.2. When the vertical datums are the same then a single indication is enough to cover S-401, S-102 and S-104.

### Inland ECDIS and Inland ECS Legend

The Inland ECDIS and Inland ECS chart legend containing the items in Table 4 must be available for display for a position selected by the boatmaster.

| Inland ECDIS and Inland ECS Legend Item | Values |
| --- | --- |
| Units for depth | Axis Unit of Measure (AXUM) subfield in the Coordinate System Axes (CSAX) field |
| Units for height | AXUM subfield in the CSAX field |
| Although the S-401 IENC Product Specification does not allow any units other than metres for depths and heights, these two elements must be stated for clarity for the boatmaster |  |
| Scale of display | Selected by boatmaster |
| Data quality indicator | - zoneOfConfidence.categoryOfZoneOfConfidence (CATZOC) attribute of the Quality Of Bathymetric Data (M_QUAL) meta-feature. When multiple temporal attributes are present: - If a single attribute value is valid for the selected viewing date range, that value must be displayed. If multiple values are valid for the selected viewing date range, the worst-case value must be displayed. When multiple features are present (to indicate bathymetric data quality at various depths): - The feature which intersects the specified safety contour value must be used. (b) Total horizontalPositionUncertainty of the Quality Of Non-Bathymetric Data (M_ACCY) meta-feature if available. Note: Due to the way quality is encoded in the IENC, both values (a and b) must be provided |
| Sounding/vertical datum | The verticalDatum attribute of the SoundingDatum feature and VerticalDatumOfData feature when available. (verticalDatum attributes of individual features must not be used for the legend.) |
| Horizontal datum | WGS84 |
| Value of safety depth | Selected by boatmaster. Default is 30 metres |
| Value of safety contour | Selected by boatmaster. Default is 30 metres If the boatmaster has selected a contour that is not available in the IENC both the contour selected and the contour displayed at the selected position must be displayed |
| WLA method | Method of Water Level Adjustment selected. The value of the dataset metadata attribute dataDynamicity of any S-104 used for Water Level Adjustment on the display. |
| Magnetic variation | MagneticVariation (MAGVAR) feature, attributes: referenceYearForMagneticVariation (RYRMGV), valueofAnnualChangeInMagneticVariation (VALACM), and valueOfMagneticVariation (VALMAG) Item must be displayed as: VALMAG RYRMGV (VALACM) For example, 4°15W 1990 (8’E) |
| Date and number of latest update affecting each dataset currently in use | Issue date and update number from the dataset discovery record (S100_DatasetDiscoveryMetadata) of the last update dataset applied. (See S-100 Part 17) |
| Edition number and date of each dataset currently in use | Edition number and issue date from the dataset discovery record (S100_DatasetDiscoveryMetadata) of the current base issue of the IENC dataset. (See S-100 Part 17) |
| Display projection | Projection used for the Inland ECDIS or Inland ECS display (For example, oblique azimuthal). This must be appropriate to the scale and latitude of the data in use |

Table 4 - Legend elements

The list above is the minimum that must be available, but the complete list need not always be shown. Individual items may be picked by the boatmaster for display for a period; examples are magnetic variation, data quality for depths, etc.

## Displaying manual and automatic updates and added information

### Manual updates

It must be possible to make the following manual updates to an IENC:

- Add feature

- Delete feature

- Modify feature

The following details must be available when the manual update object is selected by the cursor pick feature:

- All attributes available from the manual update feature catalogue

- Source of update (i.e. manual update)

- Date and time of update

- Boatmaster entered text associated with the manual update object

Manual chart updates, which are subject to human error, must be distinguished from official information and updates applied automatically. Additionally:

- Keying and Symbology. The boatmaster must be able to enter manual updates for presentation on the display as follows:

1. Enter the update so it can be displayed as described in this specification.

2. Ensure all update text information relevant to the new condition and to the source of the update, as entered by the boatmaster, is recorded by the system for display on demand.

- Indications and Alerts. The Inland ECDIS and Inland ECS must provide indications and alerts related to manual updates, using the same criteria as for IENC features.

- Presentation. The boatmaster must be able to enter manual updates to the chart using any of the symbols listed in Appendix A. Manual updates must support attribution, as well as the indications and alerts defined in Appendix A.

- Archiving of Manual Updates. It must be possible to remove from the display any manual update. The removed update must be retained in the Inland ECDIS and Inland ECS for future review until commencement of the next voyage, but will not be otherwise displayed.

### Additional Boatmaster’s Information

The boatmaster must be able to add:

- Boatmaster’s caution note (!) and boatmaster’s information note [i] used to call up a note on the text display by cursor picking

- Boatmaster’s information lines and boatmster’s information areas without colour fill, set up for cursor picking to give explanatory notes in the text display.

- Boatmaster’s text notes

- Boatmaster entered danger

IEC 61174 adds further requirements for Additional Mariner’s Information. Additional boatmaster’s Information must be presented according to IEC 61174 or IEC 62288, as applicable.

### Automatic Updates

Only ISO8211 and GML encoded data can support updates, as described in S-100 Part 10a and 10b respectively.

### Identifying automatic chart corrections on demand

On boatmaster demand automatic chart corrections of IENC information must be highlighted as described in the following sub-clauses.

#### Added Feature (automatic)

Point object: 	Superimpose symbol CHRVID01.

Curve object: 	Overwrite with line style CHRVID02.

Surface object:	Overwrite area boundary with line style CHRVID02 and superimpose symbol CHRVID01 on any centred symbol.

#### Deleted Feature (automatic)

Point object: 	Superimpose symbol CHRVDEL1.

Curve object: 	Overwrite with line style CHRVDEL2 (do not remove the original line).

Surface object:	Overwrite area boundary with line style CHRVDEL2 and superimpose symbol CHRVDEL1 on any centred symbol.

#### Modified Feature (automatic)

Point: 	Superimpose symbol CHRVID01 and symbol CHRVDEL1.

Curve: 	Overwrite with line styles CHRVID02 and CHRVDEL2.

Surface: 	Overwrite the boundary with line styles CHRVID02 and CHRVDEL2 and also superimpose symbols CHRVID01 and CHRVDEL1 on any centred symbol.

### Update Information

The S-401 Feature Catalogue also provides encoders with an Update Information feature that is used to represent a change to the information shown.

The intention of the Update Information feature is to show the extent of changes contained in either updates or new editions of S-401 datasets. This is separate to the highlighting mechanism described in the previous clause and is optionally included by data producers in IENCs.

For example, when an IENC update contains updates to various features that adjoin the feature that was actually updated, the producer will use an Update Information feature to indicate the actual feature that was affected and a brief description of the changes.

An ‘Update Review’ Context Parameter must be implemented to allow update information features to be portrayed and reviewed on demand.

### Non-official Added Chart Information

Non-official data added to existing official IENC data to augment the chart information must be distinguished from the official information as follows:

Point feature:	Superimpose symbol CHCRID01.

Curve feature:	Overwrite with line style CHCRID01.

Surface feature:	Overwrite area boundary with line style CHCRID01 and superimpose symbol CHCRID01 on any centred symbol.

Non-official data must be distinguished from manually updated chart information, which uses the same identifiers, by cursor picking.

Non-official chart information may be updated by any systematic procedure. A record of updates must be maintained.

The mariner must be able to remove all non-official chart information if the need should arise.

# Coverages and Time Series for Gridded Data (S-102 and S-104)

Coverage and time series features are encoded in the HDF5 format (see S-100 Part 10c). S-100 provides for the following types of coverage and time series data:

- Gridded data with different types of spatial grid coverages;

- Data at a set of discrete fixed points;

- Data at a set of moving platforms;

- Time series data at a set of fixed points.

For S-102 and S-104 datasets, Inland ECDIS and Inland ECS must support gridded data in DCF 2 format only (noting the one exception that S-102 QualityOfBathymetryCoverage group uses DCF 9).

Gridded data will specify either continuous or discrete interpolation (no interpolation between points) between grid points. Data for discrete fixed points, moving platform and time series at fixed points is intrinsically discrete.

## Thinning

When gridded data is portrayed using discrete symbols at each data point, the effects of scaling the display must be considered. A high-resolution display (that is, zooming in) of regularly gridded data display produces a lower density of data in the visual field.

Displaying at a low resolution (that is, zooming out) increases the density of symbols in the visual field. However, by applying a thinning algorithm, the number of symbols may be reduced.

Thinning must therefore be applied to reduce symbol density when the display is scaled.

Manufacturers may use the recommended algorithm contained in Appendix G or other algorithms of their own devising.

## Temporal variation

The metadata variables related to time are dateTimeOfFirstRecord, dateTimeOfLastRecord, timeRecordInterval, numberOfTimes, timeIntervalIndex, timePoint, startDateTime, and endDateTime (see S-100 Part 10c). The time selected for display (that is, past/present/future) will typically not correspond exactly to the timestamp (metadata variable timePoint) of the input data. For a correct display, the Inland ECDIS and Inland ECS must select the correct data.

For data with only a single timed record (where the timestamp of the earliest value equals that of the latest value) such as real-time data, the data values must be displayed only if the display time is later than the timestamp and the absolute time difference between the display time and the data timestamp is less than a discrimination interval (for example 5 minutes). For a single record, the variable timeRecordInterval can be used to set the discrimination interval.

For data with multiple times, if the selected display time is later than the first timestamp and earlier than the last timestamp, or the selected display time is in the interval [startDateTime, endDateTime], then the closest but immediately preceding values in the data must be displayed.

If the selected time is later than the last timestamp or after endDateTime, then data values at that time must be displayed only if the absolute time difference between the display time and the data timestamp is less than a discrimination interval (for example the value of the variable timeRecordInterval). If the selected display time is outside the interval [(startDateTime – discriminationInterval), (endDateTime+discriminationInterval)] then the data must not be displayed.

Some data change rapidly, so more-or-less continual revision or updating of such data is essential. For real-time observations, new values are periodically collected (for example, on the order of once every 5 minutes). For a forecast, the entire field may be created one or more times per day. Other data, such as tidal atlas or harmonic constant data are updated much less often, typically on an annual basis.

# Colours

## Colour palettes implemented by the portrayal catalogue

There are three colour palettes, all of which should be made available to the boatmaster. They are specified as part of the colour profile file(s) in the Portrayal Catalogues. They are as follows:

DAY	The "Day" palette uses a white background as a result of a comparative test outdoors in bright sunlight which showed that a display background of maximum luminance gives the best contrast achievable under near-washout conditions. This conclusion has been confirmed by subsequent sea experience.

DUSK 	The "Dusk" colour palette is based on using a black background; this palette may also be used by day as a boatmaster's option.

NIGHT 	The “Night” colour palette is intended for nighttime use. At night the light emitted by the display must be strictly limited to avoid impairing night vision. In case the luminance needs to be further reduced, the "Night" colour palette may be augmented by a luminance-reducing neutral density filter which should have 8 times attenuation, designated (logarithmically) "0.9 ND". (This is a manufacturer's option.)

# Cursor Pick Reports and Displays in Interface Panels

Inland ECDIS and Inland ECS must be capable of performing spatial queries on IENC and other S-100 data. Spatial query is understood as the ability to inspect graphical position and numerical value of spatial coordinates associated with a feature.

Spatial query must be available by means of cursor pick.

## Cursor pick rules

The rules below must be applied to all Inland ECDIS and Inland ECS cursor Pick Reports.

- Full feature and attribute names must be displayed. If a language pack is installed then the feature and attribute names must use the translated names contained in the language pack according to user preference – user language preferences are described in clause 10.3.

- Listed value names must be displayed instead of the numerical code.

- There must not be any padding of attribute values, for example, a height of 10 metres must not be padded to 10.000000 metres as this could potentially confuse or mislead the boatmaster.

- Units of measure must be included after all attribute values when available.

- Cursor pick report must extend to include both information associations and feature associations, including aggregations and compositions, which may link to information types or features that carry additional information such as a featureName.

- Dates must be given in the form “Day Month Year” DD-MMM-YYYY.

Month abbreviations must be: JAN, FEB, MAR, APR, MAY, JUN, JUL, AUG, SEP, OCT, NOV, DEC.

- The Pick Report must return information about the visible features on the Inland ECDIS and Inland ECS display. If the viewing group is turned on all features, even “no symbol” features without visible presentation within that viewing group, must be provided in the cursor Pick Report, i.e. only active drawing instructions must be included.

- Pick Report must include the information associated with the spatial object limited to the area of the cursor pick. For example, information related to accuracy.

- Complex or Simple Attributes with private feature catalogue visibility must not be included. If visibility is not defined then it must be assumed to be public.

- In addition to information returned about visible features on the Inland ECDIS and Inland ECS display, any feature which has featureReference.interoperabilityIdentifier attribute containing the interoperabilityIdentifier of the picked feature must be presented together and merged.

- Descriptions of attributes and their values, available in the feature catalogue, must be shown on demand.

## Display of dataset support files

S-100 datasets encode textual or graphical information such as diagrams and photographs in dataset support files using one of the S-100 dataset support file formats.

The pick report must enable text and graphical information to be displayed without affecting the night vision of the user. The luminance of the displayed information must be in the same band as the current palette in use at the time it is displayed.

## Pick report organisation.

### Sorting order of results

The Pick Report must be organized to facilitate navigation through complex reports in a manner logically consistent with the layering presented on the screen.

The order of features must correspond to the drawing order. Features must be prioritized using the highest priority active drawing instruction associated with the feature. Only active drawing instructions which intersect the pick area should be evaluated during sorting. In lieu of this method, the OEM may employ an alternate consistent and logical sorting method for the display of pick report results.

### Hover-over function

The Inland ECDIS and Inland ECS may provide a hover-over function for boatmasters to access important charted feature details without having to select a Pick Report. If this function is implemented within an Inland ECDIS or Inland ECS, the boatmaster must be able to turn hover-over functionality on and off.

## Tidal stream data panels

S-401 Tidal stream information in the form of speed and direction at intervals relative to the time of a “reference tide” (generally high or low water) must be formatted for display in the Inland ECDIS and Inland ECS cursor Pick Report (or other UI panel).

Table 5 shows the layout that must be used for displaying the values, encoded in the Tidal Stream Panel Data S-401 feature.

Multiple instances of Tidal Stream Panel Data features must be shown as separate tables.

The table shows how a Tidal Stream Panel Data feature and its attributes are used in the Inland ECDIS and Inland ECS display. The names of the complex attributes are shown in italics and encoded values are in blue text. The Inland ECDIS and Inland ECS display need not use the colour blue for the displayed attributes, nor display the descriptive text in italics.

| Tidal Station: (station name) Plymouth Devonport |  |  |  |
| --- | --- | --- | --- |
| Tidal Station Identifier: (station number) 0014 |  | Data From: (data product, from dataset metadata) S-401 |  |
| Stream Depth (stream depth): 1.0 | Hours(time relative to tide) | Direction of Stream (orientation value) | Rate (speed maximum) |
|  |  |  | Springs (reference tide type) |
| Before | -6 | 113 | 0.1 kn |
|  | -5 | 332 | 0.6 kn |
|  | -4 | 331 | 1.1 kn |
|  | -3 | 342 | 1.0 kn |
|  | -2 | 347 | 0.7 kn |
|  | -1 | 333 | 0.5 kn |
| High Water (reference tide) | 0 | 317 | 0.3 kn |
| After | 1 | 178 | 0.3 kn |
|  | 2 | 146 | 0.6 kn |
|  | 3 | 140 | 1.0 kn |
|  | 4 | 143 | 1.1 kn |
|  | 5 | 143 | 0.8 kn |
|  | 6 | 138 | 0.3 kn |

Table 5 - Template for tidal stream data panel

# Alerts and Indications

Inland ECDIS or Inland ECS should respond to risk of crossing, dangers, prohibited areas or areas with special conditions, during route planning (11.3) or route monitoring (11.4).

## Multiple spatial objects

If multiple spatial objects are identified then the union of those spatial objects must be highlighted. If the same spatial object identifies an alert and an indication then the drawing order must be the alert highlight followed by the drawing highlight.

# Use of Context Parameters

The Portrayal Catalogue for a product contains the set of parameters which are used in the portrayal processing for that product. This set may be different for different products. Context parameters are used for passing portrayal-related configuration information and user settings to portrayal processing, and the values of context parameters may therefore be changed by user functions in this document, or manufacturers’ custom user functions.

In order to be able to use the official Portrayal Catalogue for a data product, manufacturers must implement all the context parameters listed in the Portrayal Catalogue.

Context parameters may be added or removed by updates to the Portrayal Catalogue. Context parameters which cannot be automatically associated with Inland ECDIS or Inland ECS display functions or user interface features must be exposed to the boatmaster for use.

# Dual-fuel systems

Dual-fuel systems are systems that use both older (pre-S-100) and newer (S-100) data products that contain the same type of information (for example, S-57 based and S-401 IENCs).

All stakeholders should anticipate a transition period during which new S-100 formats are increasingly published to co-exist with older formats. An S-100 Inland ECDIS and Inland ECS must be able to handle both S-57 based and S-401 IENCs during the transition period.

## Concurrent applicability of S-52 and S-57

Dual-fuel capable systems must continue to use the principles defined in S-57 and S-52 for the presentation of chart data that conforms to S-57 instead of S-401. The principles of S-401 and this specification must be concurrently applied where S-401 data is displayed. This includes the case where one part of the chart window has S-401 data as the chart layer and another has S-57 based data as the chart layer – S-57/S-52 applies to the portion where S-57 based data is the chart layer and S-401 and this specification apply where S-401 data is the chart layer.

Seamless portrayal of S-401 data is described in Appendix E.

Overlays of S-100-based non-IENC data over S-57 based chart data are left to manufacturer discretion. Any overlays over S-57 based chart data must not interact with or modify the underlying chart data, in particular the Enhanced Safety Contour and Water Level Adjustment features must only be implemented over S-401 charts.

This specification is designed to allow such integrated displays. Note in particular:

- Any additional functions that apply only to S-100-data must be deactivated for S-57 based data.

- The principles for chart furniture and miscellaneous display elements (scale boundaries, limits of data, safety contour, depth zones, update identification, legend, etc) are the same in S-52 and this specification.

- The names of colour tokens are the same. (Future versions of this specification may add additional colour tokens, but should retain the S-52 set.)

- Management of viewing groups must be combined between S-57 and S-401. Viewing groups common to S-57 and S-100 should be managed as a single group.

Note that some elements required for complete compatibility of S-57/S-52 and S-401 presentations cannot be controlled in this specification, depending as they do on the harmonisation of Portrayal Catalogues with S-52 symbology, colour tables, and lookup tables. Among these are the shapes and dimensions of symbols and the colour coordinates assigned to colour tokens.

# Specifications for the display screen

## Physical display requirements

The requirements for the main graphic display are:

Size:	Minimum effective size of the area for chart display: 270 x 270 mm if the system is type approved as a replacement for radar..

Resolution:	Minimum lines per mm (L) given by L=864/s, where s is the smaller dimension of the chart display area. (e.g. for the minimum chart area, s=270 mm, the resolution L = 864/270 = 3.20 lines per mm, giving a "picture unit" size of 0.312 mm)

The specifications above permit a chart display whose minimum resolution (lines/mm) may vary depending on the size of the display. Maintaining a clearly readable chart display under this flexibility imposes certain requirements on the display software, which are described in clause 10.2.

Information should be displayed in the Inland ECDIS and Inland ECS on one or more physical screens, which may be divided into more than one chart display. Information may be displayed automatically, on demand or as a result of boatmaster's selection.

The physical size of the screen(s) should be appropriate to viewing conditions. Larger screens may be more suitable for situations where the operational viewing distance is higher, because the larger physical area of the display offsets the reduction of on-screen geographical extent that is caused by the greater zoom levels and symbol sizes necessitated by greater viewing distances.

Redraw during route monitoring to follow the vessel's progress, including scale changes due to change in the scale of the chart information, should take less than 5 seconds. Demands by the boatmaster that cannot be predicted by the Inland ECDIS and Inland ECS, such as draw at a different scale or in a different area may take more than 5 seconds. In the latter case:

- The boatmaster should be informed;

- The display should continue route monitoring

## Colour reproduction

Absolute accuracy in colour reproduction is not required but relative colour fidelity is important. To ensure clear contrast between colour-coded features the display screen must be calibrated before use, and this calibration should as far as possible be maintained while in service. If this is not done, lines, symbols, and area shades may become indistinct, and information may then be lost or become misleading.

## Display requirements for colours

### General

The manufacturer of a type approved Inland ECDIS or type approved Inland ECS can use any technology to build the display as long as their display fulfils the requirements of this specification.

### Display calibration and verification

The type approved Inland ECDIS or Inland ECS display must be calibrated initially in order to transform the CIE colour table coordinates to screen coordinates. The main components of the Inland ECDIS or Inland ECS display are the monitor and the image generator. Both the monitor and the image generator used to drive the Inland ECDIS or Inland ECS display can be calibrated together as a colour generating unit. Another alternative is to calibrate separately both the monitor and image generator.

The following international standards describe methods for calibration of a monitor’s RGB values to produce an output. Other methodologies may be followed, but the same verification test requirements apply regardless of method.

CIE 122-1996		Technical Report: The Relationship between Digital and Colorimetric Data for Computer-Controlled CRT Displays.

IEC 61966-3:2000	Multimedia systems and equipment - Colour measurement and management - Part 3: Equipment using cathode ray tubes, Edition 1.

IEC 61966-4:2000	Multimedia systems and equipment-Colour measurement and management – Part 4: Equipment using liquid crystal display panels, Edition 1.

The Inland ECDIS or Inland ECS display calibration must be verified for type approval of all monitors.

### Colour control, contrast and brightness controls

There are a variety of technologies available for monitors to be used for Inland ECDIS and Inland ECS display. Different technologies have different methods to control colour, contrast and brightness, but some basic rules apply for all of them.

### Effect of controls

The contrast control of a display generally shortens or extends the range of luminance available, making the display appear darker or brighter.

On the other hand, the brightness control shortens or extends the range of colour saturation available by adding white (which extends the range by making colours other than black less saturated), or subtracting white (colours become darker, more saturated.)

To the viewer, it has much the same apparent effect as contrast, but it achieves this by altering the colour contrast of the colour tables between foreground and background colours, and this may result in some features becoming harder to see, particularly at night. More importantly, making the night display more saturated may drop some dark colours below the cut-off point, into black, thus losing distinctions such as shallow versus deep water.

### Use of the controls

Colour tables are provided for day, dusk and night. The boatmaster must be provided with the control to make fine adjustments between these tables; For type approved Inland ECDIS and Inland ECS the calibration position must be marked as a reference for this.

The general principle is that all colour calibration controls of a type approved Inland ECDIS or Inland ECS must have a provision to return to the calibrated settings.

The Inland ECDIS and Inland ECS manual must carry a warning that use of the Inland ECDIS and Inland ECS with uncalibrated settings may adversely affect the visibility of information on the display.

### Setting the controls for route monitoring

The Inland ECDIS and Inland ECS display carries far more detail than a radar display, and requires correspondingly more attention to the correct selection of colour table and the correct adjustment of the controls.

This particularly affects the black-background displays, and is critically important at night, because all colours of the night table, particularly those for large areas, have to be set very dark to avoid loss of night vision, and if the control is turned down too far these colours will disappear into the black background. As a result, chart information will be lost.

The situation will be worse if one of the day tables is improperly dimmed for use at night, instead of switching to the night table.

It is important that the display be adapted to lighting conditions on the bridge by selecting the correct colour table: "Day" for bright sunlight; "Day” or "Dusk" for general daylight viewing; "Dusk" for twilight; and "Night" for the night-darkened bridge.

The controls should only be used for fine adjustment within the appropriate colour table.

To ensure that the controls are always set to a level above that at which information will be lost, the black-adjust symbol BLKADJ must be available to the boatmaster, treated as "Standard display", to be called up at any convenient point on the screen. The instructions for its use by the boatmaster must be incorporated in the Inland ECDIS and inland ECS manual.

The following text box contains an example of instruction for an LCD type of Monitor. The LCD type of monitor used in this example has only one boatmaster control, called “brilliance”. Internally the monitor has also other controls available for service engineers. These internal controls include also controls named as contrast and brightness.

First, set brilliance to calibration position. Look at the black-adjust symbol.

Then either:

- If the centre square is not visible, turn up the brilliance until it just appears;

or

- If the centre square is clearly visible, turn down the brilliance until the inner square disappears, then turn brilliance back up until the inner square is just visible again.

(If the above adjustment is not successful, select a more appropriate colour table and repeat this procedure.)

The "black level" is now correctly set. If a brighter display is required use the brilliance control, but it is better not to re-adjust the controls unless lighting conditions on the bridge change.

Note that the black-adjust symbol should be displayed to check that the inner square remains visible on the following occasions:

  - Every time that the brightness or contrast controls are adjusted.

  - Every time that the display is switched to the night colour table.

Colour display capability

Colour displays must be capable of at least 256 luminance steps in each of red, green and blue.

### Colour conversion tolerances and tests

#### Tolerances

The tolerances quoted below apply only to the process of converting CIE colour coordinates to RGB values.

Considerable operational experience will be needed before it will be possible to state colour maintenance tolerances for Inland ECDIS and Inland ECS on-board ship.

The colour tables developed have been selected to ensure maximum colour discrimination between features. Colour discrimination depends on both the colour difference and the luminance difference between two colours. Colour science (as represented by the CIE colour convention) has defined colour difference units ΔE*. The ΔE* metric is a measure of the overall discrimination (including both colour and luminance differences). As a metric for Inland ECDIS and Inland ECS colour accuracy, a measure of the discrimination in colour alone, excluding luminance differences has been defined as a subset of ΔE* referred to as Δ(u*,v*).

Δ(u*,v*) = SQRT [ (u2*-u1*)2 + (v2*-v1*)2 ]

ΔE* = SQRT [ (L2*-L1*)2 + (u2*-u1*)2 + (v2*-v1*)2]

Calculations of L*, u*, and v* must be made using as reference the chromaticity and luminance specified for the brightest white colour token in the respective colour table (Y0, u0, y0); where Y0 is the luminance in units of cd/m2. Note: this is not the brightest white of the monitor.

Note: “Δ” represents the Greek letter “Delta”, although it may appear differently on some computers.

The tolerances for the conversion of the colour tables from the CIE colours defined in these specifications to the actual RGB values for the Inland ECDIS and Inland ECS display are defined in three terms:

1. Overall discrimination between actual colours within the converted table: ΔE*. This is to ensure that all the colours within the RGB table remain discriminably separate; that is, that the relative colour distinctions have been maintained.

2. Colour discrimination differences between the defined and the actual values: Δ(u*,v*). This is to ensure that the actual RGB colours resulting from the conversion remain reasonably close to the CIE colours defined in the specifications; that is, that the blues stay blue and the greens stay green.

3. Luminance differences between the defined values and the actual values: L. This is to ensure that the luminance remains the same within acceptable limits.

Note: The CIE L*u*v* human perception colour model, which is the source of the tolerances described above, has not yet been evaluated at the low luminances of the night colour table, at which the less colour-sensitive rods of the eye take over from the daytime cones. Consequently, these tolerances should not be applied to the night table; and for type-approval purposes they are restricted to the bright sun table.

Colour tolerance values:

1. The discrimination difference between any two colours displayed (except those listed with a tabular ΔE* less than 20) must be not less than 10 ΔE* units.

| Token | Colour (x, y, L) | Token | Colour (x, y, L) | ΔE* |
| --- | --- | --- | --- | --- |
| DEPMD | (.27 .30 65) pale blue | CHWHT DEPDW UIBCK | (.28 .31 80) white | 11 |
| CHBRN | (.42 .45 30) brown | ADINF | (.41, .47 35) yellow | 14 |
| DEPMS | (.24 .26 55) medium blue | DEPVS UIAFD | (.22 .24 45) medium blue | 17 |
| DEPMD | (.27 .30 65) pale blue | CHGRF NODTA | (.28 .31 45) faint grey | 18 |

Table 6 - Tolerance exceptions

2. The difference between the colour displayed and the CIE colour defined in these specifications must be not greater than 16 Δ(u*,v*) units. If a monitor is independently tested, then the difference must not be greater than 8 Δ(u*,v*) units.

3. The luminance of the colour displayed must be within 20% of its specified value. Black is a special case and the luminance of it must not be greater than 0.52 cd/m² for bright sun colour table.

#### Instrumental calibration verification test

For type approved LCD displays, an instrumental test to check that the results of the colour conversion calibration are within tolerance should be made by

- Displaying the colours of all three colour tables (restricted to colour pairs of tabular ΔE* greater than 20)

- Measuring their CIE coordinates x,y and L

- Applying a tolerance test.

Note that since the tolerance test is intended solely to check successful colour calibration, and not to test colour maintenance at sea, this test should be performed on the bench in the manufacturer's or type-approval authority's plant under normal conditions of temperature, humidity and vibration.

Manufacturers of Inland ECDIS or Inland ECS can choose between two different methods of colour calibration.

The first method is a test of a monitor as part of an integrated system. In this method both the monitor and the image generator parts of Inland ECDIS or Inland ECS display are tested together.

The second method is an independent test of the monitor. In this method the monitor and the image generator of Inland ECDIS or Inland ECS display are separately tested against a reference (that is, the monitor is tested against a reference image generator and the image generator is tested against a reference monitor). The second method has tighter tolerance for displayed colour than the first method.

## Inland ECDIS and Inland ECS Chart 1 and Colour (Differentiation) Test Diagram

The IHO provides ECDIS Chart 1 in digital form, a graphical index of selected ECDIS symbols and symbolized lines and area boundary linestyles. This is intended to familiarize the Mariner with the colour and symbol coding used by the ECDIS. The symbols are grouped according to INT1, which is familiar to the Mariner, but are numbered with a look-up sheet, not labelled.

Since Product Specifications for data products used on ECDIS will be updated at different times, there may be a supplementary “Chart 1” for each data product other than ENC that is intended for use in ECDIS.

The ECDIS Chart 1 and any supplements are intended for use in route planning. They are not needed during route monitoring, when the Mariner can use cursor enquiry to find the meaning of symbols.

The ECDIS Chart 1 and its indexing list of symbol names and meanings arranged numerically, together with the colour differentiation test diagrams, are intended for the Mariner’s use. The use of the Colour Differentiation Test Diagrams is described in 19.6.2.

## Use of ECDIS Chart 1 and Colour Test Diagram

### Specification for ECDIS Chart 1 and the Colour Test Diagram

#### Definition

Each Chart 1 dataset must be initially displayed at the optimumDisplayScale, so as to fill all of the standard ECDIS display area (that is, the minimum 270 x 270 mm chart area).

#### Description and purpose

The ECDIS Chart 1 and the Colour Differentiation Test are for use by the Mariner and are provided in the form of an Exchange Set containing a number of S-100 datasets.

The ECDIS chart 1 is intended to familiarise the Mariner with the symbology used on ECDIS. The Mariner must be able to display each cell, and by cursor-pick get a read-out of the meaning of any feature shown.

The Colour Differentiation Test diagram is intended for display using the day, dusk or night colour tables so that the Mariner can check that the ECDIS display is providing adequate colour performance It is also used in type-approval testing. Instructions for its use are given in the following clauses.

The ECDIS Chart 1 includes the CHKSYM symbol which is intended for checking the correct size of the symbols during the type approval. The width and height of the CHKSYM is 5.0 mm.

The line width of the diagonal lines in the Colour Differentiation Test diagram are specified as 0.6 mm wide.

#### Mode of use

The operation of these diagrams is not subject to any draw-speed requirements of route monitoring.

#### Content and Encoding

Chart 1 is released as an exchange set comprising a number of S-100 datasets within the S-164 suite of test data. Portrayal and content are defined wholly by feature and portrayal catalogues.

#### Revisions

Revisions will be made by whole file replacement; that is, by the issuing of new editions. The user must be able to access the revision information of Chart 1 and the Colour Test Diagram.

#### Presentation and parameter settings

Refer to the S-164 test dataset manual for ECDIS parameter settings for portrayal of Chart 1 datasets.

### The Colour Test Diagram

A multi-purpose colour differentiation test diagram is illustrated in Figure 4. This consists of 20 squares each coloured with one of the 4 main background colour fills (such as shallow water blue); and each having a diagonal line in one of the six important foreground colours (such as mariner's orange). Each diagonal line is 0.64mm.

The diagram is in the form of a dataset and so can be displayed using any of the three colour palettes. This diagram is intended:

- For use by the Mariner to check and if necessary re-adjust the controls, particularly for use at night;

- For use by the Mariner to verify that an ageing display remains capable of providing the necessary colour differentiation; and

- For initial colour verification of the day, dusk and night colour tables.

Both the Colour Test Diagram and the instructions for its use are included within the IHO S-164 test datasets Chart 1 exchange set, and must be made available to the Mariner. In addition, a grey scale is included in 19.6 for use by maintenance technicians in checking colour tracking in an ageing display.

The Colour Differentiation Test diagram is not needed during route monitoring.

Note that the Colour Differentiation Test Diagram will not be true to colour unless it is projected on a calibrated screen and is generated using the digital format provided by IHO, which correctly reproduces the colour tokens of the Presentation Library.

The colour differentiation diagram is required in "Day" and "Dusk" colours so that the Mariner can verify that the ECDIS display monitor has the colour differentiation capability needed to distinguish between the various colour-coded areas, lines and point symbols of the ECDIS display.

Figure 4 - Colour differentiation diagram - Day

[NOTE: For illustrative purposes only. Actual ECDIS Chart 1 datasets must be used]

#### Providing the diagram

The Colour Differentiation Test diagram must be provided to enable the Mariner to verify that the display screen still retains the colour differentiation capability needed to distinguish between the various colour-coded areas, lines and point symbols of the ECDIS display.

The diagram will not be true to colour unless it is displayed on a calibrated monitor and is generated using the provided ENC portrayal catalogue.

To provide the diagram the dataset labelled “10100AACH1WOO” must be used. This dataset must be initially displayed at the optimumDisplayScale so that the extent of the imaginary chart data covers the entire ECDIS display.

The diagram consists of twenty numbered squares extending over the whole of a 270 x 270 mm screen Each square is coloured with one of the four main background area shades (such as shallow water blue, DEPVS), and each carries a two-pixel wide diagonal line in one of the important line or symbol foreground colours. These are arranged as follows:

Four main background colours:

DEPVS		(shallow water blue)				squares 3, 5, 11, 15, 18, 20.

DEPDW	(deep water, white or black)			squares 1, 7, 8, 10, 13, 19.

LANDA 	(land colour)					squares 6, 14, 17.

NODTA 	(no data shade: radar, navigation safety 		squares 2, 4, 9, 12, 16.

lines and chartwork must be visible on the

no-data part of a display)

Six important foreground colours:

DEPSC		(safety contour grey)				squares 3, 10, 17.

NINFO 		(orange, Mariner's information)			squares 5, 8, 14, 16.

ADINF 		(yellow, manufacturer's information)		squares 12, 15, 19.

TRFCD 	(magenta, traffic lanes and area 	boundaries)	squares 1, 9, 11.

RADLO 	(the lower luminance radar green)		squares 4, 6, 13, 18.

RESBL		(blue, provisionally reserved for traffic info	squares 2, 7, 20.

from transponder, VTS etc.)

The procedures for carrying out the test are documented in S-164.

### Relationship to S-57 Chart 1

The Chart 1 datasets are located alongside the existing S-57 Chart 1 cells to allow use of both in Dual Fuel mode.

### The Black-adjust symbol

The black-adjust symbol BLKADJ01 is to allow the Mariner to adjust the display for ambient illumination on the bridge of a ship. An ECDIS must make the black-adjust symbol available whenever the mariner is adjusting the display (that is, depending on the technology of the display brilliance, brightness, contrast, etc), as required by clause 18.3.

# Dataset Management

This clause covers required functionality for S-100 dataset management on the Inland ECDIS and Inland ECS to the extent that it affects the user experience and portrayal processing.

## Multiple Feature and Portrayal Catalogues

The Inland ECDIS and Inland ECS must be able to persistently install for use multiple versions of the Feature and Portrayal Catalogues for S-100 products. Catalogue management is based on the version number of the Product Specification and Catalogues

A list of the available versions of all installed feature and portrayal catalogues together with their versions must be available to the user on demand.

The Inland ECDIS and Inland ECS require an installed feature catalogue with a matching edition and revision number for each installed S-100 dataset. Multiple feature catalogues for each S-100 product are therefore required to maintain different revisions and editions of datasets within the System Database.

See Appendix B for loading and ingest processes including SSE Codes relating to catalogues.

The versioning of Feature and Portrayal catalogues follows the same rules as S-100, which uses a three-part version number (Edition.Revision.Clarification). The significance of each component is summarised below:

- Edition: New Editions introduce significant changes, such as the ability to support new functions or applications; or the introduction of new constructs or data types. New Editions are indicated by incrementing the Edition component of the version number and resetting the other components to 0.

- Revision: Revisions introduce substantive semantic changes. New Revisions are indicated by incrementing the Revision component of the version number and resetting the Clarification component to 0.

- Clarification: Clarifications are non-substantive changes. Typically, Clarifications remove ambiguity; correct grammatical and spelling errors; amend or update cross references; and/or insert improved graphics, spelling, punctuation and grammar. Clarifications must not cause any substantive semantic changes. Changes in a Clarification are minor and ensure backward compatibility with the previous versions within the same Edition. Clarifications are indicated by incrementing the Clarification component of the version number.

| Type of Change | Example | PS | FC | PC |
| --- | --- | --- | --- | --- |
| Major Change including an S-100 version change | New Concept in S-100, used in S-101 | .0.0 | .0.0 | .0.0 |
| New content | Attribute value / Feature added or removed | A..0 | A..0 | A..0 |
| FC correction, no content or portrayal impact | Clarification of definition | A.B.- | A.B. | A.B.- |

Each dataset has a product specification edition number contained within it. These are located as follows:

- ISO8211 datasets (S-100 Part 10a). The “PRED” field of the DSID record in the dataset.

- GML Datasets (S-100 Part 10b). The “ProductEdition” element of the DatasetIdentification element in the GML dataset header.

- HDF5 datasets (S-100 Part 10c) – Root Group “Product Specification” metadata value, S-100 Part 10c

And have the following common format:

INT.IHO.[S-XXX].A.B[.C] where:

S-XXX is the IHO product specification , e.g S-401, S-102 etc..

A.B and, optionally encoded, .C are the Edition, Revision and clarification numbers as above.

e.g. for an S-401 IENC dataset conforming to feature catalogue 1.4.1 the DSID PRED field contains:

INT.IEHG.S-401.1.4.1 or INT.IEHG.S-$01.1.4

Catalogues are identified by the following XML elements

- Feature Catalogue: S100XC: productid and S100XC:versionNumber

- Portrayal Catalogue: productid and version attributes of the PortrayalCatalogue root element of the portrayal_catalogue.xml contained in the root level of the compressed portrayal catalogue archive.

## Portrayal and feature catalogue compatibility

The Inland ECDIS and Inland ECS must support the ability to install and use multiple Portrayal Catalogues (including for individual products) for portrayal of the contents of the System Database.

- Compatible datasets have identical dataset Edition and Revision numbers.

- Portrayal Catalogues compatible with Feature catalogues also have identical catalogue Edition and Revision Numbers.

- Dataset and catalogue Editions with different clarification numbers but identical Edition and Revision numbers are all compatible.

- So, it is possible to process datasets conforming to one catalogue Clarification number with the Feature and Portrayal Catalogues for an earlier Clarification but with the same Edition and Revision number.

## Dataset overlaps and gaps

### Overlaps in IENC coverage

There may be cases where IENCs in the same scale range overlap. Such may be the case at agreed adjoining producer data limits, where, if it is difficult to achieve a perfect join, an overlapping buffer zone of up to 5 metres may be used.

Where an overlap of more than 5 metres between two or more datasets exists the Inland ECDIS and Inland ECS must only display one dataset for the overlap area and provide a permanent indication “overlap”.

### Overlaps and gaps between other data products

Overlaps in datasets of the same scale range in products other than the IENC must be indicated by an on-demand indication “Overlap in ??? data” (where ??? is the abbreviation for the data product as defined in the S-100 registry of product specifications (Product ID)).

An exception to this rule is S-102 and S-104 data used for Water Level Adjustment. In these cases the Inland ECDIS and Inland ECS must only select and use a single dataset from the overlapping data available and provide a permanent indication “overlap” as per IENCs.

Coverage gaps in products other than the IENC can be expected and should not be indicated.

Similarly, overlaps between different data producers’ IENC and other data products other than IENC should also be expected. Certain features, such as navigational warnings, can extend far beyond national boundaries and may therefore overlap other data producers’ IENC data. An indication is not needed in these cases.

## Updating Datasets on Inland ECDIS and Inland ECS

### Introduction

The current Editions of S-65, S-57 and S-63 continue to apply to the production and data delivery for S-57 ENC updates. S-52 Appendix I continues to apply to the processing of S-57 updates on ECDIS.

### General requirements

- Data Integrity. The Inland ECDIS and Inland ECS must be able to process updates to S-100 data products without degradation of the information content of the dataset or dataset update. For example, all information regarding attributes, logical relationships, geometry, and topology must be accounted for.

- Verification of Application. The Inland ECDIS and Inland ECS must provide a method to ensure that updates have been correctly applied to the System Database

- Log File. Inland ECDIS and Inland ECS must keep a record of updates to all S-100 data products through a log file. The log file must contain, for each update applied to or rejected by the System Database, the following information:

1. date and time of application

2. complete and unique identification of update, for example update and edition numbers for S-401 IENC

3. any anomalies encountered during application;

4. type of application: manual/automatic.

NOTE (informative): Where data products support sequential updating, sequences are determined by the update and edition numbers, not by issue date and time. Issue date/time are given in dataset discovery metadata in the Exchange Catalogue (see S-100 Part 17). Note that time is optional. Note also that it is possible for the date/time for two or more consecutive updates to be the same - in fact for data that are frequently updated, such as water levels, the date may be the same for many consecutive datasets.

### Automatic Update

- Reception of Updates. The Inland ECDIS and Inland ECS must be capable of receiving official updates, as S-100 Part 17 exchange set.

- Sequence Check. Where data products support sequential updating the Inland ECDIS and Inland ECS must ensure updates are always applied in uninterrupted sequence.

- Data Integrity and Authentication Check. All exchange set contents require the use of digital signatures and the Inland ECDIS and Inland

- .

- Summary Report. A summary report for each of the Issuing Authority's official update files must be given after completion of receipt containing at least:

1. Identification of Issuing Authority;

2. Update numbers of the update files;

3. Dataset Identifiers of datasets affected;

4. Edition number and date of dataset involved;

5. Number of updates in the affected dataset.

- Review of IENC Updates. It must be possible for the boatmaster to review IENC updates applied through displaying the System Database contents with the updates highlighted.

- Inland ECDIS and Inland ECS Update Status Report. It must be possible for the user to inspect and demonstrate the up to date status of all datasets held within the Inland ECDIS or Inland ECS against a complete list supplied by a service provider. Details of the required update status reports for both electronic chart and nautical publications datasets are given in Appendix C

## New editions, re-issues, cancellations and updates of datasets

The method for detecting whether a dataset is a New dataset/edition, re-issue, cancellation, or update is contained in the Exchange Set metadata field S100_Purpose.

The behaviour of each is described below; note that individual Product Specifications may not implement all types of dataset state. All datasets include an issue date, and optionally an edition and update number.

- New dataset – install new dataset and any included updates and record for tracking purposes.

- New Edition – replace numbered old Edition and its updates with the New Edition dataset

- Update – apply the numbered update and retain for tracking purposes. See 19.4. Where updates cannot be applied a permanent indication “Chart information not up to date” must be available in the chart display area when such a chart is in use (either displayed on chart area or used as largest scale available for chart related alerts and indications). This is also referenced by the Inland ECDIS or Inland ECS Update Status Report, Appendix C.

- Re-issue - replace the original datasets and its updates up to the date of the issue with the reissued dataset. Retention of the old issue should be as specified in the Product Specification. After a re-issue, subsequent updates may be incorporated from this reissue or from the original data kept continuously updated.

- Cancellation – cancellations are achieved either by the application of a cancellation update, or by the use of a fileless cancellation mechanism. The Inland ECDIS and Inland ECS must delete the cancelled dataset and any updates as specified in S-100 17-4.4.1 (fileless cancellations use the procedure specified in Appendix B). The system must also report any dataset(s) that have been identified as cancelled at load time. A message must be displayed informing the user of the dataset name. Depending on the method adopted by the OEM for managing cancelled datasets one of the following conditions must be observed: (1). The cancelled dataset cannot be viewed in the Inland ECDIS and Inland ECS; (2). The cancelled dataset can be viewed in the Inland ECDIS and Inland ECS with the warning message specified “Dataset <name> has been cancelled and may not be up to date. Under no circumstances should it be used for primary navigation”.

# Dataset Support Files

Exchange sets delivered to Inland ECDIS and Inland ECS only contain S-100 datasets, any required dataset support files, S-100 catalogues (feature and portrayal catalogues) and language packs, referred to as “system support files”.

The Inland ECDIS and Inland ECS must support the import of exchange set contents into the System database. There is no requirement to import or update any other content from an exchange set. This includes any of S-100’s component schemas and system support files associated with those schemas.

As defined in S-100 Part 17, each dataset support file references one or more datasets which contain a reference to it. Language packs (S-100 Part 18) reference the feature catalogue they refer to.

## Exchange set delivery

Exchange sets which contain base datasets will also contain all relevant dataset support. Exchange sets containing updates will deliver any dataset support files which are required by those updates. This may not be necessarily all the dataset support files associated with the base dataset.

## Management of automatic updates to Dataset Support files

Dataset names in S-100 are unique (S-100 Part 17). Dataset support files are referenced to one or more datasets in the exchange catalogue metadata. This allows dataset support files to be shared between multiple datasets.

The Inland ECDIS and Inland ECS must support the management of dataset support files. As with datasets, dataset support files are managed using the Exchange Set metadata field S100_SupportFileRevisionStatus. This field defines whether a dataset support file included in an exchange set is new, a replacement or should be deleted. The behaviour of each value of   S100_SupportFileRevisionStatus is described below:

- new – A new dataset support file to be installed on the system

- replacement – a replacement for an existing dataset support file with the same name.

- deletion – deletion of the dataset support file with the given name (subject to the guidance in this clause)

When a feature pointing to a text, picture or application file is deleted or updated so that it no longer references the dataset support file, the Inland ECDIS and Inland ECS must check to see whether any other feature references the same dataset support file, before that dataset support file is deleted.

Similarly, when a dataset support file is marked for deletion by a CATALOG.XML entry then the Inland ECDIS and Inland ECS must check that the support file is not used by any other datasets prior to deletion.

## Dataset Support File formats.

From all possible support file formats, Inland ECDIS and Inland ECS must at least be able to display the following (taken from the list of support file formats in S-100 Part 17 S100_SupportFileFormat enumeration)

- Plain text

- JPEG 2000

- TIFF 6.0

All dataset support files except the graphic formats use UTF-8 encoding.

System support files (feature and portrayal catalogues, and language packs) are in XML format.

Page intentionally left blank

# Appendix A – Manual Updates

## Introduction

Manual updates are data entered manually into the IENC. Manual updates are entered by the boatmaster to add, delete or modify features in an IENC. In order to provide a harmonised and consistent approach to manual updates on the Inland ECDIS and Inland ECS the set of portrayal symbols is supplied alongside this publication must be used. As a minimum, the symbols contained in the portrayal catalogue and illustrated in Table 7, Table 8 and Table 9 must be supported.

Indications and alerts must be triggered as for all official IENC data, using the alert catalogue included in the manual updates portrayal catalogue.

In order to provide a harmonised and consistent approach to manual editing and update on the Inland ECDIS and Inland ECS a set of portrayal symbols is supplied alongside this publication. These symbols, alongside an associated feature catalogue containing a recommended data structure allow the OEM to implement a manual update and editing feature on the Inland ECDIS and Inland ECS.

A visual index of the supplied symbols is provided in this Appendix in the following tables.

| Symbol Name | Portrayal Example |
| --- | --- |
| Beacon |  |
| Buoy |  |
| Caution (Object Missing) |  |
| Caution (Vicinity) |  |
| Depth |  |
| Obstruction |  |
| Other |  |
| Underwater |  |
| Wreck |  |

Table 7: Manual Update Point Symbols

| Symbol Name | Portrayal Example |
| --- | --- |
| Cable |  |
| Danger |  |
| Other |  |

Table 8: Manual update: Curve symbols

| Symbol Name | Portrayal Example |
| --- | --- |
| Caution Area |  |
| Other |  |
| Restricted Area |  |

Table 9: Manual update: Surface symbols

## Attribution of Manual Update Features

Features added by manual updates must support the attribution defined in the Manual Updates feature catalogue accompanying this publication. The supplied feature catalogue contains a restricted set of features and attributes compatible with those used for S-401 electronic charts which enable portrayal and required alert and indication processing to be carried out on manually entered updates.

# Appendix B: S-100 Data Import Error Codes and Explanations

## Introduction.

Loading, Updating, Authentication and Decryption error codes and messages are defined in this section. It is expected that application developers may also support the error conditions with an appropriate error message.

When an error occurs, this can in some instances, prevent further processing of data.

Standardised error codes must be used to communicate problems with import of datasets, data support files and catalogues into the Inland ECDIS and Inland ECS. These are defined in the following table.

| Error Code | Error Message |
| --- | --- |
| SSE 101 | Self Signed Key is invalid |
| SSE 102 | Format of Self Signed Key file is incorrect |
| SSE 103 | Signed Data Server Certificate is invalid |
| SSE 104 | Format of SA Signed DS Certificate is incorrect |
| SSE 105 | SA Digital Certificate (X.509) file is not available. A valid certificate can be obtained from the IHO website or your data supplier |
| SSE 106 | The SA Signed Data Server Certificate is invalid. The SA may have issued a new public key or the dataset may originate from another service. A new SA public key can be obtained from the IHO website or from your data supplier |
| SSE 107 | SA signed Data Server Certificate file is missing. A valid certificate can be obtained from your data supplier” |
| SSE 108 | SA Digital Certificate (X.509) file incorrect format. A valid certificate can be obtained from the IHO website or your data supplier |
| SSE 109 | Dataset Resource Digital Signature is invalid |
| SSE 110 | Dataset Permits not available for this Data Server. Contact your data supplier to obtain the correct permits. |
| SSE 111 | Dataset Permit not found. Load the permit file provided by the data supplier. |
| SSE 113 | Dataset Permit is invalid (checksum is incorrect) or the Dataset Permit is for a different system” Contact your data supplier and obtain a new or valid permit file. |
| SSE 114 | Incorrect system date, check that the computer clock (if accessible) is set correctly or contact your system supplier. |
| SSE 115 | Subscription service has expired. Please contact your data supplier to renew the subscription licence |
| SSE 117 | User Permit is invalid (checksum is incorrect). Check that the correct hardware device (dongle) is connected or contact your system supplier to obtain a valid User Permit. |
| SSE 118 | HW_ID has incorrect format |
| SSE 120 | Subscription service will expire in less than 30 days. Please contact your data supplier to renew the subscription licence |
| SSE 121 | Decryption failed no valid dataset permit found. Permits may be for another system or new permits may be required, please contact your supplier to obtain a new licence |
| SSE 122 | One or more of the SA Digital Certificate or Domain Controller certificates (X.509) have expired. A new SA public key can be obtained from the IHO website or from your data supplier |
| SSE 123 | Non sequential update, previous update(s) missing try reloading from the base media. If the problem persists contact your data supplier |
| SSE 125 | The permit for dataset < name> has expired. This dataset may be out of date and MUST NOT be used for Primary NAVIGATION |
| SSE 126 | This Dataset is not authenticated by the IHO acting as the Scheme Administrator |
| SSE128 | Error processing <file name>. The format or content could not be parsed and it could not be installed |
| SSE129 | This Catalogue is not authenticated by the IHO acting as the Scheme Administrator |
| SSE131 | Error: No Feature Catalogue or Portrayal Catalogue could be found compatible with dataset <dataset name> |
| SSE132 | Signature Path can not be validated. One or more of the Domain Controller or SA digital certificates is missing or invalid |
| SSE133 | Version mismatch between <dataset> and <catalogue>. Only <catalogue version> is supported for data of this type (<product>) |
| SSE134 | This Exchange Set is not authenticated by the IHO acting as the Scheme Administrator |
| SSE135 | This Permit file is not authenticated by the IHO acting as the Scheme Administrator |
| SSE136 | Fileless cancellation metadata is incorrect for <filename>. |
| SSE137 | No Data Producer digital signature is present for dataset <filename> |

Table B-1 - Error codes and messages

## Error code descriptions.

SSE 101 must be returned when a self signed key (SSK) cannot be validated against the public stored as part of the SSK. The data server must check that its own SSK is valid before sending it to the SA. The SA will confirm the data server SSK before returning the SA signed data server certificate.

SSE 102 is returned if the SSK is wrongly formatted or if mandatory elements are missing. SA and data servers must complete this check.

SSE 103 is returned if the Signed data server certificate does not authenticate correctly against the SA public key or any Domain Controller Intermediaries. This validation process must be carried out by the SA or Domain Controller before supplying it to the data server. The data server must also validate a received certificate. The data client must validate the digital signature against any file prior to its use.

SSE 104 must be returned if the SA signed data server certificate is wrongly formatted. This must be carried out by the data server on receipt from the SA.

SSE 105 must be returned if there is no SA certificate installed on the data client.

SSE 106 must be returned if the SA digital certificate (public key) does not validate against the following:

- SA digital certificate will not validate against the SA public key.

- One or more of the Domain Controller certificates do not validate against the SA certificate.

SSE 107 must be returned if the SA signed data server certificate is not available to the data server for checking or is not present in the Exchange Set Catalogue file when the data client attempts to authenticate it.

SSE 108 must be returned if the SA public key held in the SA digital certificate is wrongly formatted or the certificate file is unreadable.

SSE 109 must be returned if any resources (dataset or supplementary file) in the Exchange Set Catalogue do not authenticate against the SA authenticated data server public key or Domain Controller authenticated public key contained in the Exchange Set Catalogue metadata.

SSE 110 must be returned if there are no dataset permits available for a particular data server corresponding to encrypted data within the Exchange Set being loaded.

SSE 111 must be returned if there are no permits installed on the system for a requested dataset.

SSE 113 must be returned if the calculated CRC of an individual dataset permit does not validate against the CRC held in that cell permit. [Data Clients] This may be a HW_ID problem, corruption during transmission or the permits are for a different system.

SSE 114 must be returned if the system date does not agree with the date obtained from any alternative, reliable date source; for example, GPS. [Data Clients]

SSE 115 must be returned if the expiry date of the dataset permit has an earlier date than that obtained from the validated system date. [Data Clients]

SSE 117 must be returned if the CRC contained in the User Permit does not validate against the calculated CRC of the extracted HW_ID. [Data Servers]

SSE 118 must be returned if the if the decrypted HW_ID extracted from the User Permit is incorrectly formatted. [Data servers]

SSE 120 must be returned if the subscription licence is due to expire within 30 days or less.

SSE 121 must be returned if a valid dataset key (decryption key) cannot be obtained from the relevant dataset permit to enable the system to decrypt the corresponding dataset.

SSE 122 must be returned if the SA Digital Certificate or any of the intermediate Domain Controller Certificates have expired. “Expired” is when the X.509 “Valid to” date in the certificate is older than the validated system date.

SSE 123 must be returned if a dataset update being imported is not sequential with the latest update already contained in the ENDS/System Database for any given dataset. Under these conditions the update process (for the dataset) must be terminated and the Inland ECDIS or Inland ECS is to display a warning when the dataset is displayed stating that it is not up to date and should not be used for navigation.

SSE 125 must be returned if the stored permit for any given dataset has expired. It should be possible to view the dataset but a permanent warning message must be displayed informing the user; for example, “The permit for dataset <name> has expired. This dataset  may be out of date and MUST NOT be used for Primary NAVIGATION”.

SSE 126 must be returned if a signed resource (dataset, catalogue or permit) in an Exchange Set is authenticated against a certificate or public key file stored on the Data Client other than the one provided by the SA. In a chain of digital signatures the root certificate must be the SA certificate.

SSE 128 must be returned if one or more dataset resources (datasets, catalogues or permits) could not be loaded because the contents could not be validated according to the versions of the (XML) Schemas installed on the system. Only conformant content may be loaded onto any S-100 system.

SSE 129 must be returned if the Catalogue to be installed (Feature or Portrayal) has not been directly authenticated by the Scheme Administrator (as identified according to clause 8.1.1).

SSE 131 must be returned if a dataset has no Feature Catalogue or Portrayal Catalogue installed.

SSE 132 must be returned if any digitalSignatureValue for a resource (whether dataset, supplementary file or permit) is missing or fails validation.

SSE 133 must be returned if the dataset being installed is of a version which is not compatible with a Feature Catalogue or Portrayal Catalogue installed on the system.

SSE 134 must be returned if the CATALOG.XML digital signature (CATALOG.SIGN) cannot be authenticated against the Scheme Administrator’s certificate, either through the data server’s signed public key or any specified intermediate Domain Controllers. The contents of the Catalogue must not be installed if the signature is invalid.

SSE 135 must be returned if the PERMIT.XML digital signature (PERMIT.SIGN) cannot be authenticated against the Scheme Administrator’s certificate, either through the data server’s signed public key or any intermediate Domain Controllers.

SSE 136 must be returned if a received fileless cancellation contains digital signatures which do not match those received with the dataset being cancelled.

SSE 137 must be returned if a received fileless cancellation has no DataProducer signature stored with its original installation. This message may also be shown if a dataset is received with no Data Producer signature.

SSE 138 must be returned if a received fileless cancellation has no cancellation signature contained in the dataset discovery metadata. A cancellation signature is a digital signature of the original  DataProducer signature (as required under clause 8.1.1). This message may also be shown if a dataset is received with no Data Producer signature.

The flow diagrams in this Appendix show the conditions under which the SSE codes are displayed. These diagrams only relate to processes carried out on the Inland ECDIS and Inland ECS for import of exchange set contents. Data Server and Scheme Administrator processes are not defined here.

## Exchange Set Installation on Inland ECDIS and Inland ECS

This section describes the sequence of actions to import Exchange Set contents. This content is always digitally signed and may optionally be encrypted.

Before exchange sets are imported to Inland ECDIS and Inland ECS the CATALOG.XML must initially be authenticated against the digital signature contained in the CATALOG.SIGN file. This is described in the following flow diagram.

Figure 5: Verification of CATALOG.SIGN

If encrypted data is to be imported into the Inland ECDIS and Inland ECS, before any permits are imported to Inland ECDIS and Inland ECS the PERMIT.XML must initially be authenticated against the digital signature contained in the PERMIT.SIGN file.

Figure 6: Verification of PERMIT.SIGN

Once the CATALOG.XML is authenticated and, where encrypted data is to be imported, the PERMIT.XML is first authenticated, then the contents of the exchange set may be imported.

Encrypted data is imported into the System Database. For encrypted data the following process is used to establish the validity of the PERMIT.XML contents (after the PERMIT.XML is successfully authenticated against the digital signature in the PERMIT.SIGN) and its contents in respect of the end user’s subscription details. This is only for use with datasets which are encrypted and should be carried out prior to dataset file decryption according to S-100 Part 15.

Figure 7 - Check for permit validity prior to dataset decryption

Once encryption and compression has been processed the digital signatures for all Exchange Set contents to be imported can be processed. These signatures can form a chain of authentication and the following flow diagram illustrates how signatures must be processed.

Figure 8 - Authentication of Exchange Set dataset components

Under S-100 Part 15 datasets and other Exchange Set components may have multiple digital signatures, with each signature referenced to a signed public key certificate. Each Exchange Set contains all relevant certificates with the exception of the SA digital certificate which is installed separately by the end user.

Using the flow shown in Figure 8 the chain of certificates for dataset components, permits and the CATALOG.XML itself can be authenticated.

If more than one signature exists for a dataset or dataset support file then all signatures must be verified before successful import.

Full documentation of authentication processes and technical details are contained in S-100 Part 15.

Certain Exchange Set contents control the behaviour of the Inland ECDIS and Inland ECS and may only be digitally signed by the Scheme Administrator. These categories of content comprise the Feature Catalogues and Portrayal Catalogues should be authenticated using the following process.

IHO standards determine the content and revision of such Catalogues. The following flow diagram describes the authentication of catalogues prior to their import into the Inland ECDIS and Inland ECS.

Figure 9 - Authentication of Feature and Portrayal Catalogues

Fileless Cancellation of datasets is achieved by the receipt of a dataset discovery metadata entry with no accompanying datafile but with a digital signature matching the original Data Producer signature, and the S100Purpose set to “cancellation”. Additionally a cancellation signature, a signature of the original Data producer signature must be present and valid.

Prior to cancellation this signature must be checked by the S-100 Inland ECDIS and Inland ECS to ensure it matches, according to the following flow diagram.

Figure 10: Fileless Cancellation of datasets.

# Appendix C  -  Inland ECDIS and Inland ECS Update Status Reports

# Purpose

This Appendix elaborates the definition of an “up to date” dataset. All Inland ECDIS and Inland ECS users are required to be able to demonstrate to a vetting inspector that their Inland ECDIS or Inland ECS and its installed datasets are “up to date”.

This Appendix specifies the format and content of an “Update Status Report” which will demonstrate the revision status of datasets within the System Database of the Inland ECDIS and Inland ECS. The Inland ECDIS and Inland ECS must, therefore, implement an Update Status Report:

- An IENC Update Status Report, which reports the contents of electronic charts and directly related information, S-102 and S-104;

Use and Responsibility

An Update Status Report is designed as a concise and standardised format to assist end users in satisfying themselves that their data is “up to date” and help satisfy inspection requirements in that respect.

The report is designed for two individual use cases:

- To ensure that all navigational data loaded into the System Database is up to date for a section of a particular route;

- To ensure that all navigational data loaded into the System Database is up to date.

All S-100 datasets use Edition and, sometimes, update numbers to denote their individual revision (see S-100 Part 17 for details). These numbers uniquely identify how up to date a dataset is. The difficulty with using Edition/update numbers in isolation is that they do not document the date of revision of all datasets relative to the installation date of a data server’s service giving the end user no indication of whether a dataset (or a collection of datasets) is up to date with respect to the last dataset installed.

The driving use case for the Update status report is that an inspector (for example flag State, port State or vetting) or end user can check that an Inland ECDIS or Inland ECS is up to date with respect to the last dataset installed from each data server’s data within the System Database.

Note that this report specification is not a concrete definition of what an “up to date” dataset (or Inland ECDIS or Inland ECS) is; it only shows a status of each dataset with respect to the last data installed on the system. If a period of time has passed since the last update then some data may well be “out of date” relative to the time the report is run – the report only records the up to date status relative to the last data installed (the date being defined by the issue date of the latest S-128 dataset). The equipment manual should explain terms “up to date” and “out of date”.

This report only reports the status of datasets delivered as part of a data service using S-100 Part 17 Exchange Set metadata and (as part of dual-fuel mode) S-57 based datasets (as illustrated in the hybrid Exchange Set in S-100 Part 17, clause 17-4.2). Where other S-100 datasets are installed on the system from other sources (for example, Unencrypted IENCs) then they should be displayed with a status of “unknown”. The Equipment manual should explain terms “unknown”.

This Appendix provides a specification for a report which can be run on a compliant Inland ECDIS and Inland ECS to determine the status of S-100 datasets held within the System Database.

Completeness and Up-to-datedness

Report Definitions

Report Contents

The mariner must be able to select the type of report to be created:

- An IENC Update Status Report, which reports the contents of electronic charts and directly related information. This report shows the status of IHO S-100 datasets of the following product types:

  - IENC, either S-401 or S-57 based

  - S-102

  - S-104

- An ENP (Electronic Nautical Publication) Update Status Report which reports the status of ENPs. All other S-100 products listed in 6.1.1 installed in the System Database are in this category.

The report name at the top of the report header shows the type of Report being produced.

Report Filters

The boatmaster must be able to select the report filter to be one of the following:

- A report detailing the status of all datasets within the System Database of the Inland ECDIS or Inland ECS for the appropriate type (IENC).

- A report detailing the status of each S-100 dataset along a predefined section of a route contained within the Inland ECDIS or Inland ECS.

Example

An example of an Update Status Report for current Inland ECDIS or Inland ECS with S-57 base IENCs is shown in the Figure 1, followed by an illustrative revised report for S-100 datasets in Figure 2. The example shown is an IENC Update Status Report.

Figure C-1-1 - Update Status Report – current report example

Figure C-1-2 - Update Status Report - notional example for S-100 datasets

Report specification

The IENC Update Status Report is a formatted output from the Inland ECDIS and Inland ECS. It should be formatted to fit within the width of the Inland ECDIS or Inland ECS screen and, if printable, split into individual A4 pages.

Reports of both types (complete System Database contents and filtered by route) are divided into two sections:

- A header containing information on the vessel and report content selected (either filtered for a route (or part of a route) or for the entire System Database contents. This header contains vessel information and report reference dates followed by a summary totals for each report status.

- A table containing information for each cell within the chosen content type. This provides detailed information on each dataset within the chosen content category.

Report header

The data content of each of the header fields is defined in Table 3 below:

| Field Name. | Type | Source |
| --- | --- | --- |
| - Report Name | Text | Two Report names are possible depending on which Report is being shown. Either: - Inland Electronic Navigational Charts (IENC) Update Status Report |
| - Vessel Name | Text | The name of the vessel as recorded within the Inland ECDIS or Inland ECS. |
| - Identifier | Text | A unique identifier, the MMSI, vessel IMO number or other identification number. |
| - Date of report | Date | The date the Report was run. |
| - Content | Text | Each Report can be optionally filtered for an individual route plan or report the full System Database contents for each Report type: - “Filtered for Route Plan XXX to YYY” where XXX and YYY are the textual names of the point of origin and destination on the chosen route. - Full System Database contents. |
| - Start WP | Text | This field is only present if the Report is filtered for a route. It should comprise the textual name of the starting waypoint of the route (if one exists) and the lat/long coordinates of the waypoint. There is no fixed form that the coordinates should take. |
| - End WP | Text | This field is only present if the Report is filtered for a route. It should comprise the textual name of the last waypoint of the route (if one exists) and its lat/long coordinates. There is no fixed form that the coordinates should take. |

Table C-1-1 - Data content of header fields

Filtering of Update status report for route section

Where an Update Status Report is filtered for a route plan then the datasets of the appropriate type (IENC) in the System Database whose status are checked are defined by the intersection of the route corridor with the dataset boundaries (as defined by dataset’s coverage features (or coverage defined in the S-100 CATALOG.XML within the System Database)).

The width of the filtering corridor is equal to the “user specified distance” implemented inside the Inland ECDIS and Inland ECS to fulfil IMO MSC.532(106) 11.3.7:

“A graphical indication should be given if the boatmaster plans a route closer than a user-specified distance from the boundary of a user-selectable category of prohibited area or a geographic area for which special conditions exist (see appendix 4). A graphical indication should also be given if the mariner plans a route closer than a user specified distance from a user-selectable category of point objects, such as a fixed or floating aid to navigation or isolated danger. The user-selectable categories should be the same as the user selections for the display of objects and be based on IEHG standards. There should be a permanent indication when any user-selectable categories are deselected. Details of the deselection should be available on demand.“

This is not necessarily the same as the XTD. distance.

Summary totals

The summary section of the Report follows directly after the header. The summary contains the following information:

- The title: “Chart Status Summary”.

- Totals of cells with the relevant status in the order defined below.

  - Total – the total number of datasets available in the System Database for the content type (IENC), selected for the Report (either full or filtered by route).

  - Up to date – the total number of datasets (for the content selected) which have status “Up to date”.

  - Not up to date – the total number of datasets (for the content selected) which have status “Not up to date”.

  - Withdrawn – the total number of datasets (for the content selected) which have status “Withdrawn”.

  - Cancelled – the total number of datasets (for the content selected) which have status “Cancelled”.

  - Unknown – the number of datasets for which a status cannot be determined for any reason.

- The possibilities for each dataset’s status are listed in Table 4 below along with their definitions.

Data Server content tables

The detailed tables in each Report are arranged by data server – each separate data server or dataset data source within the System Database has its own separate table listing all datasets by content type (as reported in the “Content” field in the report header) and S-100 Product Specification order.

The detailed tables contain the following information:

- Title: Data Server Name – this is the data server identified by the S-128 Producer Code (which can either be a data producer, an aggregator or data aggregator (as identified by clause 8.1.1)

- For each cell installed in the System Database from the data server:

  - Dataset Name – the name of the dataset. (DSNM).

  - Edition – the Edition of the dataset in the System Database (EDTN).

  - Update – the update number of the dataset in the System Database. If the product does not support updates, then this must be labelled “-“.

  - Issue Date – the issue date of the last applied update to the dataset in the System Database (or “-“ where no updates are supported for the S-100 product).

  - Status – the status of the cell. The status may have one of four values determined according to the criteria in the following table:

| Status | Specification |
| --- | --- |
| Withdrawn | The number of datasets which have been withdrawn by the data server or cancelled but which are still available within the System Database. |
| Unknown | Datasets for which a status cannot be determined for any reason. If the revision information in the latest S-128 dataset is incomplete for any reason then all datasets in the data server’s service not included in the partial S-128 must be deemed to be “Unknown” as no definitive information on them can be determined. S-128 content is required to specify the status of all datasets in a data server’s service. |

Table C-1-2  - Status values

Example of ENC update status report

Report Name: ENC Update Status Report

Vessel Name: HMS Goteborg

Identifier: IMO 4653321

Update Reference Date: 16 May 2013

Date of Report: 1 Jun 2013

Content: Full

Chart Status Summary:

Chart Status: Count

Total: 50

Up to Date 38/50

Not Up to Date 10/50

Withdrawn 2/50

Unknown 0/50

Table:

Data Server: XXXX

| Product | Dataset Name | Edition | Update | Issue Date | Status |
| --- | --- | --- | --- | --- | --- |
| S-101 | 101US23495820 | 10 | 4 | 2020-01-02 | Up to Date |

Example of ENP update status report

Report Name: Electronic Nautical Publications (ENP) Update Status Report

Vessel Name: HMS Goteborg

Identifier: IMO 4653321

Update Reference Date: 16 May 2013

Date of Report: 1 Jun 2013

Content: Full

Chart Status Summary:

Chart Status: Count

Total: 50

Up to Date 38/50

Not Up to Date 10/50

Withdrawn 2/50

Unknown 0/50

Table:

| Data Server: XXXX |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product | Dataset Name | Edition | Update | Issue Date | Status |
| S-123 | 123US3245394 | 5 |  | 2020-01-02 | Up to Date |
| S-127 | 127US2345032 | 6 |  |  |  |

# Appendix D – Enhanced Safety Contour and Water Level Adjustment

# Enhanced Safety Contour

This Appendix defines how the “Enhanced Safety Contour” (ESC) feature is defined and implemented.

Enhanced Safety Contour, or ESC, means creation of the safety contour from the bathymetric grid data based on the value set by the user.

NOTE: On an S-100 Inland ECDIS and Inland ECS with the Enhanced Safety Contour feature switched off (or an S-57 based Inland ECDIS or Inland ECS), the user sets the value for the safety contour, but if the exact value is not found from the available depth information then the safety contour defaults to the next deepest which may be over 10 metres deeper than the value set by the user. The Enhanced Safety Contour feature addresses this issue.

The contents of this Appendix are not currently integrated with the existing S-100 portrayal mechanisms specified in S-100 Edition 5.2 Part 9. This may be rectified in future editions of S-100. Until then, this appendix contains the normative specification for these features and may use different language, terms and conventions than S-100 itself.

In these processes an end user selects:

- Suppression of specific S-101 features by S-102.

- A Safety Contour value of their choice.

When a user selects a safety contour value, outside areas of S-102 coverage the source for the displayed safety contour is the existing S-101 ENC (or S-57 ENC in dual-fuel mode) as defined in 11.6.1.

When the safety contour has been generated from S-102 data then the area over which it is generated, that is the S-102 coverage area, must be surrounded by a boundary line using colour token DEPWL  see Figure D-1-1.

Figure D-1-1 - Boundary line of S-102 coverage

# Provision of Sun Illumination

Sun illumination may be provided. Where provided:

- The mariner must be able to select whether it is enabled or not;

- Selection of Base or Standard Display must disable sun-illumination;

- The ECDIS must use a consistent sun azimuth and elevation values for all data portrayed;

- The mariner should be able to vary the sun azimuth and elevation values.

# Control over the use of S-102

The ECDIS must only allow the use of S-102 for the application of ESC if at least one of the following conditions is met between the S-102 data and the extent of the ENC data it spatially coincides with:

- The data producer of the S-102 data is the same as the data producer of the ENC data; or

- The S-102 dataset has a digital signature issued by the ENC producer; or

- The ECDIS has a certificate issued by the ENC producer with the S-102 data producer as its Subject.

Where certificates and identities are defined by clause 8.1.1

# Data Constraints

- Portrayal of S-102 data and application of ESC must be restricted by minimum and maximum display scale values contained in the exchange catalogue as specified in clause 11.2.

- Coverage – S-102 data is not expected to overlap, but in case of an overlap of greater than one grid cell the ECDIS must indicate an overlap by the text “OVERLAP” and the user must have the ability to select which producer in the overlapped area has priority and will be selected for processing safety contour. An alternative mechanism for selecting which producer in the overlapped area has priority may also be implemented by the ECDIS.

  - Only datasets with overlapping scale ranges (as defined in clause 11.2) can contain overlapping data. Overlaps are then defined between spatially overlapping grid cells containing data, as opposed to grid cells with no-data (fill) values.

  - Datasets may contain overlapping grid cells, and different datasets may overlap, most commonly at national borders, but also in areas where there are multiple vertical datums within a single dataset. In these cases:

    - Overlaps within the same dataset (in areas with multiple vertical datums) the overlap must be resolved by taking the shoalest value for each grid cell (including vertical uncertainty when the Uncertainty selector is enabled) within the intersection. This is also used to resolve situations when the Mariner Selected Viewing Scale (MSVS) means that multiple grid cells are to be portrayed at the same screen position (the common point rule) – most often when the grid is being portrayed at a smaller zoom scale than the dataset resolution;

    - Overlaps between different producers. The end user must be able to select which dataset (or data producer) they wish to use.

- No complex interpolation is required between points in the S-102 grid. Nearest neighbour is used to define the depth in each S-102 point's neighbourhood (that is, the same depth everywhere within the grid square), regardless of any interpolationType defined for the S-102 dataset (S-100 Part 8, clause 8-7.7.3), as illustrated by Figures D-1-2 and D-1-3:

Figure D-1-2 - The extents of the S-102 points overlaid on the S-102 grid (grid spacing = ‘d’)

Figure D-1-3 - Extents of each S-102 points 
showing nearest neighbour interpolation (S-100 Part 8-7.1.4)

- Only rectangular grids for S-102 are allowed. The method outlined may be extended to other grids; for example, triangular grids in the future.

- Enhanced Safety Contour must only be provided in areas where the Sounding Datum of suppressed S-101 features is the same as the vertical datum of the S-102 Coverage.

- When the Uncertainty selector is enabled vertical uncertainty information must be taken into account when extracting depth data from S-102 datasets.

# Enhanced Safety Contour Selection and Implementation

The user must be able to select S-102 to be used as the source of the depth information and the Enhanced Safety Contour.

The Enhanced Safety Contour must be enabled by default.

When S-102 is selected for use, in areas where S-102 coverage exists, the S-102 suppresses the drawing instructions associated with the following S-101 depth features

- Depth Area

- Dredged Area

- Depth Contour

This also suppresses the safety contour, the shallow water pattern, Accuracy portrayal, text indicating the dredged depth and contour labels.

The process to construct the enhanced safety contour then consists of a selection of common edges from rectangular extents for each S-102 dataset point (including vertical uncertainty if the Uncertainty selector is selected), as shown in Figure D-1-4, where 11.2 metres is the threshold value set by the user.

Figure D-1-4 - Common edges from rectangular extents

Common edges are selected when the S-102 points on either side of them lie on either side of the boundary of the safety contour value. In Figure D-1-5 below, 11.5 metres is the threshold value set by the user. Edges on the boundary of the dataset, or which lie between data values and S-102 fill values are not selected. See figure D-1-5

Figure D-1-5 - Edges from rectangular extents and at boundaries

The source of the displayed safety contour are the selected edges as defined. The areas of safe and unsafe water formed by the selected edges are used for processing of the safety contour related alarms under IMO MSC.530(106)/Rev.1 11.4.3, which the OEM must implement.

The border defined in clause D-1 must be selected from data values only, which border the dataset, or around data fill values (S-102 No Data Value) which are not on the dataset border, for example:

Figure D-1-6 – Construction of S-102 border

The combined effect of these drawing operations is shown in the following diagram:

Figure D-1-7 – ESC construction

# Vertical Uncertainty in the calculation of depth values.

When the Uncertainty selector is enabled calculation of depth values in S-102 datasets must take into consideration the vertical uncertainty contained within the dataset for all depth values. Vertical uncertainty is expressed as a positive number which indicates the maximum vertical uncertainty (and hence can be added or subtracted from a depth value contained in the S-102 dataset).

If the vertical uncertainty is stored in the grid cells themselves then each S-102 depth has its own uncertainty value which must be taken into account when extracting the depth values from the dataset.

If no uncertainty values are stored within the grid data then the value of maximumUncertainty must be used, these will be equal in a dataset with no separate uncertainty grid values.

If the maximumUncertainty value is equal to the dataset fill value, the depth value calculation must be handled as if the Uncertainty selector is not enabled.

Vertical uncertainty must only be taken into account when the Uncertainty selector is enabled.

Uncertainty values must be used in such a way as to safety-bias the depth calculation. Depth is encoded “positive-down” and therefore the uncertainty value must be subtracted from each of the individual depth values in order to achieve the required safety-bias.

With the user option “uncertainty” switched on:

Depth 	= (S-102 depth corrected for uncertainty) + (S-104 depth corrected for uncertainty)

= (S-102 depth value – S-102 uncertainty value) + (S-104 depth value – S-104 uncertainty value)

For example, given the following S-102 and S-104 depth and uncertainty values:

| S-102 depth | 10.00 m |
| --- | --- |
| S-102 uncertainty | ±0.42 m |
| S-104 water level height | 0.32 m |
| S-104 uncertainty | ±0.04 m |

Table 10: Example Values - Water Level Adjustment

The calculation is :

Depth 	= (10 – 0.42m) + (0.32m – 0.04m)

= 9.86m

With the user option “uncertainty” switched off, the calculation is simply:

Depth 	= (S-102 depth) + (S-104 depth)

Depth = 10m + 0.32m = 10.32m

# Water Level Adjustment (WLA)

The user must be able to select depth information adjustment by water level height in areas where both S-102 and S-104 data products are available. This adjustment must then be applied in areas where compatible S-101, S-102 and S-104 data products are available

- The system must default to no water level adjustment.

- The mariner must be able to select one of the following methods of depth adjustment:

- Current (or user selected) date and time;

- A mariner specified date and time period;

- Where the route includes a schedule, the date and time of transit in each area along a route

- When Water Level Adjustment is activated as defined in this section:

- The functionality and portrayal of the safety contour, depth zone shades, safety depth and indication of isolated dangers must use the adjusted depth;

- The ECDIS pick report must indicate both adjusted and unadjusted depth;

- Details of the Water Level Adjustment must at least be available on demand, such as the data source, temporal extent, and applicable areas;

- It must be possible to de-activate Water Level Adjustment via simple operator action;

- There must be a permanent indication as described in D-2.5, D-2.6 and D-2.7.

- S-104 vertical uncertainty values must additionally be used in all depth calculations if the Uncertainty selector is enabled.

- The ECDIS voyage recording must include:

- The state of Water Level Adjustment (method applied);

- The mariner specified date and time or date and time period when in use for WLA, or the scheduled date and time, the check area distance and the time resolution when in use for WLA.

# Scope

This clause defines how the “ECDIS Water Level Adjustment feature” is implemented. Water Level Adjustment is referred to in this section as WLA. This section defines how the adjustment of depth information by water level is provided for:

- Selected single date and time;

- Selected date and time period;

- Linked to an estimated route schedule with selected check distance and time resolution.

# Constraints on input data

- Coverage – As for S-102 data, S-104 datasets are not expected to spatially overlap within the same scale range, but in case of an overlap of greater than one grid cell the approach must be same as for overlapping S-102 data. The ECDIS must provide a permanent indication of “Overlap” and the user must have the ability to select which producer in the overlapped area has priority and will be selected for processing WLA.

- As for S-102 data, the ECDIS must only allow WLA processing if at least one of the following conditions is met between the S-104 data and the extent of the ENC data it spatially coincides with:

- The data producer of the S-104 data is the same as the data producer of the ENC data; or

- The S-104 dataset has a digital signature issued by the ENC producer; or

- The ECDIS has a certificate issued by the ENC producer with the S-104 data producer as its Subject.

Where certificates and identities are defined by clause 8.1.1

- As with S-102, each S-104 point is assigned a rectangular extent with nearest neighbour interpolation.

- As with S-102, each S-104 value used in calculations must additionally take into account the vertical uncertainty values contained within the dataset when the Uncertainty selector is enabled.

- WLA must not be applied unless the S-102 and S-104 features are on the same vertical and horizontal datum. If the S-102 and S-104 features are not on the same vertical and/or horizontal datum then an indication “Incompatible datums” must be given to the user.

# User Inputs

When WLA is selected for use, in areas where WLA coverage exists, one of the three different options must be used:

- WLA Option 1: WLA at a single date and time (current or user selected).

- WLA Option 2: WLA for a date and time period (from start date and time to end date and time).

- WLA Option 3: WLA linked to an estimated route schedule. In this case the user also specifies:

  - A distance parameter, the limit of the check area as specified by IMO MSC.530(106)/Rev.1 11.3.7 when route planning and MSC.530(106)/Rev.1 11.4.4 when route monitoring.

  - A time resolution tu used to construct the individual WLA sections. This time resolution reflects the uncertainty or tolerance of the time schedule of the route, for example 10 minutes if the user assumes that they could follow the time schedule within 10 minutes.

# Implementation - general

WLA can be applied only in areas where there is data from both S-102 and S-104. For example, the intersection of the Red and Blue outline in figure D-2-6 below. WLA is applied to S-101 features after substitution of depths by S-102 as defined in clauses D-1.2 and clause D-1-3.

Figure D-2-6 - WLA can only be computed in areas where there is S-104 and S-102 data

Further, WLA can only be carried out:

- When WLA option 1 or WLA option 2 is selected, where the temporal extent of the S-104 fully covers the required date and time instant or period selected by the user

- When WLA option 3 is selected, where the temporal extent of the S-104 fully covers the estimated date and time of a part of the route; see figure D-2-16.

Where these conditions are not met the WLA processing must not be carried out.

When WLA has been processed the area for which it is defined must be surrounded by a boundary line using colour token DEPWL; see Figure D-2-7.

Figure D-2-7 - Boundary line of WLA coverage

NOTE: In this example S-102 coverage is larger than WLA coverage.

In addition to the display of the boundary line of the WLA coverage there should be a permanent indication about the application of WLA and the applied date and time, see details in WLA Options 1, 2 and 3.

In cases that the WLA area lines and ESC lines are co-located, only the WLA Area lines must be displayed

OEMs are free to design their user interface. Usable ideas include, for example, a mouse roller to change datetime, use of a slider to change datetime, or even to provide an animation from user selected start date and time to user selected end date and time.

# Implementation of WLA Option 1 – WLA for a single datetime instant

WLA is based on S-104 values closest to the selected datetime instant. Each S-102 point has an extent of coverage which is closest to it. Each S-104 point, similarly has an extent. The adjustment of the S-102 values is calculated by adjusting each S-102 point by the shoalest of the S-104 values, for all S-104 points whose extent intersects the extent of the S-102 point.

When an S-104 record does not exist for the precise time specified the shoalest of the two S-104 adjacent values is selected from the S-104 dataset. S-104 values can only be selected within the temporal extent of the S-104 dataset. In the example shown in Figure D-2-8 (where S-104 values, “V” defined every 30 minutes and a user selected datetime of 07:24) the two values selected are 07:00 and 07:30, WLA would select 1.3m (the 07:00 value).

Figure D-2-8 - Selection of time-varying value

In Figure D-2-9 below, the S-102 point X is adjusted by the shoalest (that is, minimum) value of the S-104 values from (a), (b), (c) and (d) at the required datetime instant because the S-102 point extent overlaps the S-104 extents of a,b,c and d.

Figure D-2-9 - Adjustment of S-102 values by S-104

Format of the permanent indication is as below:

WLA 12:34 08 Nov 2021

# Implementation of WLA Option 2 – WLA for a datetime range specified by the user as a time period

When WLA is based on a datetime range, then the process is identical to that followed for WLA Option 1 except each S-104 value selected is the shoalest of all values available in the S-104 within the selected time period.

All S-104 points contributing to the WLA must be defined across the time range required, otherwise the WLA is not computable and the user must be informed. The S-104 records selected are those which lie either within the user defined time period; immediately before the start point of the time period and immediatelyjust after the end point of the time period.

In the following examples:

Figure D-2-10 - Examples of S-104 data selection

- User selected period 1, four values are from 0700, 0730, 0800 and 0830 - WLA would select 08:30 value = 1.2m (even though the time period doesn't overlap).

- User selected period 2, two values are from 0630 and 0700 - WLA would select 06:30 value V=1.2m (even though the time period doesn't overlap).

- User selected period 3, three values are from 0600, 0630 and 0700 - WLA would select 06:00 V=1.1m (even though the time period doesn't overlap).

- User selected period 4, five values are from 0630, 0700, 0730, 0800 and 0830 – WLA would select 07:30 V=1.2m.

When adjusting depth values the shoalest (smallest) value from the selected S-104 records must be used in order to produce the safest WLA values.

Format of the permanent indication is as below:

WLA from 12:34 08 Nov 2021 to 14:56 08 Nov 2021

# Implementation of WLA Option 3 – linking of WLA to a defined route with planned waypoints and times

When WLA is based on a route then the limit of check area around the route is set by the user as specified by IMO MSC.530(106)/Rev.1 11.3.7.

The same user-specified distance must be used for the check of safety contour, prohibited areas, geographic areas for which special conditions exist and navigational hazards and equivalent requirements when route monitoring.

The WLA is processed within this check area. The boundary of the area of the display where WLA has been carried out must be displayed (as shown in figure D-2.16). In the process description this distance is referred to as ‘a’.

A route could be either a Planned route or a Monitored route. Both could be processed for the WLA, but not at the same time in a single display area. In case of multiple display areas, it is possible that one area is WLA processed for Planned route and another area is WLA processed for the Monitored route.

For a Planned route the datetime period applied for the WLA process is based on the schedule of the planned route itself.

For a Monitored route the datetime period applied for the WLA process is user selectable either:

- Based on the planned schedule of the monitored route itself; or

- Based on the monitored route adjusted for the current own ship position.

When WLA is based on the planned schedule and own ship is not keeping to schedule a Caution must be raised to indicate the water level being experienced may be different to that being applied by the ECDIS.

The region of WLA is restricted to a distinguishable bordered polygon around the route. The process works as follows:

- For each section of the route a series of estimated time markers is defined along the route at times t0 – tn. The diagram also shows the “limit of check area” as specified by IMO MSC.530(106)/Rev.1 11.3.7. Each time marker is delimited before and after by the time +/- half the user selected tu interval so the rectangle in the diagram represents the extent of time t1 which ranges from time t1-tu2 to t1+tu2. This is the WLA adjustment polygon (shown in red) corresponding to time t1:

Figure D-2-11 - WLA adjustment polygon

- Where the route follows a curved section the polygon formed is not a rectangle but an area defined by the user selected limit as shown by areas a and b:

Figure D-2-12 - WLA adjustment polygon along curved section

- The individual S-102 points (blue dots in Figure D-2-13 below) are assigned rectangular extents. For each WLA adjustment rectangle corresponding to each ti the S-102 extents which spatially intersect are selected. A full example is shown in Figure D-2-13 for one of the WLA polygons (shown in red).

Figure D-2-13 - Grid cell selection

- For each S-102 point whose extent intersects the WLA adjustment polygon the Water level is adjusted using a similar process to Option 1. The S-104 value used for adjustment is the shoalest value from each of the S-104 points whose extents intersect the S-102 point (as described in WLA Option 1) across the time period (ti-tu2) and (ti+tu2). In Figure D-2-14, illustrating this step, two S-102 points are adjusted by S-104 values in the time period (t1-tu2) and (t1+tu2), for calculation of t1. S-102 point x is adjusted using values drawn from S-104 points a and b (because its extent intersects the extents of points a and b) and S-102 point y is only adjusted with values from S-104 point b.

Figure D-2-14 - Selection of S-104 data

- Each of the S-102 points are assigned the adjusted water level equal to the S-102 value + the calculated (shoalest) S-104 value as defined in the previous step.

- Once the WLA polygon for time ti is processed the WLA polygon for time ti+1 is processed.

- S-102 points whose extents are intersected by the both the WLA polygon for ti and ti+1 are assigned the shoaler of the two values, that is the shoalest of the S-104 values between the two time periods (ti-tu2) and (ti+tu2), and (ti+1-tu2) and (ti+1+tu2).

This completes the WLA process for (red) polygon ti . In Figure D-2-15 below the orange border shows S-102 points lying in WLA polygons t1 and t2 and which would be assigned the shoaler of the two S-104 values corresponding to the t1 and t2 WLA process.

Figure D-2-15: Time-dependent adjustment of S-102 data with S-104 data

This process is extended to all ti along the planned route.

Figure D-2-16 shows a boundary line around all the WLA processed S-102 grid cells. This marks the boundary of the Water Level Adjustment area and requires portrayal to inform the user which areas of the display are subject to WLA. This area is surrounded by a distinguishable boundary line using colour token DEPWL).

Figure D-2-16 - Boundary of adjusted area

Figure D-2-17 below shows how the rectangle is positioned in the middle of S-104 coverage area and at the edge of the S-104 coverage are.

Figure D-2-17 - Positioning in interior and at edge

Format of the permanent indication is as below:

WLA from 12:34 08 Nov 2021 to 14:56 08 Nov 2021

# Treatment of depth and water level related S-101 features

## Substitution and adjustment of depth values

In areas covered by only S-102, all depth values must be substituted for all ENC features which have depth attribution (that is, the attribute valueOfSounding bound to them in the S-101 Feature Catalogue and the z value of soundings), when viewed at a MSVS that is larger than or equal to the minimumDisplayScale of the overlapping S-102 dataset

In areas where WLA is processed, all depth values must also be adjusted for all ENC features which have depth attribution (that is, the attribute valueOfSounding bound to them in the S-101 Feature Catalogue).

## Areas covered by S-102 only or by both S-102 and S-104

Adjustment of different geographic primitives.

- Point Features

  - For Areas covered by S-102 only, a value for the attribute valueOfSounding must be taken from the S-102 grid cell extents which intersect the point feature (as shown in Figure D-3-18).

  - For Areas covered by S-102 and S-104, the value for attribute valueOfSounding must be taken from the S-102 grid cell extents which intersect the point feature. WLA is then applied to this value using S-104 records selected from the S-104 grid cell intersecting the point feature. The S-104 value selected is as defined in clauses D-2.5, D-2.6 and D-2.7.

- For Curve and Surface features

  - For Areas covered by S-102 only, the value for valueOfSounding must be the shoalest value of all S-102 grid cells whose extents intersect the feature’s geometry within the S-102 coverage available. If the curve or surface feature is not completely within the S-102 then the value defined is the shoalest resulting from the original feature value in S-101 attribute and the value selected from the intersecting S-102 grid cells.

  - For Areas covered by S-102 and S-104, the value for valueOfSounding is first selected, as the shoalest value of all S-102 grid cells whose extents intersect the feature’s geometry. WLA adjustment must then applied to this value by selecting the shoalest value from all intersecting S-104 grid cells. The value selected is as defined in clauses D-2.5, D-2.6 and D-2.7 and shown in Figure D-3-19. If the curve or surface feature is not completely within the S-104 coverage then the adjusted value is the shoalest value resulting from the original feature value in S-101 attribute and the WLA adjusted value.

Figure D-3-18 - Substitution of sounding value from S-102

Figure D-3-19 - Substitution of valueOfSounding attribute in S-101 Obstruction from S-102 data

The S-101 features for which this depth substitution must be implemented within S-102 coverage areas are all those which bind valueOfSounding in the S-101 feature catalogue along with multipoint geometries, unless specifically excluded in this clause.

Soundings are included (either individual soundings or those which are part of an array) substituting the S-102 grid cell whose extents intersect the sounding position for the defined ZCOO (ISO8211), See figure D-3-18.

If the depth substituted S-101 feature is covered by S-104, then WLA must be processed using the same method as for the underlying depth area.

Depth values must also be substituted in all ENC features at all scales which have the attribute depthRangeMinimumValue bound to them in the S-101 Feature Catalogue.

For all features with substituted (and possibly adjusted values), the ECDIS Pick Report must indicate the substituted/adjusted value and its source. The format and portrayal of depths and drying heights (that is, number of decimals, etc) is unchanged.

For example, for S-102 only covered area:

Value Of Sounding 12.3 m [S-102]

When an S-101 attribute has been WLA adjusted the pick report must indicate the WLA adjusted value, source and time/date. The format of time date is: hh:mm dd mmm yyyy.

For example, for both S-102 and S-104 covered area:

Value Of Sounding 15.5m [WLA 12:34 08 Nov 2021]

Value Of Sounding 15.5m [WLA from 12:34 08 Nov 2021 to 14:56 08 Nov 2021]

## Adjustment of vertical clearance values in areas covered by both S-102 and S-104

Where S-101 features contain features with vertical clearance values, and the S-104 vertical datum matches the sounding datum defined for the S-101 feature, and a datum correction value is supplied in the S-101, then then the S-101 vertical clearance values must also be calculated and adjusted.

Adjustment of vertical clearance values must only take place in areas where S-102 and S-104 coverage overlaps and WLA is applied. Where the datum correction values are not included in S-101 or not available (as defined in this clause) the calculations must not be applied.

Any vertical clearance value adjustment must be adjusted using the intersecting S-104 grid cell(s) water level value and using the maximum of all intersecting S-104  data grid cells.  Adjustment must only be applied on any features which intersect with the coverage of the S-104 data.

Adjustment is defined for all features which bind verticalClearanceValue in the S-101 feature catalogue using either verticalClearanceFixed, verticalClearanceSafe, verticalClearanceOpen or verticalClearanceClosed unless specifically excluded in this clause.

The S-101 attribute adjusted is verticalClearanceValue. When adjusting vertical clearance values the largest9 value from all intersecting S-104 records must be used in order to produce the safest values.

The vertical clearance defined on each S-101 feature must be corrected using the S-104 value and the datumSeparationValue. The datumSeparationValue (if available) is included in the complex attribute of the S-101 feature containing the verticalClearanceValue, which may be verticalClearanceFixed, verticalClearanceSafe verticalClearanceOpen or verticalClearanceClosed.

For example the datum difference information datumSeparationValue on a verticalClearanceFixed complex attribute is contained in:

verticalClearanceFixed.datumDifferenceInformation

with the vertical clearance value to be adjusted defined in :

verticalClearanceFixed.verticalClearanceValue

Note that datum difference information is not mandatory and may not always be defined for all S-101 features which define vertical clearance.

When adjusting the vertical clearance the following formula must be used:

Adjusted verticalClearanceValue = verticalClearanceValueS-101 + datumSeparationValueS-101 – S-104 value9

See also the footnote to Table 10: Example Values - Water Level Adjustment regarding positive and negative values for S-104 data.

Additionally, if the user has the Uncertainty selector selected (clause 11.9.10), then the S-104 value must also be adjusted according to the S-104 vertical uncertainty value and the following formula must be used:

Adjusted verticalClearanceValue = verticalClearanceValue(S-101) + datumSeparationValue(S-101)) – S-104 value9 -  S-104 uncertainty

The user must be given a permanent indication that the Water Level (S-104) values have adjusted S-101 attribute values in the data display as described in D-1.5, D-1.6 and D-1.7.  Where vertical clearance values are adjusted as defined in this clause the permanent indication given to the user must indicate that some or all vertical clearance values have been adjusted. Any alerts that take into account vertical clearance must also include the adjusted figure.

When verticalClearanceValue in S-101 has been substituted or adjusted the ECDIS Pick Report must indicate the substituted value, source and time/date

## Alerts and indication details

Substituted or adjusted values in ENCs at the largest scale must be used as input to any alert/indication processing. The substituted values are portrayed as shown in Figure D-3-20 which shows the graphical highlight (red boundary and red transparent fill) for the intersected safety contour.

Figure D-3-20 - Graphical highlight for intersected safety contour

## Legend details

As stated in 11.10.1, the legend must indicate the vertical datum of S-102 and S-104. If the vertical datums are the same then a single indication is enough to cover S-101, S-102 and S-104.

# Appendix E – Dataset Loading and Display (Rendering) Algorithms

# Introduction

Details of the dataset loading and data display algorithms are contained within this Appendix.

Algorithms for dataset loading, unloading, and rendering (display) within a navigation system are prescribed for S-401 (and possibly other products which provide multiple generalizations for a given geographic area) in order for the appropriate data to be viewed at the boatmaster’s selected viewing scale (BSVS).

These algorithms are intended to provide clear and concise rules on how and when data is loaded and unloaded; and the order in which datasets are to be displayed.

Note 1: These algorithms only address loading and display related to visualization within the system graphics window.

Note 2: Light sectors and other features may span dataset boundaries and the algorithms contained in this Appendix may not be sufficient in all cases to ensure such features are portrayed correctly. Manufacturers are encouraged to test thoroughly (using published test data) and enhance implementations to avoid any clipping of features at dataset boundaries and to ensure that all relevant features are portrayed. It should be possible, for example, for the boatmaster to be capable of identifying the colour of the sectors affecting the ship, even if the lights involved are off the display.

Scales and scale ranges are defined in Clause 12.1.1. The scale range of a dataset is the range of display scales between which a producer intends the data for use. This scale range is determined by the scale ranges of the component Data Coverage features. Each Data Coverage feature has a minimum display scale and an optimum display scale attribute; the dataset scale range is from the smallest to the largest of these attribute values.

  - Dataset Loading Algorithm

### Prerequisites

A collection of Data Coverages, each containing:

- A polygon describing the Data Coverage: (dataCoverage);

- A set of scale bands: scaleBands(dataCoverage);

- The associated dataset: dataset(dataCoverage).

A projection projection that can convert geographic polygons geoPolygon to device polygons: projection(geoPolygon);

  - Scale Bands

Attributes which represent scales store the denominator of the scale; an attribute value of “22,000” represents a scale of “1:22,000”. Larger values represent smaller scales, while smaller values represent larger scales.

A scale range is the set of scales between different scale values. The attributes minimum display scale and optimum display scale describe the scale range of a Data Coverage feature. The scale range of a dataset is a set of ranges, each range taken from each of the component Data Coverage.

Scale bands describe specific scale ranges, as defined in table 11. Each scale band is defined by its minimum and optimum scale denominators and is defined using a numerical index.

Note that whenever scales are compared in these algorithms the numerical comparison and min/max functions are based on scales, not on scale denominators for example a scale of 45,000 is less than (“<”) a scale of 22,000 and min(22,000,45,000) = 45,000.

| scale band index | minimumScale | optimumScale | Remarks |
| --- | --- | --- | --- |
| 1 | NULL (∞) | 10,000,000 | For all values larger than 10,000,000 |
| 2 | 10,000,000 | 3,500,000 |  |
| 3 | 3,500,000 | 1,500,000 |  |
| 4 | 1,500,000 | 700,000 |  |
| 5 | 700,000 | 350,000 |  |
| 6 | 350,000 | 180,000 |  |
| 7 | 180,000 | 90,000 |  |
| 8 | 90,000 | 45,000 |  |
| 9 | 45,000 | 22,000 |  |
| 10 | 22,000 | 12,000 |  |
| 11 | 12,000 | 8,000 |  |
| 12 | 8,000 | 4,000 |  |
| 13 | 4,000 | 3,000 |  |
| 14 | 3,000 | 2,000 |  |
| 15 | 2,000 | 1,000 |  |
| 16 | 1,000 | 500 |  |
| 17 | 500 | 200 |  |

Table 11: Scale Band definitions

Algorithm GetScaleBand(scale)

Input: A scale

Output The index of the scale band

- If scale < 10,000,000

  - Return 1

- For index = 2 to 15

  - If scale>minimumScale[index] AND scale <= optimumScale[index]

    - Return index

- Return 15

The following algorithm associates a scale band index with a (display) scale:

Algorithm scaleBands(dataCoverage)

Input: A Data Coverage

Output: A set of associated scale band indices S

- minimumDisplayScale – The minimum display scale of the coverage (if not defined it is assumed that the scale is 1:∞ = 0)
optimumDisplayScale – The optimum display scale of the coverage

- Create an empty set S

- If minimumDisplayScale < 10,000,000

  - S = S ∪1

- For index = 2 to 15

  - If max(minimumDisplayScale, minimumScale[index]) < min(optimumDisplayScale,optimumScale[index])

    - S = S ∪index

- Return S

The following algorithm associates a set of scale band indexes with a Data Coverage feature:

  - Dataset Selection Process

The following algorithm selects Data Coverage features. The output is used to load each dataset which is associated with any of the selected features (S).

The algorithm evaluates a collection of Data Coverage features and selects those which overlap both the viewport and a supplied scale. If selected, the coverage footprint is subtracted from the viewport. This process is repeated until the viewport is empty or the entire inventory has been evaluated.

Algorithm SelectDataCoverages(collection, scale, viewport, projection) Input: A collection of Data Coverage features collection A scale for which the Data Coverage features will be selected (usually the display scale) A device-polygon viewport describing the device area that should be covered with data A projection projection Output: A set of Data Coverage features S - 𝑆 = ∅ - ScaleBand = 𝐺𝑒𝑡𝑆𝑐𝑎𝑙𝑒𝐵𝑎𝑛𝑑(𝑠𝑐𝑎𝑙𝑒) - While 𝑣𝑖𝑒𝑤𝑝𝑜𝑟𝑡 ≠ ∅ do - For all dataCoverage in collection - If ScaleBand ∈ 𝑠𝑐𝑎𝑙𝑒𝐵𝑎𝑛𝑑𝑠(dataCoverage) AND (𝑝𝑟𝑜jection(dataCoverage) ∩ 𝑣𝑖𝑒𝑤𝑝𝑜𝑟𝑡) ≠ Ø - 𝑆 = 𝑆 ∪ dataCoverage - 𝑣𝑖𝑒𝑤𝑝𝑜𝑟𝑡 = 𝑣𝑖𝑒𝑤𝑝𝑜𝑟𝑡 \ 𝑝𝑟𝑜jection(dataCoverage) - ScaleBand = ScaleBand – 1 - If ScaleBand = 0 - Return S - Return S

Comments:

| Row | Description |
| --- | --- |
| 1. | Create an empty set of inventory Data Coverage features |
| 2. | Get the scale band to which scale belongs and assign it to the variable ScaleBand |
| 3. | While the viewport area is not empty |
| 3.a | Loop over all Data Coverage features in the inventory |
| 3.a.i | If ScaleBand is an element of the scale bands of the Data Coverage and the projected Data Coverage polygon overlaps the viewport |
| 3.a.i.1. | Add the Data Coverage to S |
| 3.a.i.2. | Remove the projected Data Coverage polygon from the viewport, The viewport will now only define the uncovered part of the original viewport |
| 3.b. | Decrement ScaleBand |
| 3.c. | If ScaleBand equals to zero (no scale band left to investigate) |
| 3.c.i. | Return the collected result |
| 4. | Return the collected result |

  - Data display Algorithm

    - Rendering Algorithm

The first step is to use information from the loading algorithm to assign a mask to each of the Data Coverage features within each dataset.

Each mask represents the footprint of obscuring Data Coverages; it indicates areas of a dataset which should not be visible. Each dataset is then assigned a mask from the component Data Coverage features. The dataset mask is the union of the component Data Coverage masks.

Algorithm: AssignMasks(dataSets, viewport, projection)

Input: 	A set of datasets dataSets (previously selected using the dataset loading algorithm )

A device-polygon viewport describing the device area that should be covered with data

A projection projection

- Collect all Data Coverage features from dataSets into dataCoverages.

- Sort dataCoverages by minimum display scale into sortedCoverages (from smallest to largest scale)

- For each dataCoverage in sortedCoverages

  - Set mask of dataCoverage to Ø

  - For each obscuringCoverage in sortedCoverages

    - If obscuringCoverage <> dataCoverage AND scale(dataCoverage) < scale(obscuringCoverage) AND projection(polygon(dataCoverage)) ∩ projection(polygon(obscuringCoverage)) ≠ Ø

      - dataCoverage.mask = dataCoverage.mask ∪ projection(polygon(obscuringCoverage))

- For each dataset in dataSets

  - Set mask of dataset to Ø

  - For each dataCoverage in dataset

    - dataset.mask = dataset.mask ∪ dataCoverage.mask

Algorithm: RenderChartImage(dataSets, viewport)

Input: 	A set of datasets dataSets (previously selected using the dataset loading algorithm)

A device-polygon viewport describing the device area that should be covered with data

- Fill viewport as per Section 12.5.1 ENC No data areas.

- Collect all active* drawing instructions from all dataSets into drawingInstructions.

- Sort drawingInstructions by drawing priority into sortedInstructions (from smallest to largest)**

  - Instructions which share a drawing priority must be ordered as follows:

    - all null instructions, followed by

    - all area instructions, followed by

    - all line instructions, followed by

    - all point instructions, followed by

    - all text instructions

- For each drawingInstruction in sortedInstructions

  - RenderInstruction(drawingInstruction)

* The viewingGroup(s), scaleMinimum, scaleMaximum, date dependency, line suppression, and any other properties of the drawing instruction which may affect the instructions visibility must be taken into account. (See S-100 Part 9).

** To enhance the readability of text, an implementation may consider the guidance in S-100 Part 9 regarding text rendering to adjust this algorithm as needed.

The next step is rendering. The screen is filled with the no data pattern, then the active drawing instructions are collected from all loaded datasets and sorted by drawing priority. Each drawing instruction is then rendered using the RenderInstruction algorithm.

  - The RenderInstruction Algorithm

This algorithm describes how each drawing instruction is to be rendered. Instructions originating from non-point geometries use mask to clip or mask the rendered output; those originating from occluded point geometries are not rendered, and those originating from non-occluded point geometries are rendered without masking or clipping.

Algorithm: RenderInstruction(drawingInstruction, dataset, viewport)

Input: 	A drawingInstruction generated by dataset portrayal

The dataset of the drawing instruction

A device-polygon viewport describing the device area that should be covered with data

- isPoint = false

  - If drawingInstruction is an augmented point

    - isPoint = true

    - point = augmented geometry

  - Else If geometry of drawingInstruction feature reference is a point

    - isPoint = true

    - point = feature reference geometry

- If not isPoint

  - Render the drawing instruction, using the dataset mask to either clip or mask the rendered output. Portions of the rendered output which intersect mask should not be visible.

- Else If dataset.mask ∩ point <> Ø

  - Do not render the instruction

- Else

  - Render the instruction (without masking or clipping)

# Appendix F – GML Coverage Polygons

# GML Coverage Polygons

## Introduction

GML is used as an integral part of S-100 Part 17, specifically in the XML elements defined by the Exchange Catalogue Schema. For the purpose of representing coverage objects in CATALOG.XML, however only a very small number of possibilities are required. Implementers benefit from a tighter standardization through a reduction of the CATALOG.XML coverage polygons.

## Specification

In order to simplify the job of S-100 implementers for CATALOG.XML the only requirement is for a representation which encodes a simple, connected polygon with a single exterior and 0 or more holes.

Such polygons will be implemented using the GML vocabulary implemented in the included S-100 schemas as follows:

<S100XC:boundingPolygon>

<gex:polygon>

<gml:Polygon gml:id="DC1">

<gml:exterior>

<gml:LinearRing>

<gml:posList srsName="http://www.opengis.net/def/crs/EPSG/0/4326">0.0 0.0 1.0 0.0 1.1 1.1 0.0 1.0 0.0 0.0</gml:posList>

</gml:LinearRing>

</gml:exterior>

<gml:interior>

<gml:LinearRing>

<gml:posList srsName="http://www.opengis.net/def/crs/EPSG/0/4326">0.25 0.25 0.75 0.25 0.75 0.75 0.25 0.75 0.25 0.25</gml:posList>

</gml:LinearRing>

</gml:interior>

</gml:Polygon>

</gex:polygon>

</S100XC:boundingPolygon>

This implements bounding Polygons restricted to a single GML Polygon (with a gml:id and an SRS defined in each of the posList coordinates of http://www.opengis.net/def/crs/EPSG/0/4326) with a single exterior element and optional (i.e. 0 or more ) interior elements. Each exterior or optional interior is a Linear Ring with >=4 coordinate pairs expressed as a single space separated posList, (with no leading or trailing spaces), and with the first and last coordinate pair being identical, i.e. the Linear Ring is closed, e.g.

<gml:posList srsName="http://www.opengis.net/def/crs/EPSG/0/4326">44.4736122 -63.6849136 44.7395111 -63.6849136 44.7395111 -63.4249994 44.4736122 -63.4249994 44.4736122 -63.6849136</gml:posList>

Coordinate order is always as per http://www.opengis.net/def/crs/EPSG/0/4326, latitude followed by longitude.

No further stipulations are required as geometry must also conform to the S-100 geometry model and be conformant with the XML Schema for exchange catalogues.

# Appendix G - Thinning Algorithms

# Introduction

Thinning algorithms resize symbols and ensure they do not conflict on screen. Thinning is only required when zooming within the visible scale range (maximum – minimum display scale) of a dataset. The Inland ECDIS and Inland ECS implementer must use a thinning algorithm to reduce the possibility of screen clutter. The algorithm described meets these requirements but implementers may also use their own.

## Regularly gridded data

Let the grid cell’s diagonal for the unthinned grid at the current display scale be given by D mm. Note that D is dependent on the dataset and the characteristics of the viewing monitor. If every nth cell is displayed, the displayed spacing is nD. Next, suppose the maximum dimension of the largest scaled symbol in the displayed field is Lsmax mm. Then the ratio R of the maximum symbol dimension to the displayed grid spacing is constrained to be less than a prescribed maximum value, Rmax. A typical value for Rmax can be taken to be 0.5. (Given that on a navigation display there may be point features from other products within the extent of the grid, Rmax=0.5 may be too high for practical use; the optimal value of Rmax is left to manufacturer determination, and may be different for different products, depending on the shape of the symbol.)  Then the following inequality must be satisfied for the thinned grid:

R = LsmaxnD < Rmax

If the above inequality cannot be met with increment n equal to 1, then a new value for n is computed by the following formula:

n =1+ fixLsmaxDRmax

Where fix() is a function that returns the truncated integer value of its argument. For plotting, arrows at every nth column and every nth row are drawn, making sure that the row and column with the maximum-size symbol is drawn. The value of n must be calculated by the system. It also requires identifying a “seed point,” a grid point with the maximum-size symbol from which counting starts. Designating this seed point as (x0, y0), the grid points where symbols are drawn are given by:

xk=x0±n×kyk=y0±n×k                for k=0, 1, 2, 3 …

This algorithm assumes the grid spacing can be represented by its cell diagonal. It can be adapted to allow for symbols that are aligned parallel to grid axes or for rectangular cells.

The Figures below illustrate the use of this algorithm. Figure G-1-1 depicts a grid coverage feature symbolised by arrows of dimensions varying according to the value of an attribute at the grid point. Figure G-1.2 depicts the same data thinned with Rmax = 0.5 (outline-only arrows are suppressed). Figure G-1.3 depicts the thinning of the same data with Rmax = 0.33. The nominal dimensions and parameters for the three cases are:

Grid spacing at the display scale (D): 36mm (grid diagonal).

Scaling of symbols by data attribute values at grid points produced symbols of four sizes (arrow length):

- L0 = 30.4mm = Lsmax

- L1 = 17.7mm

- L2 = 16.2mm

- L3 = 12.7mm

Figure G-1-1 - Grid without thinning

For Rmax = 0.5, with n = 1, the value of R is greater than Rmax:

R= Lsmax1×D = 30.436 =0.844 >0.5

Applying the formula for computing n:

n=1+ fix30.436×0.5=2

Counting in row-major order from the grid origin (0,0), the first symbol of size Lsmax is located at (1,1). Denoting this point as the seed point (x0, y0), the only grid points where symbols are displayed are:

xk = x0 ± 2×kyk = y0 ± 2×k           for k=0, 1, 2, 3 …

The results are depicted in the following figure.

Figure G-1-2 - Grid thinned with Rmax = 0.5

For Rmax = 0.33, the formula for n yields:

n=1+ fix30.436×0.33=3

and

xk =x0 ±3×kyk=y0±3×k            for k=0, 1, 2, 3…

The results are shown in Figure 10.

Figure G-1-3 - Grid thinned with Rmax = 0.33

The algorithm may hide significant characteristics of the data; for example, counting displayable rows and columns starting with the first instance of a maximised symbol may suppress significant information in nearby grid points and produce the wrong overall impression. In Figure G-1.3, row 2 would be suppressed even if all the data points in that row are of the same size as the symbol at (1,1) — this would suppress more data points with scaled-up symbols, which may represent data of more significance to the Mariner.

Grid diagonal as a measure of cell spacing is more suitable for grids where cell dimensions along both axes are approximately equal in display units (that is, in millimetres at the display scale); if there are significant differences, the D and Rmax parameters will need to be different for the two axes.

Execution of this algorithm would be faster if the grid coordinates of the seed point (x0, y0) are known in advance, either encoded by the producer as instance metadata, or calculated when the dataset is ingested into the system. S-100 does not yet provide a standard way of encoding this information.

Manufacturers may extend or adapt this algorithm in various ways, for example:

- Adapt Rmax to the shape and proportions of the symbol (that is, its perceived effect on the display).

- For grids with cells whose dimensions in display units are very different along different axes, use different D and Rmax parameters for the two axes, giving different values of n for different axes.

- Pre-compute and cache the scale values where n changes, so that suppression or revelation of symbols can be determined by the scale of the display.

- Adapt the determination of the seed point to show as many significant values as possible.

In order to avoid confusing the mariner, reasonable consideration should be given to generally maintaining the regular appearance of the grid coverage, though some irregularity is probably unavoidable with greater thinning.

# Appendix H- Import and Use of projected data on S-100 Inland ECDIS and Inland ECS

# Introduction

The Inland ECDIS and Inland ECS implementer must use an algorithm for the conversion of UTM coordinates to geographic latitude/longitude. The algorithm described meets these requirements but implementers may also use their own.

Clause H-1.1 and H-1.2 provide a set of numbered, steps for the conversion calculations.

## Conversion from UTM to WGS 84

1

For the calculation of latitude φ and longitude λ (both in degrees of arc) from easting E and northing N (both in meters) from a Universal Transverse Mercator (UTM) coordinate reference system (CRS) where the UTM CRS has a geodetic CRS of WGS 84, it is given for all UTM zones:

False easting FE                  = 500000 m

Scale at natural origin kO=0.9996

Semi-major axis a               = 6378137.000 m

2

Inverse flattening  1f           = 298.257223563

Where b is the semi-minor axis, because 1f=aa-b

it follows that f  = 3.35281066475 ×10-3

that b=a 1-f

and that b=6356752.31425

1

- For all UTM North zones, false northing FN           = 0 m

For all UTM South zones, false northing FN          	= 10000000 m

1

Each UTM zone has a corresponding central meridian λ0, which can be found in the EPSG dataset. North and south zones of the same number (e.g., UTM Zone 18N and UTM Zone 18S) share a central meridian.

Algorithmically, the value can be found as follows:

For zone numbers Nz between 1 and 30 (inclusive): 	λ0=183-(6×Nz)° W

For zone numbers Nz between 31 and 60 (inclusive):    	λ0={6×Nz-30-3}° E

Example: For UTM Zone 45S, Nz=45, so λ0=87° E. λ0 is also 87° E for UTM Zone 45N.

3

2

We next calculate constants for the projection:

e= 2f-f2=8.18191908 × 10-2

n= f(2-f)=1.67922039 × 10-3

B=a(1+n)1+n24+n464=6367449.145823415

h1'=n2-23n2+3796n3+1360n4=8.37732164 ×10-4

h2'=148n2-115n3+4371440n4=5.84321837 × 10-8

h3'=17480n3-37840n4=1.67348888 ×10-10

h4'=4397161280n4=2.16773776 × 10-13

4

- Then

η'=E-FEB kO= (E-500000)6364902.166165086

ξ'=(N-FN)BkO=(N-FN)6364902.166165086

4

ξ1'=h1'sin2ξ'cosh2η'   				η1'=h1'cos2ξ'sinh2η'

ξ2'=h2'sin4ξ'cosh4η'   				η2'=h2'cos4ξ'sinh4η'

ξ3'=h3'sin6ξ'cosh6η'   				η3'=h3'cos6ξ'sinh6η'

ξ4'=h4'sin8ξ'cosh8η'   				η4'=h4'cos8ξ'sinh8η'

ξ0'= ξ'- ξ1'+ξ2'+ξ3'+ξ4'   			η0'= η'-η1'+η2'+η3'+η4'

β'=sin-1sinξ0'coshη0'

Q'=sinh-1tanβ'

Q"=Q'+e tanh-1e tanhQ'

=Q'+e tanh-1e tanhQ"    which should be iterated until the change in Q" <10-8 ‡

4

Finally

φ=tan-1sinhQ"180π

λ=λO+sin-1tanhη0'cosβ'180π    ∎

## Conversion from UPS North to WGS 84

1

For the calculation of latitude φ and longitude λ (both in degrees of arc) from easting E and northing N (both in meters) from the Universal Polar Stereographic (UPS) North CRS with a geodetic CRS of WGS 84, it is given that:

False easting FE                  = 2000000 m

False northing FN               = 2000000 m

Scale at natural origin kO=0.994

Semi-major axis a               = 6378137.000 m

Inverse flattening  1f           = 298.257223563

2

It follows that f  = 3.35281066475 ×10-3

and that e= 2f-f2=8.18191908 × 10-2

5

1

We next calculate the value t':

t'=E-FE2+(N-FN)2(1+e)(1+e)(1-e)(1-e)2akO

=7.91307065 ×10-8(E-2000000)2+N-20000002

5

We then calculate the value χ. For UPS North:

χ= π2-2tan-1t'

4

1

We then find φ:

φ= 180πχ+e22+5e424+e612+13e8360sin2χ+7e448+29e6240+811e811520sin4χ+7e6120+81e81120sin6χ+4279e8161280sin8χ

= 180πχ+3.35655147×10-3sin2χ+6.57187270×10-6sin4χ+1.76456433×10-8sin6χ+5.32847842×10-11sin8χ

1

5

Finally

if E = 2000000, λ=0°

else, λ=180πatan2E-2000000,2000000-N  ∎

## Conversion from UPS South to WGS 84

1

For the calculation of latitude φ and longitude λ (both in degrees of arc) from easting E and northing N (both in meters) from the UPS South CRS with a geodetic CRS of WGS 84, it is given that:

False easting FE                  = 2000000 m

False northing FN               = 2000000 m

Scale at natural origin kO=0.994

Semi-major axis a               = 6378137.000 m

Inverse flattening  1f           = 298.257223563

2

2

it follows that f  = 3.35281066475 ×10-3

and that e= 2f-f2=8.18191908 × 10-2

1

5

We next calculate the value t':

t'=E-FE2+(N-FN)2(1+e)(1+e)(1-e)(1-e)2akO

=7.91307065 ×10-8(E-2000000)2+N-20000002

5

We then calculate the value χ. For UPS South:

χ= 2tan-1t'-π2

1

4

We then find φ:

φ= 180πχ+e22+5e424+e612+13e8360sin2χ+7e448+29e6240+811e811520sin4χ+7e6120+81e81120sin6χ+4279e8161280sin8χ

= 180πχ+3.35655147×10-3sin2χ+6.57187270×10-6sin4χ+1.76456433×10-8sin6χ+5.32847842×10-11sin8χ

1

5

Finally

if E = 2000000, λ=0°

else, λ=180πatan2E-2000000,N-2000000  ∎

## References [Informative]

The following references are provided for background, and to explain the origin of the formulas stated in this specification.

- “EPSG Geodetic Parameter Dataset” v. 11.023, 5 December 2024, https://epsg.org/

- IOGP Report 373-07-02 “Coordinate conversions and transformations including formulas” v. 68, August 2024, https://epsg.org/guidance-notes.html, p. 7

- IOGP Report 373-07-02, p. 60

- IOGP Report 373-07-02, p. 61

- IOGP Report 373-07-02, p. 84

Portrayal, Interrogation via Pick Reports and Inclusion in Update Status Reports.

Much of the described functionality in this clause is effected by implementation of the S-100 portrayal catalogues. When the S-100 portrayal mechanism is implemented the layers defined will implement the mandatory IMO layers. The description of those layers here is, thus, informative.

For colour token DEPWL, xyL values are: DAY x= 0.15, y=0.15, L=30.0; DUSK x=0.15, y=0.15, L=7.5; NIGHT x=0.15, y=0.15, L=1.2

In the case of a difference in resolution the maximum overlap is the size of the larger of the overlapping grid cell.

Although depths are always measured positive down in S-102 it is also possible for S-104 to be encoded either positive up or positive down. S-104 contains the attribute verticalCS which determines whether water level is encoded positive up (6498) or positive down (6499). In the example the water level height of 0.32 m would need to be multiplied by -1 if the S-104 verticalCS is 6499

In the case of a difference in resolution the maximum overlap is the size of the larger of the overlapping grid cell.

Thick dark blue dash line, colour token DEPWL xyL values are: DAY x= 0.15, y=¬0.15, L=30.0; DUSK x=0.15, y=0.15, L=7.5; NIGHT x=0.15, y=0.15, L=1.2.

thick dark blue dash dot line, colour token DEPWL, xyL values are: DAY x= 0.15, y=¬0.15, L=30.0; DUSK x=0.15, y=0.15, L=7.5; NIGHT x=0.15, y=0.15, L=1.2

See also the footnote to Table 10: Example Values - Water Level Adjustment regarding positive and negative values for S-104 data. This section is written with the assumption that S-104 values are positive depths, therefore the maximum of values should be used in order to produce the safest values. This should be modified if S-104 has negative (i.e. depth up) values to ensure the safest values are produced in all cases.

Data Coverage features selected after this point in the algorithm are subject to the OVERSC01 grossly overscaled pattern as described in clause 11.3.2

Values are for hypothetical data and display, and the figures have been reduced for reproduction in this document.