conveyAnnex B

S-57 ENC to S-401 Conversion Guidance

Edition 1.2.0 - February 2026 – Draft 2

History

| Date | Change description | Author |
| --- | --- | --- |
| 20-03-2025 | Table export based on Feature Catalogue Ed. 1.2.0 (20-03-2025) | Gert Morlion |
| 22-04-2025 | All differences are marked according to table 3-1 | Gert Morlion |
| 23-04-2025 | Added TOPMAR object to all beacon and buoy features. Added Light All Around feature rule to all buoys. | Gert Morlion |
| 17-10-2025 | Table export bases on Feature Catalogue Ed. 1.2.0 (17-10-2025, updated according to approved DCEG Ed. 1.2.0) | Gert Morlion |
| 26-01-2026 | Complete review of color coding and rules based on FC Ed. 1.2.0 | Claudia Heckert |
| 02-03-2026 | Adding the general rules to all features Checking and updating the general rules in accordance with decisions | Gert Morlion |

# Introduction

## General

The following clauses specify the conventions that are recommended for preparing and finalising S-57 IENC datasets for conversion to S-401 Edition 2.0.0 IENC compliant data.  This document is laid out, as far as possible, along the lines of the IEHG publication IENC Product Specification, Annex A – Data Classification and Encoding guide.

This document describes how to adapt S-57 IENC data so as to optimise the automation of S-57 IENC data conversion to S-401 data.  It is important to note that S-401 is not a “clone” or “duplication” of the IENC Feature Catalogue  and the  IENC Product Specification.  New functionality introduced in S-100 and improvements from the S-57 data model that have been implemented in S-401 as a result of lessons learned from S-57 IENC operational use mean that there is not a direct “one to one” equivalence between S-57 encoding and the corresponding S-401 encoding in many cases.  Also, automated conversion processes differ in their capabilities and operations and the model for co-production of both S-57 and S-401 data from a common database may vary between individual Data Producers.  This may result in an inability for full automated conversion of an operational S-57 IENC dataset to a fully operational and compliant S-401 dataset, thus requiring the Data Producer to apply further manual changes to the converted dataset.  Where manual intervention may be required by the Data Producer after an automated conversion process has been completed, this guidance is also included in this document.

It is important to note the following:

- The guidance included in this document is intended to optimise S-57 IENC data for initial conversion to S-401.

- Where possible, every effort must be made such that the performance of officially published S-57 IENCs in Inland ECDIS is not compromised.  For example, this document includes guidance on the population of the S-57 INFORM attribute to facilitate automated conversion.  Such attribute population may adversely affect the use of this data in Inland ECDIS (display of unwanted “information” indicators and additional information not required by the boatmaster for safe navigation).

- It is strongly recommended that, where possible, these changes are made at the database or product source dataset level only, and not included in the officially published S-57 IENC dataset for use in Inland ECDIS.

Because of the differences between the S-57 and S-401 data models, there are instances where an S-57 Object class, attribute or enumerate value will not be converted to S-401 during the automated conversion process due to an equivalent concept not being included in S-401.  These instances are identified individually throughout this document in the relevant S-57 Object class-specific clauses, along with any recommendations for pre- and post-conversion encoding.  Conversely, there have been enhancements made in the S-401 data model that have no equivalency in S-57 and therefore cannot be implemented as part of the automated S-57 to S-401 conversion process.  This document does not provide guidance as to how these enhancements may be manually implemented post-conversion, however references to these enhancements and the recommended encoding guidance included in S-401 Annex A – Data Classification and Encoding Guide, is included in Appendix A to this document.  Data Producers should also note that conversion tools may be customised so as to adapt to their specific data encoding policies and practices (for example variations in national spelling conventions and conventions for the encoding of specific text strings in the attribute INFORM).  Where such customisation has been implemented, Data Producers should take this into account when implementing the guidance included in this document.

## S-57 Acronyms

The conversion guidance will only mention uppercase acronyms and will not make a distinction between inland and martime specific elements. The rules also apply to the lowercase elements.

# General rules

## Cartographic framework

### Horizontal datum

The value of the horizontal datum encoded in the “Horizontal Geodetic Datum” [HDAT] subfield of the “Data Set Parameter” [DSPM] field for the S-57 dataset is populated in the “Datum Name” [DTNM] subfield of the “Geodetic Datum” [GDAT] field for the S-401 dataset.  As for S-57, the horizontal datum for S-401 ENCs must be WGS 84.

S-57 Meta object:		Horizontal datum (M_HOPA)		(A)

There is no equivalent meta feature in S-401 for the S-57 meta object M_HOPA.  It is considered that this information is not required for S-401.  Data Producers should consider removing instances of M_HOPA from their S-57 data for consistency.

### Vertical datum

The default vertical datum for the entire data set encoded in the “Vertical Datum” [VDAT] subfield of the “Data Set Parameter” [DSPM] field for the S-57 dataset will be populated in the “Datum Identifier” [DTID] subfield of the “Vertical Datum” [VDAT] field for the S-401 dataset.  This value will also be populated in the mandatory verticalDatum field for the Dataset Discovery Metadata of the S-401 dataset.

The vertical datum populated for VDAT and VERDAT on M_VDAT must be taken from the following table in order for the values to be directly converted to S-401:

| ID | Meaning |
| --- | --- |
| 1 | Mean low water springs |
| 2 | Mean lower low water springs |
| 3 | Mean sea level |
| 4 | Lowest low water |
| 5 | Mean low water |
| 6 | Lowest low water springs |
| 7 | Approximate mean low water springs |
| 8 | Indian spring low water |
| 9 | Low water springs |
| 10 | Approximate lowest astronomical tide |
| 11 | Nearly lowest low water |
| 12 | Mean lower low water |
| 13 | Low water |
| 14 | Approximate mean low water |
| 15 | Approximate mean lower low water |
| 19 | Approximate mean sea level |
| 22 | Equinoctial spring low water |
| 23 | Lowest astronomical tide |
| 24 | Local datum |
| 25 | International Great Lakes datum 1985 |
| 26 | Mean water level |
| 27 | Lower low water large tide |
| 30 | Highest Astronomical Tide |
| 31 | Local Low Water Reference Level |
| 32 | Local High Water Reference Level |
| 33 | Local Mean Water Reference Level |
| 34 | Equivalent Shipping height of Water |
|  | table 2.1 |

All other values in the S-57 VERDAT attribute are prohibited for vertical datum in S-401. Data Producers should consider replacing prohibited values with a permitted value before conversion to S-401. Note that other information (typically attribute HEIGHT or VERCLR, etc.) may need to be reviewed (if relevant) as a consequence of a modification of the vertical datum.

S-57 Meta object:		Vertical datum (M_VDAT)		(A)

S-401 Meta feature:	Vertical Datum						(S)					(S-401 DCEG Clause 3.10)

Conversion of these features can be automated only if the value populated for VERDAT is in accordance with table 2.1 above.  If a value other than those listed in table 2.1 is populated, Data Producers should consider replacing this value with a permitted value before conversion to S-401. Note that other related encoded information (such as values for the attributes HEIGHT, VERCLR, etc.) may need to be reviewed as a consequence of a modification of the vertical datum.

Value 42 (Approximate LAT) of the S-57 attribute VERDAT shall be converted into value 10 of the S-401 attribute Vertical Datum.

The following is a list of additional S-57 Object Classes requiring a value for VERDAT populated from the list in table 2.1 above in order for the vertical datum attribute for the corresponding S-401 feature(s) to be converted automatically:

BERTH 		BUISGL		CBLOHD	CONVYR		CRANES		EXCNST		FLODOC

GATCON			LNDMRK (CATLMK=19)						LIGHTS

PIPOHD		TUNNEL

VEHTRF		WTWGAG			WTWPRF

### Sounding datum

The default sounding datum for the entire data set encoded in the “Sounding Datum” [SDAT] subfield of the “Data Set Parameter” [DSPM] field for the S-57 dataset will be populated in the “Datum Identifier” [DTID] subfield of the “Vertical Datum” [VDAT] field for the S-401 dataset.  This value will also be populated in the mandatory soundingDatum field for the Dataset Discovery Metadata of the S-401 dataset.

The sounding datum populated for SDAT and VERDAT on M_SDAT must be taken from the following table:

| ID | Meaning |
| --- | --- |
| 1 | Mean low water springs |
| 2 | Mean lower low water springs |
| 3 | Mean sea level |
| 4 | Lowest low water |
| 5 | Mean low water |
| 6 | Lowest low water springs |
| 7 | Approximate mean low water springs |
| 8 | Indian spring low water |
| 9 | Low water springs |
| 10 | Approximate lowest astronomical tide |
| 11 | Nearly lowest low water |
| 12 | Mean lower low water |
| 13 | Low water |
| 14 | Approximate mean low water |
| 15 | Approximate mean lower low water |
| 19 | Approximate mean sea level |
| 22 | Equinoctial spring low water |
| 23 | Lowest astronomical tide |
| 24 | Local datum |
| 25 | International Great Lakes datum 1985 |
| 26 | Mean water level |
| 27 | Lower low water large tide |
| 30 | Highest Astronomical Tide |
| 31 | Local Low Water Reference Level |
| 32 | Local High Water Reference Level |
| 33 | Local Mean Water Reference Level |
| 34 | Equivalent Shipping height of Water |
| 35 | Highest Shipping Height of Water |
| 36 | Reference Low Water Level According to Danube Commission |
| 37 | Highest Shipping Height of Water According to Danube Commission |
| 38 | Dutch River Low Water Reference Level |
| 39 | Russian Project Water Level |
| 40 | Russian Normal Backwater Level |
| 41 | Ohio River Datum |
| 43 | Dutch High Water Reference Level |
| 44 | Baltic Sea chart datum 2000 |
| 45 | Dutch Estuary Low Water Reference Level |
|  | table 2.2 |

All other values in the S-57 VERDAT attribute are prohibited for sounding datum in S-401.  Producing Authorities should consider replacing prohibited values with a permitted value before conversion to S-401. Note that other information (such as sounding values and values for attribute VALSOU, etc.) may need to be changed (if relevant) as a consequence of a modification of the vertical datum.

Value 42 (Approximate LAT) of the S-57 attribute VERDAT shall be converted into value 10 of the S-401 attribute Vertical Datum.

S-57 Meta object:		Sounding datum (M_SDAT)		(A)

S-401 Meta feature:	Sounding Datum					(S)					(S-401 DCEG Clause 3.9)

Conversion of these features is automated only if the value populated for VERDAT is in accordance with table 2.2 above.  If a value other than those listed in table 2.2 is populated, Data Producers should consider replacing this value with a permitted value before conversion to S-401.  Note that other related encoded information (such as sounding values and values for the attribute VALSOU, etc.) may need to be reviewed as a consequence of a modification of the sounding datum.

### Units

The S-57 attribute hunits will be converted into the S-401 attribute DistanceUnitOfMeasurement as described in the table below:

| S-57 hunits value | S-401 DistanceUnitOfMeasurement |
| --- | --- |
| 1 (metres) | 1 (metres) |
| 2 (feet) | -- |
| 3 kilometres) | 3 (kilometres) |
| 4 (hectometres) | 7 (hectometres) |
| 5 (statute miles) | 4 (statute miles) |
| 6 (nautical miles) | 5 (nautical miles) |
| -- | 2 (yard) |
|  | table 2.3 |

### Accuracy of non-bathymetric data

#### Quality of positions

S-57 Meta Object:		Accuracy of data (M_ACCY)			(A)

S-401 Meta Feature:	Quality of Non-Bathymetric Data	(S)				(S-401 DCEG Clause 3.4)

All instances of encoding of the S-57 Meta Object M_ACCY and its binding attributes will be converted to an instance of the S-401 Meta Feature Quality of Non-Bathymetric Data during the automated conversion process.

#### Horizontal accuracy

Values populated for the S-57 attribute HORACC will be converted to the S-401 sub-attribute horizontal distance uncertainty. Note however that while HORACC is an allowable attribute for the following S-57 Object classes, horizontal distance uncertainty is prohibited for the corresponding S-401 features, and will therefore not be converted:

[CANALS]				Canal

[DOCARE]				Dock Area

[DRYDOC]				Dry Dock

[FLODOC]				Floating Dock

[GRIDRN]				Gridiron

[HULKES]				Hulk

[LITFLT]					Light Float

[LITVES]						Light Vessel

[LOKBSN]		Lock Basin

[LKBSPT]		Lock Basin Part

[SLCONS]		Shoreline Construction

[TUNNEL]		Tunnel

It is considered that horizontal distance uncertainty is not relevant for these features in S-401.

Where HORACC has been populated for an instance of the S-57 Object class BRIDGE, this will be converted to horizontal distance uncertainty on an instance of the S-401 Feature type Span Fixed or Span Opening, noting that horizontal distance uncertainty is prohibited for the S-401 Feature type Bridge (see clause 6.6.1).

#### Vertical accuracy

Values populated for the S-57 attribute VERACC will be converted to the S-401 complex attribute vertical uncertainty, sub-attribute uncertainty fixed where allowed. Note however that vertical uncertainty has been prohibited for most S-401 features for which VERACC is allowable for the corresponding S-57 Object class, as it is considered that vertical uncertainty is not relevant for these features in S-401. Where this is the case, it is stated against the individual Object classes within this document.

Where VERACC has been populated for an instance of the S-57 Object class BRIDGE, this will be converted to vertical uncertainty/uncertainty fixed on an instance of the S-401 Feature type Span Fixed or Span Opening, noting that vertical uncertainty is prohibited for the S-401 Feature type Bridge (see clause 6.8.1).

## Dates

### Reported date, dredged date, swept date

Except for reported dates, there is no equivalent S-401 attribute for the S-57 attribute SORDAT, as it is considered that this information is not required for S-401 ENCs.  In S-401, reported dates are encoded using the attribute reported date.  During the automated conversion process, where an S-57 feature converts to an S-401 feature having reported date as an allowable attribute, values populated in SORDAT will be converted to reported date.  Data Producers are advised to evaluate their data holdings to ensure that the value populated in SORDAT for these instances is actually the date that the instance was reported.

### Periodic date range, fixed date range, survey date range

The S-57 attributes DATEND, DATSTA, PEREND, PERSTA, SUREND and SURSTA are replaced in S-401 by the complex attributes fixed date range (DATEND, DATSTA), periodic date range (PEREND, PERSTA) and survey date range (SUREND, SURSTA). Unless otherwise stated against an individual Object class within this document, all encoded dates will be converted to the appropriate S-401 attribute automatically on conversion.

The encoding guidance for taking into account leap years (“last day in February”) for PEREND/PERSTA remains unchanged in S-401.

Data Producers should consider interrogating their S-57 data holdings and deleting any objects where the date indicated by the attribute DATEND means that the object is time expired (that is, the date in DATEND is earlier than the data of conversion).

S-401 Information feature:		Non-Standard Working Day	(N)				(S-401 DCEG Clause 24.3)

#### Seasonal Objects

Unless otherwise stated against an individual Object class within this document, all instances of encoding of attribute STATUS = 5 (periodic/intermittent) will be converted to the S-401 attribute status on conversion.  See also Appendix A, Table A.2.

### Times

Not applicable.

S-401 Information feature:	Service Hours		(N)							(S-401 DCEG Clause XX.X)

## Source information

Information contained in the S-57 attribute SORIND on individual Object instances in S-57 is encoded in S-401 using the complex attribute sourceIndication, sub-attribute source. All other sub-attributes are optional and may be encoded if known.

## Textual information

Information contained in the S-57 attributes INFORM, NINFOM, TXTDSC and NTXTDS on individual Object instances in S-57 is encoded in S-401 using the attributes information (complex attribute) and pictorial representation on the feature instance or by using the information type Nautical Information. Nautical Information is associated to the feature instance for which the information applies using the association Additional Information.

S-401 Information type:		Nautical Information		(N)	(S-401 DCEG Clause 24.4)

S-401 Information type:		Contact Details		(N)	(S-401 DCEG Clause 24.1)

S-401 Association:		Additional Information	(N)	(S-401 DCEG Clause 25.1)

Information contained in the S-57 attributes INFORM and NINFOM will generally be converted directly to an instance of the S-401 complex attribute information, sub-attribute text for the corresponding S-401 feature instance during the automated conversion process. However, the following exceptions and issues must be noted:

- In some cases, information encoded using INFORM/NINFOM in S-57 has been implemented in S-401 as an enhancement to the data model such as a new dedicated feature, attribute or enumerate value. Within this document, this is indicated against the relevant Object class along with any additional guidance to assist in the automated conversion process. This guidance may include instruction as to a standard text string to be populated in INFORM that can be recognised by the S-57 to S-401 converter so as to convert to a new S-401 feature/attribute/enumerate. This may be specific to a particular conversion technology and will require Data Producers to check their S-57 IENC portfolio prior to conversion and apply these changes as required, noting however that data conversion tools may include the capability to customise the conversion process in accordance with national encoding practices. In such cases an instance of information may not be created;

- In relation to the above, Data Producers must note that additional encoded instances of INFORM in an IENC dataset so as to aid in the conversion process may result in excessive screen clutter (display of “information” symbols) in certain S-57 Inland ECDIS display settings. Data Producers should evaluate the impact for the skippers of guidance within this document to populate INFORM additional to existing instances in their S-57 IENC portfolio and consider options to mitigate this impact. This may include population of INFORM (or database specific variant attributes) in the S-57 source database and filtering out these instances on creation of the S-57 product dataset; and

- Information encoded in NINFOM, when converted to S-401, requires an entry in the information complex attribute instance, sub-attribute language to indicate the language of the text string. There is no corresponding attribute in S-57 to provide this information. Data Producers may be required to manually populate this attribute during the conversion process, however a suitably configured converter may populate this attribute as part of the automated conversion process (see S-401 DCEG clause 2.4.6).

The attributes TXTDSC and NTXTDS will be converted directly to an instance of the S-401 complex attribute information, sub-attribute file reference for the corresponding S-401 feature instance during the automated conversion process. However, the following issues must be noted:

- The file naming convention for support files in S-401 is different from the convention used in S-57. Data Producers will be required to revisit automatically converted instances of the file reference sub-attribute during the conversion process and apply the new convention for both the file reference value and the name of the referenced file itself (see S-401 Main document clause XX.X); and

- Information encoded in NTXTDS, when converted to S-401, requires an entry in the information complex attribute instance, sub-attribute language to indicate the language of the text in the associated text file. There is no corresponding attribute in S-57 to provide this information. Data Producers may be required to manually populate this attribute during the conversion process, however a suitably configured converter may populate this attribute as part of the automated conversion process (see S-401 DCEG clause 2.4.6).

Where information contained in INFORM, NINFOM, TXTDSC and NTXTDS is duplicated for multiple Object instances in an S-57 dataset, this may be encoded more economically in the corresponding S-401 dataset by associating an instance of the S-401 Information type Nautical Information to the relevant S-401 Geo Features (see S-401 DCEG clause 24.4) using the association Additional Information (see S-401 DCEG clause 25.1). Where this is considered to be the preferred encoding, Data Producers will be required to manually encode the Nautical Information; associate this feature to the relevant S-401 Geo Features using the association Additional Information; and remove the complex attribute information from these Geo Features. Note that this encoding may also be considered where textual information is duplicated across multiple datasets within the S-57/S-401 ENC portfolio.

The sub-attribute language will be set to the 3-character code following ISO 639-2/T of the national language which is used.

All converted instances of the S-57 attributes NINFOM, NOBJNM and NTXTDS will require manual intervention in the S-401 converted dataset to provide an indication of the national language used. This is not indicated in the table below. See clauses X.X and X.

When information is contained in INFORM and TXTDSC it should be converted into different instances of the complex attribute information and should not be combined in one instance. The same applies to NINFOM and NTXTDS.

## Location codes

S-57 Meta Attribute:		location MRN (locmrn)

S-401 Meta Attribute:		interoperabilityIdentifier 								(S-401 DCEG Clause 27.161)

A resource name (or locationMRN) shall be defined for this feature and will be encoded in the S-401 attribute interoperabilityIdentifier.

In Europe the attribute interoperability identifier must be encoded:

- with an existing maritime MRN

- in accordance with letter a of 27.161 of the DCEG with a MRN type uuid, if no maritime MRN but a RIS-ID is available.

## Radar conspicuous

The S-57 attribute CONRAD value 1 or 3 shall be converted into the S-401 attribute radarConspicuous value true

The S-57 attribute CONRAD value 2 shall be converted into the S-401 attribute radarConspicuous value false

## Topmark

The S-57 structure/equipment relation between the beacon type or buoy type object class and TOPMAR shall be converted into the S-401 beacon type or buoy type feature, where the complex attribute topmark shall be used for the encoding of the S-57 TOPMAR attributes according to the general and expected rules.

## Lights

If the S-57 object class LIGHTS is coded as an equipment object to a beacon type or buoy type object class, it shall be converted into the S-401 feature Light All Around

# S-57 – S-401 Feature rules

| A feature, attribute or value which is new in S-401 | If it’s a mandatory element, it should be filled manually. |
| --- | --- |
| A feature, attribute or value which is deleted in S-401 | This feature, attribute or value will not be converted. |
| Convert S-57 object class attribute into S-401 attribute | Add a rule indicating how the conversion has to be done ifno general conversion rule is described in the introduction. The attribute numbers in the S-57 column are corresponding with the attributes in the S-401 column. |

Table 3-1

| S-57 to S-401 - Edition 1.2.0 - Conversion guidance | Page 10 |
| --- | --- |

3.1 Administration Area (ADMARE)
A defined area within which a jurisdiction applies. It may or may not be named. For IENCs a defined and named administrative area (e.g. country, state, district) or a defined and named administrative area of a river surveillance

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | curve surface |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [M]. 1.; NOBJNM (complex attribute "featureName - name "); [O]. 2.; CONDTN (attribute "inDispute") (3).; [C]. 3.; INFORM (complex attribute "information - text "); [O]. 3.; NINFOM (complex attribute "information text "); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference"); [O]. 5.; JRSDTN (1, 2, 3).; [M]. 6.; NATION; [M]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication" - source); [C]. | 1.; featureName; [1,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; inDispute; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; jurisdiction (1, 2, 3).; [1,1]. 6.; nationality; [1,*]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

- The automatic conversion shall change the S-57 attribute CONDTN with value ‘3’ (under reclamation) into the S-401 attribute inDispute as a Boolean with value ‘true’.

Manual conversion

- The S-401 Feature type Vessel Traffic Service Area has been introduced in S-401 as a dedicated feature to encode such areas. This information is encoded in S-57 using the Object class ADMARE. In order for this information to be converted across to S-401, the text string encoded in INFORM on the ADMARE should be in a standardised format, such as Vessel traffic service area. Data Producers are advised to examine any Vessel Traffic Service Area features created during the automated conversion process and confirm the attribution for these features as required, including any values populated for the complex attribute information. If INFORM is encoded in this way, ADMARE must be converted according to 3.170.

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS)

3.2 Airport/Airfield (AIRARE)
A defined area on land (including any buildings, installations and equipment) intended to be used either wholly or in part for the arrival, departure and surface movement of aircraft.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATAIR (1, 2, 4, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [C]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name "); [O]. 4.; INFORM (complex attribute "information - text"); [O]. 4.; NINFOM (complex attribute "information - text "); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PICREP; [O]. 7.; SORDAT ("reportedDate"); [C]. 8.; SCAMIN; [M]. 9.; SORIND (complex attribute "sourceIndication" - source); [C]. 10.; STATUS (5).; [O]. 11.; DATEND; [O]. 12.; DATSTA; [O]. 15.; PEREND; [O]. 16.; PERSTA; [O]. | 1.; categoryOfAirportAirfield (1, 2, 3, 4, 5, 6, 8, 9).; [0,*]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 2, 4, 5, 6, 7, 8, 12, 14).; [0,*]. |

Conversion rules

Automatic conversion

- If the S-57 attribute CATAIR has the value 4 and the primitive of the object class AIRARE is a point, the instance must be converted into the S-401 feature type Helipad.

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS)

3.3 Anchorage Area (achare)
An area in which vessels or seaplanes anchor or may anchor.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; catach (1, 2, 3, 4, 5, 6, 7, 9, 10, 11, 12, 13).; [O]. 3.; clsdng (1, 2, 3, 4, 5).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name "); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information" - text); [O]. 6.; NINFOM (complex attribute "information - text "); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 8.; NATSUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [O]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 12.; restrn (1, 2, 7, 8, 13, 14, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 38).; [O]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication" - source); [C]. 15.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [O]. 16.; unlocd; [C]. | 1.; categoryOfAnchorage (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,*]. 2.; categoryOfCargo (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15).; [0,*]. 3.; classOfDangerousCargo (1, 2, 3, 4, 5).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [0,*]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; reportedDate; [0,1]. 12.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18, 19, 20, 21, 23, 24, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 38, 39).; [0,*]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (1, 2, 3, 4, 5, 6, 7, 8, 9, 12, 14, 16, 17).; [0,*]. 16.; uNLocationCode; [0,1]. 17.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

- The value 10 of the S-57 attribute catach shall be converted into the S-401 value 16 of the attribute categoryOfAnchorage.

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.4 Anchor Berth (achbrt)
A designated area of water where a vessel, sea plane, etc., may anchor. For IENCs a designated area of water where a single vessel, convoy, sea plane, etc. may anchor.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; catach (1, 2, 3, 4, 5, 6, 7, 9, 10, 11, 12, 13).; [O]. 3.; clsdng (1, 2, 3, 4, 5).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name "); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information" - text); [O]. 6.; NINFOM (complex attribute "information - text "); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 8.; NATSUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [O]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; restrn (1, 2, 7, 8, 13, 14, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 38).; [O]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [O]. 17.; unlocd; [C]. | 1.; categoryOfAnchorage (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 16).; [0,*]. 2.; categoryOfCargo (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15).; [0,*]. 3.; classOfDangerousCargo (1, 2, 3, 4, 5).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [0,1]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; radius; [0,1]. 12.; reportedDate; [0,1]. 13.; restriction (1, 2, 7, 8, 13, 14, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 38).; [0,*]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 2, 3, 4, 5, 6, 7, 8, 9, 12, 14, 16, 17).; [0,*]. 17.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

- The value 10 of the S-57 attribute catach shall be converted into the S-401 value 16 of the attribute categoryOfAnchorage.

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.5 Berth (berths)
A place, generally named or numbered, where a vessel may moor or anchor. For IENCs: A designated named or numbered place at the bank of the river or in a harbour basin for the mooring of vessels

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; catbrt (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [O]. 3.; clsdng (1, 2, 3, 4, 5).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; INFORM (complex attribute "information" - text); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; locmrn; [C]. 11.; DRVAL1; [O]. 12.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 12.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 13.; PICREP; [O]. 14.; QUASOU (1, 2, 8, 10, 11).; [C]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 18.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [O]. 19.; trshgd (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [O]. 20.; unlocd; [C]. 21.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [C]. 22.; SOUACC (complex attribute "verticalUncertainty - uncertaintyFixed "); [C]. | 1.; categoryOfBerth (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [0,*]. 2.; categoryOfCargo (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15).; [0,*]. 3.; classOfDangerousCargo (1, 2, 3, 4, 5).; [0,1]. 4.; featureName; [1,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; horizontalClearanceLength; [0,1]. 7.; horizontalClearanceWidth; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,2]. 10.; maximumPermittedDraught; [0,1]. 11.; minimumBerthDepth; [0,1]. 12.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 13.; pictorialRepresentation; [0,1]. 14.; qualityOfVerticalMeasurement (1, 2, 8, 10, 11).; [0,*]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (1, 2, 3, 4, 5, 7, 8, 9, 12, 14, 16, 17).; [0,*]. 19.; transshippingGoods (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [0,*]. 20.; uNLocationCode; [0,1]. 21.; verticalDatum (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 19, 22, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45).; [0,1]. 22.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute DRVAL1 into the S-401 attribute minimumBerthDepth if the S-57 attribute DRVAL1 is used

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.6 Bollard (MORFAC)
Small shaped post, mounted on a wharf or dolphin used to secure ship's lines.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information" - text); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "sourceDate" or "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication" - source); [C]. 12.; STATUS (5).; [O]. 14.; WATLEV (1, 2, 3, 4, 5).; [O]. 15.; BOYSHP (2, 3, 7).; [C]. 16.; CATMOR (1, 2, 3, 4, 5, 7).; [M]. 17.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [O]. 18.; HORACC; [O]. 19.; NATCON (1, 2, 3, 4, 5, 6, 7, 8, 9).; [O]. 20.; VERACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 2, 3, 4, 5, 6, 7, 8, 12, 14).; [0,*]. 13.; uNLocationCode; [0,1]. 14.; waterLevelEffect (1, 2, 3, 4, 5, 6).; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 object class MORFAC with the attribute CATMOR value 3 (Bollard) shall be converted into the S-401 feature type Bollard with the corresponding attributes.

Manual conversion

- If a bollard object with the primitive line and area is encoded in S-57, it shall be replaced by the S-401 bollard (which in S-401 is always a point feature).

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.7 Bridge (bridge)
(1) An elevated structure extending across or over the weather deck of a vessel, or part of such a structure. The term is sometimes modified to indicate the intended use, such as navigating bridge or signal bridge. (2) A structure erected over a depression or an obstacle such as a body of water, railroad, etc., to provide a roadway for vehicles or pedestrians.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | curve surface noGeometry |
| Attributes | Attributes |
| 1.; CATBRG (1, 3, 4, 5, 7, 9, , 11, 12, 13).; [M]. 4.; CATTEV (4, 5, 6).; [O]. 7.; CONDTN (1, 2, 3, 5).; [C]. 8.; hunits (1, 2, 3, 4, 5, 6).; [C]. 9.; OBJNAM (complex attribute "featureName - name"); [O]. 9.; NOBJNM (complex attribute "featureName - name"); [O]. 10.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 10.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 12.; INFORM (complex attribute "information" - text); [C]. 12.; NINFOM (complex attribute "information - text"); [O]. 12.; TXTDSC (complex attribute "information - file reference"); [O]. 12.; NTXTDS (complex attribute "information - file reference "); [O]. 13.; locmrn; [C]. 16.; PICREP; [O]. 18.; SORDAT (attribute "reportedDate"); [C]. 19.; SCAMIN; [M]. 20.; SORIND (complex attribute "sourceIndication - source"); [C]. 21.; STATUS (5).; [O]. 22.; unlocd; [C]. 23.; VERACC (complex attribute "verticalUncertainty - uncertaintyFixed"); [O]. 25.; wtwdis; [C]. 26.; elevwl; [O]. 27.; HORACC; [O]. 28.; HORCLR; [O]. 29.; vcrlev; [O]. 30.; PEREND; [O]. 31.; PERSTA (; [O]. 32.; refgag; [C]. 33.; reflev (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [O]. 34.; VERCLR; [C]. 35.; VERCCL; [C]. 36.; VERCOP; [C]. 37.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [C]. 38.; vcrval; [O]. | 1.; bridgeConstruction (1, 2, 3, 4, 5).; [0,1]. 2.; bridgeFunction (1, 2, 3, 4).; [0,*]. 3.; categoryOfOpeningBridge (3, 4, 5, 7).; [0,1]. 4.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 5.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 6.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 7.; condition (1, 2, 3, 5).; [0,1]. 8.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 7).; [0,1]. 9.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 10.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 11.; height; [0,1]. 12.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 13.; interoperabilityIdentifier; [0,1]. 14.; natureOfConstruction (1, 2, 6, 7, 11).; [0,*]. 15.; openingBridge; [0,1]. 16.; pictorialRepresentation; [0,1]. 17.; radarConspicuous; [0,1]. 18.; reportedDate; [0,1]. 19.; scaleMinimum; [1,1]. 20.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 21.; status (1, 2, 4, 5, 7, 12).; [0,*]. 22.; uNLocationCode; [0,1]. 23.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 24.; visualProminence (1, 2, 3).; [0,1]. 25.; waterwayDistance; [0,1]. |

Conversion rules

Automatic conversion

- All instances of the S-57 object class BRIDGE of the geometric primitive ‘area’ over navigable water and its binding attributes shall be converted into a S-401 feature Bridge and Span Opening or Span Fixed as appropriate, and these features aggregated using the S-401 association Bridge Aggregation. - All instances of the S-57 object class BRIDGE of the geometric primitive ‘area’ over non-navigable water, shall be converted into the S-401 feature type Bridge and its corresponding binding attributes only without any associations or aggregations. - If the S-57 attribute CATBRG value is 1 shall the S-401 attribute OpeningBridge be converted into the value false. - If the S-57 attribute CATBRG value is 3, 4, 5 and 7 shall the S-401 attribute OpeningBridge be converted into the value true - The S-57 attribute CATBRG values will be converted into the S-401 attributes bridgeConstruction and categoryOfOpeningBridge as described in the table below: S-57 CATBRG; S-401 bridgeConstruction; S-401 categoryOfOpeningBridge; S-401 bridgeFunction. 1 (fixed Bridge). 3 (swing bridge); 3. 4 (lifting bridge); 4. 5 (bascule bridge); 5. 7 (draw bridge); 7. 9 (footbridge); 3. 11 (aqueduct); 4. 12 suspension bridge); 4. 13 (bridge arch); 1. - The S-57 attribute hunits will be converted into the S-401 attribute DistanceUnitOfMeasurement as described in the table below: S-57 hunits value; S-401 DistanceUnitOfMeasurement. 1 (metres); 1 (metres). 2 (feet); --. 3 kilometres); 3 (kilometres). 4 (hectometres); 7 (hectometres). 5 (statute miles); 4 (statute miles). 6 (nautical miles); 5 (nautical miles). --; 2 (yard). - The S-401 attribute bridgeFunction will be filled with the value 1 (vehicular) unless the function of the bridge is known. In that case the attribute will filled manually with the correct value.

Manual conversion

General rules:

### 2.1.4: Units (hunits) ### 2.1.5.3: Vertical accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.8 Building (BUISGL)
A free-standing self-supporting construction that is roofed, usually walled, and is intended for human occupancy (for example: a place of work or recreation) and/or habitation. For IENCs: Buildings with a special function, which may be of interest for the boatmaster.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 4.; CONDTN (1, 2, 3, 4, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; FUNCTN (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42).; [C]. 9.; INFORM (complex attribute "information" - text); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 14.; PICREP; [O]. 16.; SORDAT (attribute "reportedDate"); [C]. 17.; SCAMIN; [M]. 18.; SORIND (complex attribute "sourceIndication" - source); [C]. 21.; CONVIS (attribute "visualProminence") (1, 2).; [O]. | 1.; buildingShape (5, 6, 7, 8, 9).; [0,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; condition (1, 2, 3, 4, 5).; [0,1]. 5.; elevation; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; function (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 44, 45, 46, 47, 48).; [0,*]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; inTheWater; [0,1]. 12.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 13.; natureOfConstruction (1, 2, 6, 7, 8, 12).; [0,*]. 14.; pictorialRepresentation; [0,1]. 15.; radarConspicuous; [0,1]. 16.; reportedDate; [0,1]. 17.; scaleMinimum; [1,1]. 18.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 19.; status (4, 7, 8, 12, 13, 14).; [0,*]. 20.; verticalLength; [0,1]. 21.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.9 Built-Up Area (BUAARE)
An area of land or construction over the water containing a concentration of buildings and/or other structures.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATBUA (1, 2, 3, 4, 5).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; INFORM (complex attribute "information" - text); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; locmrn; [C]. 8.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; unlocd; [C]. | 1.; categoryOfBuiltUpArea (1, 2, 3, 4, 5, 6).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; height; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; inTheWater; [0,1]. 8.; pictorialRepresentation; [0,1]. 9.; radarConspicuous; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; uNLocationCode; [0,1]. 14.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.10 Bunker Station (bunsta)
A station, at which a ship is able to bunker fuel, water or ballast or to obtain electrical power supply.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; bunves (1, 2).; [M]. 2.; catbun (1, 2, 3, 4, 5, 6, 7, 8).; [O]. 3.; CATTEV (4, 5, 6).; [O]. 4.; CONDTN (1, 2, 3, 5).; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information" - text); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; locmrn; [C]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 11.; allcon (complex attribute "powerCharacteristics - allowedConsumption"); [O]. 11.; amoamp (complex attribute "powerCharacteristics - amoundOfAmperage"); [C]. 11.; catfrq (complex attribute "powerCharacteristics - categoryOfFrequency") (1, 2).; [C]. 11.; catplg (complex attribute "powerCharacteristics - categoryOfPlug"); [O]. 11.; catvol (complex attribute "powerCharacteristics - categoryOfVoltage") (1, 2).; [C]. 11.; shrnum (complex attribute "powerCharacteristics - numberOfShoreConnectors"); [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; STATUS (5).; [O]. 16.; unlocd; [C]. 17.; HORACC; [O]. 18.; VERACC; [O]. | 1.; bunkerVesselAvailability (1, 2).; [1,1]. 2.; categoryOfBunkerStation (1, 2, 3, 4, 5, 6, 7, 8).; [0,*]. 3.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 4.; condition (1, 2, 5).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; powerCharacteristics; [0,*]. allowedConsumption; [0,1]. amountOfAmperage; [0,1]. categoryOfFrequency (1, 2).; [0,1]. categoryOfPlug; [0,1]. categoryOfVoltage (1, 2).; [0,1]. numberOfShoreConnectors; [0,1]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (5, 18).; [0,*]. 16.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute allcon shall be converted into the S-401 complex attribute powerCharacteristics, sub-attribute allowedConsumption. - The S-57 attribute amoamp shall be converted into the S-401 complex attribute powerCharacteristics, sub-attribute amoundOfAmperage - The S-57 attribute catfrq shall be converted into the S-401 complex attribute powerCharacteristics, sub-attribute categoryOfFrequency - The S-57 attribute catplg shall be converted into the S-401 complex attribute powerCharacteristics, sub-attribute categoryOfPlug - The S-57 attribute catvol shall be converted into the S-401 complex attribute powerCharacteristics, sub-attribute categoryOfVoltage - The S-57 attribute shrnum shall be converted into the S-401 complex attribute powerCharacteristics, sub-attribute categoryOfShoreConnectors

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.11 Buoy Line ()
A buoy line is a set of buoys positioned on a curve indicating a certain restriction.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | curve |
| Attributes | Attributes |
|  | 1.; directionOfImpact (1, 2, 3, 4, 5).; [0,*]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; restriction (7).; [0,*]. 5.; scaleMinimum; [1,1]. 6.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.12 Cable Area (CBLARE)
An area which contains one or more submarine cables.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATCBL (1, 3, 4, 5, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information" - text); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; RESTRN (1, 38).; [O]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication" - source); [C]. 12.; STATUS (5, 18).; [C]. | 1.; categoryOfCable (1, 3, 4, 5, 6, 7, 8, 10).; [0,*]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 12, 13, 14, 16, 17, 18, 20, 23, 24, 25, 38, 39).; [0,*]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 5, 7, 13, 18).; [0,*]. 13.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.13 Cable Overhead (cblohd)
A single continuous rope-like bundle consisting of multiple strands of fiber, plastic, metal, and/or glass, which is supported by structures such as poles or pylons and passing over or nearby navigable waters.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; catcbl (1, 3, 4, 5, 6, 7).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 3.; CONDTN (1, 2, 3, 5).; [O]. 4.; hunits (1, 2, 3, 4, 5, 6).; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; INFORM (complex attribute "information" -text); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; locmrn; [C]. 11.; vcrlev; [O]. 12.; PICREP; [O]. 14.; refgag; [C]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication" - source); [C]. 18.; STATUS (5).; [O]. 19.; unlocd; [C]. 20/21.; VERACC (complex attribute "verticalClearanceFixed – verticalUncertainty – uncertaintyFixed"); [O]. 20/21.; VERCLR (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [M]. 22.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [O]. 23.; vcrval; [O]. 24.; wtwdis; [O]. 25.; HORACC; [O]. | 1.; categoryOfCable (1, 3, 6, 7, 10).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 7).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; iceFactor; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 11.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 12.; pictorialRepresentation; [0,1]. 13.; radarConspicuous; [0,1]. 14.; referenceGauge; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (1, 4, 5, 7, 12, 28).; [0,*]. 19.; uNLocationCode; [0,1]. 20.; verticalClearanceFixed; [1,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 21.; verticalClearanceSafe; [0,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 22.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 23.; verticalRiverDatumReferenceLevelValue; [0,1]. 24.; visualProminence (1, 2, 3).; [0,1]. 25.; waterwayDistance; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute CATCBL value 4 (telephone) and value 5 (telegraph) shall be converted into the S-401 corresponding attribute categoryOfCable value 10.

Manual conversion

- If the S-57 attribute VERCLR is indicating the real height under the bridge (without safety margin), the S-401 complex attribute verticalClearanceFixed shall be used. In this case the VERCLR is converted into the sub-attribute verticalClearanceValue and the S-57 attribute VERACC is converted into the complex sub-attribute verticalUncertainty. - If the S-57 attribute VERCLR is indicating the height under the bridge with safety margin, the S-401 complex attribute verticalClearanceSafe shall be used. In this case the VERCLR is converted into the sub-attribute verticalClearanceValue and the S-57 attribute VERACC is converted into the complex sub-attribute verticalUncertainty.

General rules:

### 2.1.4: Units (hunits) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.14 Cable Submarine (CBLSUB)
An assembly of wires or fibres, or a wire rope or chain, which has been laid underwater or buried beneath the seafloor.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 2.; CATCBL (1, 3, 4, 5, 6).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; STATUS (4, 5, 18).; [C]. 12.; SORIND (complex attribute "sourceIndication" - source); [C]. | 1.; buriedDepth; [0,1]. 2.; categoryOfCable (1, 3, 4, 5, 6, 7, 8, 9, 10).; [0,1]. 3.; condition (1, 5).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; status (1, 4, 5, 13, 18).; [0,*]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.15 Canal (CANALS)
An artificial waterway with no flow, or a controlled flow, used for navigation, or for draining or irrigating land (ditch). This feature is only used for non-navigable canals in Inland ENCs.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information" - text); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication" - source); [C]. | 1.; categoryOfCanal (1, 2, 3).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; horizontalClearanceFixed; [0,1]. horizontalClearanceValue; [1,1]. horizontalDistanceUncertainty; [0,1]. 6.; horizontalWidth; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 3, 4, 5, 6, 8, 14).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.16 Cardinal Beacon (BCNCAR)
A cardinal beacon is used in conjunction with the compass to indicate where the mariner may find the best navigable water. It is placed in one of the four quadrants (North, East, South and West), bounded by inter-cardinal bearings from the point marked.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BCNSHP (1, 2, 3, 4, 5, 6, 7).; [M]. 2.; CATCAM (1, 2, 3, 4).; [M]. 3.; COLOUR (2, 6).; [M]. 4.; COLPAT (1).; [M]. 5.; CONDTN (1, 2, 3, 5).; [O]. 7.; OBJNAM (complex attribute "featureName - name"); [O]. 7.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 8.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; HEIGHT; [O]. 10.; INFORM (complex attribute "information" - text); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 10.; NTXTDS (complex attribute "information - file reference "); [O]. 12.; MARSYS (1, 2).; [C]. 14.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 14.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 15.; PICREP; [O]. 16.; CONRAD (attribute "radarConspicuous") (3).; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication" - source); [C]. 20.; STATUS (5).; [O]. 22.; VERLEN; [O]. 21. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation - text"); [O]. NINFOM (complex attribute “topmark - shapeInformation - text”); [O]. SCAMIN; [M]. SORDAT; [C]. SORIND; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; beaconShape (1, 2, 3, 4, 5, 6, 7).; [1,1]. 2.; categoryOfCardinalMark (1, 2, 3, 4).; [1,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [1,1]. 5.; condition (1, 2, 5).; [0,1]. 6.; elevation; [0,1]. 7.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 8.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 9.; height; [0,1]. 10.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 11.; interoperabilityIdentifier; [0,1]. 12.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 13.; natureOfConstruction (1, 2, 6, 7, 8).; [0,*]. 14.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 15.; pictorialRepresentation; [0,1]. 16.; radarConspicuous; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; status (1, 2, 4, 5, 7, 8, 12, 18).; [0,*]. 21.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 22.; verticalLength; [0,1]. 23.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR)

3.17 Cardinal Buoy (BOYCAR)
A cardinal buoy is used in conjunction with the compass to indicate where the mariner may find the best navigable water. It is placed in one of the four quadrants (North, East, South and West), bounded by inter-cardinal bearings from the point marked. For IENCs a cardinal buoy is used to mark the position of danger points, obstacles and special features on lakes and broad waterways.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BOYSHP (1, 2, 3, 4, 5, 6, 8).; [M]. 2.; CATCAM (1, 2, 3, 4).; [M]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 4.; COLPAT (1, 2, 3, 4, 5, 6).; [C]. 5.; OBJNAM (complex attribute "featureName - name"); [C]. 5.; NOBJNM (complex attribute "featureName - name"); [C]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information" - text); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; MARSYS (1, 2).; [C]. 10.; mmsico; [O]. 12.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 12.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 13.; PICREP; [O]. 14.; CONRAD (attribute "radarConspicuous") (3).; [O]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication" - source); [C]. 18.; STATUS (5).; [O]. 19. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation - text"); [O]. NINFOM (complex attribute “topmark – shapeInformation - text”); [O]. SCAMIN; [M]. SORDAT; [C]. SORIND; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; buoyShape (1, 2, 3, 4, 5, 6, 7, 8).; [1,1]. 2.; categoryOfCardinalMark (1, 2, 3, 4).; [1,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; marksNavigationalSystemOf (1, 2, 9, 11).; [0,1]. 10.; mMSICode; [0,1]. 11.; natureOfConstruction (6, 7, 8, 11).; [0,*]. 12.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 13.; pictorialRepresentation; [0,1]. 14.; radarConspicuous; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (1, 2, 5, 7, 8, 18).; [0,*]. 19.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 20.; verticalLength; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR) - 2.8: Lights (LIGHT)

3.18 Cargo Transhipment Area (CTSARE)
An area designated for transfer of cargo from one vessel to another sometimes in order to reduce a vessel's draught.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [O]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 2.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 3.; INFORM (complex attribute "information" - text); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 5.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 6.; PICREP; [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication" - source); [C]. 11.; STATUS (2, 4, 5).; [O]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; restriction (2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18, 19, 20, 21, 22, 24, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43).; [0,*]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 2, 3, 4, 5, 6, 7, 9).; [0,*]. 12.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.19 Causeway (CAUSWY)
A raised way across low or wet ground or water.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 1.; CONDTN (1, 2, 3, 5).; [O]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information" - text); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PICREP; [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 8.; SCAMIN; [M]. 9.; SORIND (complex attribute "sourceIndication" - source); [C]. 11.; WATLEV (2, 4).; [M]. | 1.; condition (1, 2, 3, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; natureOfConstruction (1, 2, 3, 4, 5, 6, 7).; [0,*]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 7, 8, 12, 14).; [0,*]. 11.; waterLevelEffect (1, 2, 3, 4, 5, 6).; [1,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.20 Caution Area (CTNARE)
Generally, an area where the mariner has to be made aware of circumstances influencing the safety of navigation.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information" - text); [M]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication" - source); [C]. 11.; STATUS (5).; [O]. 12.; CATTEV (4, 5, 6).; [O]. 13.; HORACC; [O]. 14.; VERACC; [O]. | 1.; condition (1, 3, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (5, 7).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.21 Checkpoint (chkpnt)
An official location at which to register, declare and/or inspect goods and/or people.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; catchp (1, 2).; [M]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information" - text); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; locmrn; [C]. 5.; NATION; [M]. 6.; PICREP; [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 8.; SCAMIN; [M]. 9.; SORIND (complex attribute "sourceIndication" - source); [C]. 10.; STATUS (5).; [O]. 11.; unlocd; [C]. 12.; DATEND; [O]. 13.; DATSTA; [O]. 14.; NINFOM; [O]. 15.; NOBJNM; [O]. 16.; PEREND; [O]. 17.; PERSTA; [O]. 18.; NTXTDS; [O]. | 1.; categoryOfCheckpoint (1, 2).; [1,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; nationality; [1,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 2, 5, 7, 9, 12).; [0,*]. 11.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.22 Coast Guard Station (CGUSTA)
A station at which a visual/radio/radar marine watch is kept either continuously or at certain times only.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point surface |
| Attributes | Attributes |
| 1.; COMCHA; [M]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information" - text); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication" - source); [C]. 12.; STATUS (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 12, 13, 14, 15, 16, 17, 18).; [C]. | 1.; communicationChannel; [1,*]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; isMRCC; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 2, 4, 5, 16, 17).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.23 Coastline (COALNE)
The line where shore and water meet. Shoreline and coastline are generally used synonymously. (IHO Dictionary - S-32). The official name 'Coastline' is a synonym for the term 'shoreline' commonly used in inland navigation.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATCOA (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11).; [O]. 2.; CATTEV (4, 5, 6).; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; INFORM (complex attribute "information" - text); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; PICREP; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 12.; SCAMIN; [M]. 13.; SORIND (complex attribute "sourceIndication" - source); [C]. 14.; VERACC (complex attribute "verticalUncertainty - uncertaintyFixed"); [O]. 16.; HORACC; [O]. | 1.; categoryOfCoastline (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11).; [0,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 6, 7, 8, 11, 13).; [0,*]. 4.; elevation; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17).; [0,*]. 9.; pictorialRepresentation; [0,1]. 10.; radarConspicuous; [0,1]. 11.; reportedDate; [0,1]. 12.; scaleMinimum; [1,1]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 15.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.1.5.3: Vertical accuracy (VERACC) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.24 Collision Regulations Limit ()
Convention on the International Regulations for Preventing Collisions at Sea, 1972 (COLREGs). The demarcation line between inland navigation rules and international navigation rules.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | curve |
| Attributes | Attributes |
|  | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 6.; regulationCitation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.25 Communication Area (comare)
Indicates the coverage of an area, in which a vessel has to report or may request information.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; catcom (1, 2, 3, 4, 5, 6, 7, 8, 9).; [O]. 2.; COMCHA; [M]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information" - text); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication" - source); [C]. 11.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [O]. | 1.; categoryOfCommunication (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,*]. 2.; communicationChannel; [1,*]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS)

3.26 Conveyor (CONVYR, convyr)
A mechanical device for conveying bulk material or people using an endless moving belt or series of rollers.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 1.; CATCON (1, 2).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; INFORM (complex attribute "information - text "); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 13.; vcrlev; [O]. 14.; PICREP; [O]. 15.; PRODCT (1, 2, 3, 4, 5, 6, 7, 8, 14, 15, 17, 21, 22).; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication"); [C]. 20.; STATUS (4, 12).; [O]. 21.; VERACC (complex attribute "verticalClearanceFixed – verticalUncertainty – uncertaintyFixed"); [O]. 21.; VERCLR (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [O]. 24.; vcrval; [O]. 26.; HORACC; [O]. 1.; CATCON (1, 2).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; INFORM (complex attribute "information - text "); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 13.; vcrlev; [O]. 14.; PICREP; [O]. 15.; PRODCT (1, 2, 3, 4, 5, 6, 7, 8, 14, 15, 17, 21, 22).; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication"); [C]. 21.; VERACC (complex attribute "verticalClearanceFixed – verticalUncertainty – uncertaintyFixed"); [O]. 21.; VERCLR (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [O]. 22.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [O]. 24.; vcrval; [O]. 26.; HORACC; [O]. | 1.; categoryOfConveyor (1, 2, 3, 4).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; liftingCapacity; [0,1]. 12.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 13.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 14.; pictorialRepresentation; [0,1]. 15.; product (4, 5, 6, 10, 11, 12, 13, 14, 15, 16, 17, 21, 22, 25).; [0,*]. 16.; radarConspicuous; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; status (4, 12).; [0,*]. 21.; verticalClearanceFixed; [0,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 22.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 23.; verticalLength; [0,1]. 24.; verticalRiverDatumReferenceLevelValue; [0,1]. 25.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute VERACC shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalUncertainty sub-attribute uncertaintyFixed. - The S-57 attribute VERCLR shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalClearanceValue.

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.27 Crane (CRANES, cranes)
A machine for lifting, shifting and lowering objects or materials by means of a swinging boom or with a lifting apparatus supported on an overhead track.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATCRN (2, 3, 4, 5).; [O]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 12.; vcrlev; [O]. 14.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. 22.; VERACC (complex attribute "verticalClearanceFixed – verticalUncertainty – uncertaintyFixed"); [O]. 22.; VERCLR (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [O]. 25.; vcrval; [O]. 27.; HORACC; [O]. 1.; CATCRN (2, 3, 4, 5).; [O]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; locmrn; [C]. 12.; vcrlev; [O]. 14.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. 21.; unlocd; [C]. 22.; VERACC (complex attribute "verticalClearanceFixed – verticalUncertainty – uncertaintyFixed"); [O]. 22.; VERCLR (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [O]. 23.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [O]. 25.; vcrval; [O]. 27.; HORACC; [O]. | 1.; categoryOfCrane (2, 3, 4, 5, 6).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; height; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; inTheWater; [0,1]. 11.; liftingCapacity; [0,1]. 12.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 13.; orientation; [0,1]. orientationUncertainty; [0,1]. orientationValue; [1,1]. 14.; pictorialRepresentation; [0,1]. 15.; radarConspicuous; [0,1]. 16.; radius; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; status (1, 4, 6, 12).; [0,*]. 21.; uNLocationCode; [0,1]. 22.; verticalClearanceFixed; [0,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 23.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 24.; verticalLength; [0,1]. 25.; verticalRiverDatumReferenceLevelValue; [0,1]. 26.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute VERACC shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalUncertainty sub-attribute uncertaintyFixed. - The S-57 attribute VERCLR shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalClearanceValue.

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.28 Current - Non-Gravitational (curent)
Any current that is caused by other than tide producing forces. For IENCs Current is preferably indicated at high and low water conditions to aid with planning, navigation and maneuvering.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; curvhw; [C]. 2.; curvlw; [C]. 3.; curvmw; [C]. 4.; curvow; [C]. 5.; dirimp (1, 2, 3, 4).; [C]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 10.; othnam; [C]. 11.; hignam; [C]. 12.; lownam; [C]. 13.; meanam; [C]. 14.; ORIENT (complex attribute "orientation -orientationValue "); [C]. 15.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 15.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 16.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. 22.; STATUS (5).; [O]. | 1.; currentVelocityAtHighWaterLevel; [0,1]. 2.; currentVelocityAtLowWaterLevel; [0,1]. 3.; currentVelocityAtMeanWaterLevel; [0,1]. 4.; currentVelocityAtOtherWaterLevel; [0,1]. 5.; directionOfImpact (1, 2, 3, 4).; [0,*]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; nameOfOtherLocallyRelevantWaterLevel; [0,1]. 11.; nameOfRelevantHighWaterLevel; [0,1]. 12.; nameOfRelevantLowWaterLevel; [0,1]. 13.; nameOfRelevantMeanWaterLevel; [0,1]. 14.; orientation; [0,1]. orientationUncertainty; [0,1]. orientationValue; [1,1]. 15.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 16.; pictorialRepresentation; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; speed; [0,1]. speedMaximum; [1,1]. speedMinimum; [0,1]. 21.; speedUnits (1, 2, 3, 4).; [1,1]. 22.; status (5).; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute ORIENT shall be converted into the S-401 complex attribute orientation sub-attribute orientationValue.

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.29 Custom Zone (CUSZNE)
The area within which national custom regulations are in force.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface |
| Attributes | Attributes |
|  | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; nationality; [1,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.30 Dam (DAMCON)
A barrier to check or confine anything in motion; particularly one constructed to hold back water and raise its level to form a reservoir, or to prevent flooding.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 1.; CATDAM (1, 2, 3).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; INFORM (complex attribute "information - text "); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; NATCON (1, 2, 3, 4, 5, 6, 7, 8, 9).; [O]. 12.; PICREP; [O]. 14.; SORDAT (attribute "reportedDate"); [C]. 15.; SCAMIN; [M]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 19.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 22.; HORACC; [O]. | 1.; categoryOfDam (1, 2, 3).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; natureOfConstruction (1, 2, 3, 6, 7).; [0,*]. 12.; pictorialRepresentation; [0,1]. 13.; radarConspicuous; [0,1]. 14.; reportedDate; [0,1]. 15.; scaleMinimum; [1,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (1, 2, 6, 7, 8, 14, 28).; [0,*]. 18.; verticalLength; [0,1]. 19.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 20.; visualProminence (1, 2, 3).; [0,1]. 21.; waterLevelEffect (1, 2, 3, 6).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.31 Data Coverage (M_COVR, M_CSCL)
A geographical area that describes the coverage and extent of spatial objects.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATCOV (1, 2).; [M]. 7.; SORDAT (attribute "reportedDate"); [C]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfCoverage (1, 2).; [1,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; maximumDisplayScale; [1,1]. 5.; minimumDisplayScale; [1,1]. 6.; optimumDisplayScale; [1,1]. 7.; reportedDate; [0,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.5: InteroperabilityIdentifier

3.32 Daymark (DAYMAR, daymar)
(1) The identifying characteristics of an aid to navigation which serve to facilitate its recognition against a daylight viewing background. On those structures that do not by themselves present an adequate viewing area to be seen at the required distance, the aid is made more visible by affixing a daymark to the structure. A daymark so affixed has a distinctive colour and shape depending on the purpose of the aid. (2) An unlighted navigational mark. For IENCs daymarks are used to code passing and crossing day beacons on the inland river system.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 2.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 3.; COLPAT (1, 2, 3, 4, 5, 6).; [C]. 4.; CONDTN (1, 2, 3, 5).; [O]. 7.; OBJNAM (complex attribute "featureName - name"); [O]. 7.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 8.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 10.; INFORM (complex attribute "information - text "); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 10.; NTXTDS (complex attribute "information - file reference "); [O]. 14.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 14.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 15.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 20.; SORIND (complex attribute "sourceIndication - source"); [C]. 21.; STATUS (5).; [O]. 22.; TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33).; [M]. 2.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 3.; COLPAT (1, 2, 3, 4, 5, 6).; [C]. 4.; CONDTN (1, 2, 3, 5).; [O]. 5.; dirimp (1, 2, 3, 4).; [C]. 7.; OBJNAM (complex attribute "featureName - name"); [O]. 7.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 8.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 10.; INFORM (complex attribute "information - text "); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 10.; NTXTDS (complex attribute "information - file reference "); [O]. 13.; ORIENT (complex attribute "orientation -orientationValue "); [C]. 14.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 14.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 15.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 20.; SORIND (complex attribute "sourceIndication - source"); [C]. 21.; STATUS (5).; [O]. 22.; TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33).; [M]. | 1.; categoryOfSpecialPurposeMark (1, 2, 3, 4, 5, 6, 7, 8, 10, 11, 12, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 60, 61, 62, 63).; [0,*]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; condition (1, 2, 3, 5).; [0,1]. 5.; directionOfImpact (1, 2, 3, 4, 5).; [0,1]. 6.; elevation; [0,1]. 7.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 8.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 9.; height; [0,1]. 10.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 11.; interoperabilityIdentifier; [0,1]. 12.; natureOfConstruction (1, 2, 4, 6, 7, 8, 11).; [0,*]. 13.; orientation; [0,1]. orientationUncertainty; [0,1]. orientationValue; [1,1]. 14.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 15.; pictorialRepresentation; [0,1]. 16.; radarConspicuous; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; shapeInformation; [0,*]. language; [0,1]. text; [1,1]. 20.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 21.; status (1, 4, 5, 7, 8, 12).; [0,*]. 22.; topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33).; [1,1]. 23.; verticalLength; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute ORIENT shall be converted into the S-401 complex attribute orientation sub-attribute orientationValue.

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.33 Depth Area (DEPARE, depare)
A water area whose depth is within a defined range of values. For IENCs detailed depth information (surface): - either referred to one reference water level only: water surface within the waterway whose detailed depth information is within a defined range of values that refer to only one vertical datum, the reference water level. or - referred to a water level model that is applied to Depth Areas: a water surface within the waterway in which detailed depth information is known within a defined range of values referenced to a vertical datum (the reference water level). The actual water level is provided by a water level model.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; DRVAL2; [M]. 3.; DRVAL1; [M]. 7.; INFORM (complex attribute "information - text "); [C]. 7.; NINFOM (complex attribute "information - text"); [O]. 9.; QUASOU (1, 2, 8, 10, 11).; [C]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 14.; HORACC; [O]. 1.; CATTEV (4, 5, 6).; [O]. 2.; DRVAL2; [M]. 3.; DRVAL1; [M]. 4.; hunits (1, 2, 3, 4, 5, 6).; [M]. 5.; eleva1; [C]. 6.; eleva2; [C]. 9.; QUASOU (1, 2, 8, 10, 11).; [C]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 13.; wtwdis; [M]. 14.; HORACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; depthRangeMaximumValue; [1,1]. 3.; depthRangeMinimumValue; [1,1]. 4.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 7).; [0,1]. 5.; elevation1OfSurface; [0,1]. 6.; elevation2OfSurface; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; qualityOfVerticalMeasurement (1, 2, 8, 10, 11).; [0,1]. 10.; reportedDate; [0,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 13.; waterwayDistance; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.1.4: Units (hunits) ### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.34 Depth Contour (DEPCNT)
A line connecting points of equal water depth which is sometimes significantly displaced outside of soundings, symbols, and other chart detail for clarity as well as generalization. Depth contours therefore often represent an approximate location of the line of equal depth as related to the surveyed line delineated on the source. For IENCs a line of constant depth denoting the depth between Shallow Depth and Fairway / Project Depth

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 4.; PICREP; [O]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SCAMIN; [M]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. 8.; VALDCO; [M]. 9.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 10.; HORACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 8.; valueOfDepthContour; [1,1]. 9.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.1.5.3: Vertical Accuracy (VERACC) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM) - 2.5: InteroperabilityIdentifier

3.35 Distance Mark (dismar)
A distance mark indicates the distance measured from an origin and consists of either a solid visible structure or a distinct location without special installation. Usually found on canals. On inland waterway it is used to serve as a reference along the waterway. Due to natural or historic changes in the waterway, the distance can deviate from real distance to the origin.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; CATDIS (1, 2, 3, 4).; [M]. 3.; hunits (1, 2, 3, 4, 5, 6).; [M]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information" - text); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (5).; [O]. 14.; unlocd; [C]. 15.; wtwdis; [M]. | 1.; categoryOfDistanceMark (1, 2, 3, 4).; [1,1]. 2.; distanceMarkVisible; [0,1]. 3.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 7).; [1,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; measuredDistanceValue; [0,1]. distanceUnitOfMeasurement (1, 2, 3, 4, 5, 6, 7).; [1,1]. referenceLocation; [0,1]. waterwayDistance; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (5).; [0,1]. 14.; uNLocationCode; [0,1]. 15.; waterwayDistance; [1,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.1.4: Units (hunits) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.36 Dock Area (DOCARE)
An artificially enclosed area within which ships may moor and which may have gates to regulate water level.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface |
| Attributes | Attributes |
|  | 1.; categoryOfDock (1, 2).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; horizontalClearanceFixed; [0,1]. horizontalClearanceValue; [1,1]. horizontalDistanceUncertainty; [0,1]. 6.; horizontalClearanceLength; [0,1]. 7.; horizontalClearanceWidth; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; maximumPermittedDraught; [0,1]. 11.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 12.; pictorialRepresentation; [0,1]. 13.; reportedDate; [0,1]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 4, 6, 8, 14).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

3.37 Dolphin (MORFAC (CATMOR = 1 and 2))
A post or group of posts, used for mooring or warping a vessel, or as an aid to navigation. The dolphin may be in the water, on a wharf or on the beach.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point surface |
| Attributes | Attributes |
| 1.; CATMOR (1, 2, 3, 4, 5, 7).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 7.; OBJNAM (complex attribute "featureName - name"); [O]. 7.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 8.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 10.; INFORM (complex attribute "information - text "); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 22.; NTXTDS (complex attribute "information - file reference "); [O]. 12.; NATCON (1, 2, 3, 4, 5, 6, 7, 8, 9).; [O]. 13.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 13.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 14.; PICREP; [O]. 16.; SORDAT (attribute "reportedDate"); [C]. 17.; SCAMIN; [M]. 18.; SORIND (complex attribute "sourceIndication - source"); [C]. 19.; STATUS (5).; [O]. 21.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 23.; BOYSHP (2, 3, 7).; [C]. 24.; HORACC; [O]. 25.; WATLEV (1, 2, 3, 4, 5).; [O]. | 1.; categoryOfDolphin (1, 2, 3, 4).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; elevation; [0,1]. 7.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 8.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 9.; height; [0,1]. 10.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 11.; interoperabilityIdentifier; [0,1]. 12.; natureOfConstruction (1, 2, 6, 7).; [0,*]. 13.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 14.; pictorialRepresentation; [0,1]. 15.; radarConspicuous; [0,1]. 16.; reportedDate; [0,1]. 17.; scaleMinimum; [1,1]. 18.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 19.; status (1, 2, 3, 4, 5, 6, 7, 8, 12, 14, 18).; [0,*]. 20.; verticalLength; [0,1]. 21.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 22.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 object class MORFAC with the attribute CATMOR value 1 (dolphin) an value 2 (deviation dolphin) shall be converted into the S-401 feature type Dolphin with the corresponding attributes. - The S-57 attribute CATMOR=2 (deviation dolphin) shall be converted into the S-401 attribute CategoryOfDolphin=2 (deviation dolphin). - The S-57 object class MORFAC with the attribute CATMOR value 3 (Bollards) shall be converted into the S-401 feature type Bollard with the corresponding attributes.

Manual conversion

General rules:

### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.38 Dredged Area (DRGARE)
An area of the bottom of a body of water which has been deepened by dredging.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 3.; DRVAL1; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; PICREP; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 13.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 17.; HORACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; depthRangeMaximumValue; [0,1]. 3.; depthRangeMinimumValue; [1,1]. 4.; dredgedDate; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; maximumPermittedDraught; [0,1]. 9.; pictorialRepresentation; [0,1]. 10.; qualityOfVerticalMeasurement (10, 11).; [0,1]. 11.; reportedDate; [0,1]. 12.; restriction (1, 2, 3, 4, 5, 6, 8, 11, 12, 13, 16, 17, 18, 19, 20, 21, 23, 25, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43).; [0,*]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; techniqueOfVerticalMeasurement (1, 2, 3, 6, 8, 9, 13, 15, 16, 17, 18).; [0,*]. 15.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 16.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

- If the reported date is indicating the date of dredging, the S-57 attribute SORDAT shall be converted into the S-401 attribute dredgeDate. - If the reported date is indicating the date of reporting, the S-57 attribute SORDAT shall be converted into the S-401 attribute reportedDate.

General rules:

### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.39 Dry Dock (DRYDOC)
An artificial basin fitted with a gate or caisson, into which vessels can be floated and the water pumped out to expose the vessel's bottom. Also called graving dock.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; DRVAL1; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; HORCLR; [O]. 9.; HORACC; [O]. 10.; HORLEN; [O]. 11.; HORWID; [O]. 12.; INFORM (complex attribute "information - text "); [O]. 12.; NINFOM (complex attribute "information - text"); [O]. 12.; TXTDSC (complex attribute "information - file reference"); [O]. 12.; NTXTDS (complex attribute "information - file reference "); [O]. 15.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. 21.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; depthRangeMinimumValue; [0,1]. 4.; elevation; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; horizontalClearanceLength; [0,1]. 8.; horizontalClearanceWidth; [0,1]. 9.; horizontalDistanceUncertainty; [0,1]. 10.; horizontalLength; [0,1]. 11.; horizontalWidth; [0,1]. 12.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 13.; interoperabilityIdentifier; [0,1]. 14.; maximumPermittedDraught; [0,1]. 15.; pictorialRepresentation; [0,1]. 16.; qualityOfVerticalMeasurement (2, 3, 4, 6, 7, 8, 9).; [0,*]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; status (1, 4, 6, 8, 12, 14).; [0,*]. 21.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.1.5.2: Horizontal Accuracy (HORACC) ### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.40 Dumping Ground (DMPGRD)
A sea area where dredged material or other potentially more harmful material, for example explosives, chemical waste, is deliberately deposited.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | point surface |
| Attributes | Attributes |
| 1.; CATDPG (2, 4, 5).; [M]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name "); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PICREP; [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 8.; RESTRN (1, 3, 5, 7, 8, 24).; [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfDumpingGround (2, 3, 4, 5, 6).; [1,*]. 2.; dateDisused; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27).; [0,*]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 2, 4, 6, 7).; [0,*]. 12.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.41 Dyke (DYKCON, Dike)
A dyke (or dike) is an artificial embankment to contain or hold back water.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; HEIGHT; [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; PICREP; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 12.; SCAMIN; [M]. 13.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 17.; HORACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; height; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; natureOfConstruction (1, 2, 3, 4, 5, 6, 7).; [0,*]. 9.; pictorialRepresentation; [0,1]. 10.; radarConspicuous; [0,1]. 11.; reportedDate; [0,1]. 12.; scaleMinimum; [1,1]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; verticalLength; [0,1]. 15.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 16.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.42 Emergency Wreck Marking Buoy ()
An emergency wreck marking buoy is a buoy moored on or above a new wreck, designed to provide a prominent (both visual and radio) and easily identifiable temporary first response.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point |
| Attributes | Attributes |
|  | 1.; buoyShape (1, 2, 3, 4, 5, 6, 7, 8).; [1,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 9.; mMSICode; [0,1]. 10.; natureOfConstruction (6, 7, 8, 11).; [0,*]. 11.; pictorialRepresentation; [0,1]. 12.; radarConspicuous; [0,1]. 13.; reportedDate; [0,1]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 17.; verticalLength; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.43 Exceptional Navigation Structure (excnst)
An exceptional navigational construction like aqueduct, lift-lock, etc.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; catexs (1, 2, 3, 4, 5).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 3.; CONDTN (1, 2, 3, 5).; [O]. 4.; DRVAL1; [M]. 5.; hunits (1, 2, 3, 4, 5, 6).; [M]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; horclw; [C]. 9.; HORACC; [O]. 10.; INFORM (complex attribute "information - text "); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 10.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; locmrn; [C]. 12.; sdrlev; [O]. 13.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 13.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 14.; PICREP; [O]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; sdrval; [O]. 18.; SORIND (complex attribute "sourceIndication - source"); [C]. 19.; STATUS (5).; [O]. 20.; unlocd; [C]. 21.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [C]. 22.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 23.; wtwdis; [M]. | 1.; categoryOfExceptionalStructure (1, 2, 3, 4, 5).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; depthRangeMinimumValue; [1,1]. 5.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 6, 7).; [1,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; horizontalClearanceWidth; [0,1]. 9.; horizontalDistanceUncertainty; [0,1]. 10.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 11.; interoperabilityIdentifier; [0,1]. 12.; nameOfSoundingDatumReferenceLevel; [0,1]. 13.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 14.; pictorialRepresentation; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; soundingDatumReferenceLevelValue; [0,1]. 18.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 19.; status (5).; [0,1]. 20.; uNLocationCode; [0,1]. 21.; verticalDatum (10, 12, 23, 24, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45).; [0,1]. 22.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 23.; waterwayDistance; [1,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.1.4: Units (hunits) ### 2.1.5.2: Horizontal Accuracy (HORACC) ### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.44 Fairway (FAIRWY)
That part of a river, harbour and so on, where the main navigable channel for vessels of larger size lies. It is also the usual course followed by vessels entering or leaving harbours, called 'ship channel'. For IENCs the part of the navigable waterway area where a certain water depth within a certain width is available for the continuous navigation.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 12.; SCAMIN; [M]. 13.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; depthRangeMinimumValue; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; maximumPermittedDraught; [0,1]. 7.; orientationValue; [0,1]. 8.; pictorialRepresentation; [0,1]. 9.; qualityOfVerticalMeasurement (1, 2, 6).; [0,*]. 10.; reportedDate; [0,1]. 11.; restriction (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43).; [0,*]. 12.; scaleMinimum; [1,1]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; status (1, 3, 6, 7, 9, 28).; [0,*]. 15.; trafficFlow (1, 2, 3, 4).; [0,1]. 16.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 17.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.45 Fairway System (C_AGGR)
That part of a river, harbour and so on, where the main navigable channel for vessels of larger size lies. It is also the usual course followed by vessels entering or leaving harbours, called 'ship channel'. A fairway system is an aggregation of connected fairway features making up a complex fairway system.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface noGeometry |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [M]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information - text "); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; locmrn; [C]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [O]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; unlocd; [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; maximumPermittedDraught; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

- The use of a fairway system aggregation has not been described in edition 2.6. If the fairway systems should be aggregated, it should be done manually.

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.46 Fence/Wall (FNCLNE)
A man-made barrier used as an enclosure or boundary or for protection.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATFNC (1, 4).; [M]. 4.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; PICREP; [O]. 13.; SORDAT (attribute "reportedDate"); [C]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfFence (1, 3, 4).; [1,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; condition (1, 2, 3, 5).; [0,1]. 5.; elevation; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; height; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; natureOfConstruction (1, 2, 3, 6, 7, 11).; [0,*]. 11.; pictorialRepresentation; [0,1]. 12.; radarConspicuous; [0,1]. 13.; reportedDate; [0,1]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 7, 12, 13).; [0,*]. 17.; verticalLength; [0,1]. 18.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.47 Ferry Route (FERYRT, feryrt)
A route in a body of water where a ferry crosses from one shoreline to another.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve surface |
| Attributes | Attributes |
| 1.; CATFRY (1, 2).; [M]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; locmrn; [C]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [C]. 12.; unlocd; [C]. 1.; catfry (4).; [M]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; locmrn; [C]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [O]. 12.; unlocd; [C]. | 1.; categoryOfFerry (1, 2, 3, 4, 5).; [1,*]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 2, 3, 4, 5, 6, 7, 8, 9, 12, 14, 16, 17).; [0,*]. 12.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.48 Fishery Zone (FSHZNE)
The offshore zone in which exclusive fishing rights and management are held by the coastal nation.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface |
| Attributes | Attributes |
|  | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; nationality; [1,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; status (1, 5, 6, 7).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

3.49 Fishing Facility (FSHFAC)
A structure for fishing purposes which can be an obstruction to ships in general. The position of these structures may vary frequently over time.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; CATFIF (1, 2, 3, 4).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; STATUS (2, 4, 5).; [C]. 12.; VERLEN; [O]. | 1.; categoryOfFishingFacility (1, 2, 3, 4).; [0,1]. 2.; condition (1, 2, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 4, 5, 6, 7, 8, 12, 18, 28).; [0,*]. 12.; verticalLength; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.50 Floating Dock (FLODOC, flodoc)
A form of dry dock consisting of a floating structure of one or more sections which can be partly submerged by controlled flooding to receive a vessel, then raised by pumping out the water so that the vessel's bottom can be exposed.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | curve surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 4.; CONDTN (1, 2, 3, 5).; [O]. 5.; DRVAL1; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; HORCLR; [O]. 10.; HORACC; [O]. 11.; HORLEN; [O]. 12.; HORWID; [O]. 13.; INFORM (complex attribute "information - text "); [O]. 13.; NINFOM (complex attribute "information - text"); [O]. 13.; TXTDSC (complex attribute "information - file reference"); [O]. 13.; NTXTDS (complex attribute "information - file reference "); [O]. 17.; sdrlev; [O]. 18.; vcrlev; [O]. 20.; PICREP; [O]. 22.; SORDAT (attribute "reportedDate"); [C]. 24.; sdrval; [O]. 25.; SORIND (complex attribute "sourceIndication - source"); [C]. 26.; STATUS (1, 4, 6, 7, 8, 12).; [O]. 29.; vcrval; [O]. 30.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 1.; CATTEV (4, 5, 6).; [O]. 4.; CONDTN (1, 2, 3, 5).; [O]. 5.; DRVAL1; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; horcll; [C]. 9.; horclw; [C]. 9.; HORCLR; [O]. 10.; HORACC; [O]. 11.; HORLEN; [O]. 12.; HORWID; [O]. 13.; INFORM (complex attribute "information" - text); [O]. 13.; NINFOM (complex attribute "information - text"); [O]. 13.; TXTDSC (complex attribute "information - file reference"); [O]. 13.; NTXTDS (complex attribute "information - file reference "); [O]. 17.; sdrlev; [O]. 18.; vcrlev; [O]. 19.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 19.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 20.; PICREP; [O]. 22.; SORDAT (attribute "reportedDate"); [C]. 23.; SCAMIN; [M]. 24.; sdrval; [O]. 25.; SORIND (complex attribute "sourceIndication - source"); [C]. 26.; STATUS (1, 4, 6, 7, 8, 12).; [O]. 27.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [O]. 29.; vcrval; [O]. 30.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; condition (1, 2, 3, 5).; [0,1]. 5.; depthRangeMinimumValue; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; horizontalClearanceLength; [0,1]. 9.; horizontalClearanceWidth; [0,1]. 10.; horizontalDistanceUncertainty; [0,1]. 11.; horizontalLength; [0,1]. 12.; horizontalWidth; [0,1]. 13.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 14.; interoperabilityIdentifier; [0,1]. 15.; liftingCapacity; [0,1]. 16.; maximumPermittedDraught; [0,1]. 17.; nameOfSoundingDatumReferenceLevel; [0,1]. 18.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 19.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 20.; pictorialRepresentation; [0,1]. 21.; radarConspicuous; [0,1]. 22.; reportedDate; [0,1]. 23.; scaleMinimum; [1,1]. 24.; soundingDatumReferenceLevelValue; [0,1]. 25.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 26.; status (1, 4, 5, 6, 7, 8, 12).; [0,*]. 27.; verticalDatum (10, 12, 13, 23, 24, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45).; [0,1]. 28.; verticalLength; [0,1]. 29.; verticalRiverDatumReferenceLevelValue; [0,1]. 30.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 31.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

- If the S-57 object class FLODOC is encoded, it must be ensured that the area beneath the floating dock is filled with the Group 1 object UnsurveyedArea.

Manual conversion

- If the S-57 object class is FLODOC, the S-401 attribute scaleMinimum shall be filled with the value 22000.

General rules:

### 2.1.5.2: Horizontal Accuracy (HORACC) ### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.51 Fog Signal (FOGSIG)
A warning signal transmitted by a vessel, or aid to navigation, during periods of low visibility. Also, the device producing such a signal.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; CATFOG (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [M]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SIGFRQ; [O]. 11.; SIGGEN (1, 2).; [O]. 12.; SIGGRP; [C]. 13.; SIGPER; [C]. 14.; SIGSEQ; [C]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; STATUS (5).; [O]. 17.; VALMXR; [O]. | 1.; categoryOfFogSignal (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [1,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; signalFrequency; [0,1]. 11.; signalGeneration (1, 2, 3, 4, 5, 6).; [0,1]. 12.; signalGroup; [0,*]. 13.; signalPeriod; [0,1]. 14.; signalSequence; [0,*]. signalDuration; [1,1]. signalStatus (1, 2).; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 2, 4, 5, 7, 8, 15).; [0,*]. 17.; valueOfMaximumRange; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.52 Fortified Structure (FORSTC)
A structure that is specifically designed or reinforced to provide for defence from armed attack.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; CATFOR (1, 2, 3, 4, 5, 6).; [M]. 2.; CONDTN (2).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; NATCON (1, 2, 6, 7).; [O]. 9.; PICREP; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 12.; SCAMIN; [M]. 13.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; CONVIS (attribute "visualProminence") (1, 2).; [O]. | 1.; categoryOfFortifiedStructure (1, 2, 3, 4, 5, 6, 8, 9).; [1,1]. 2.; condition (1, 2).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; height; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; inTheWater; [0,1]. 8.; natureOfConstruction (1, 2, 3, 6, 7).; [0,*]. 9.; pictorialRepresentation; [0,1]. 10.; radarConspicuous; [0,1]. 11.; reportedDate; [0,1]. 12.; scaleMinimum; [1,1]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; status (4, 7, 8, 12, 13, 14, 28).; [0,*]. 15.; verticalLength; [0,1]. 16.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.53 Foul Ground (OBSTRN, CATOBS=7)
Areas over which it is safe to navigate but which should be avoided for anchoring, taking the ground or ground fishing.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; EXPSOU (1, 2, 3).; [C]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PICREP; [O]. 7.; QUASOU (2, 6, 7, 8).; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; VALSOU; [C]. 14.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 15.; WATLEV (1, 2, 3, 4, 5).; [C]. 16.; CATOBS (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11).; [O]. 17.; HORACC; [O]. 18.; NATSUR (9).; [C]. 19.; NATCON (1, 2, 3, 4, 6, 7, 8).; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; expositionOfSounding (1, 2, 3).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; pictorialRepresentation; [0,1]. 7.; qualityOfVerticalMeasurement (1, 2, 3, 4, 6, 7, 8, 9).; [0,*]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (13, 18, 28).; [0,*]. 12.; techniqueOfVerticalMeasurement (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18).; [0,*]. 13.; valueOfSounding; [0,1]. 14.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 15.; waterLevelEffect (3, 4, 5).; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 object class OBSTRN with the attribute CATOBS value 7 (foul ground) shall be converted into the S-401 feature type Foul Ground with the corresponding attributes.

Manual conversion

General rules:

### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.54 Free Port Area (FRPARE)
A port where certain import and export duties are waived (unless goods pass into the country) to facilitate reshipment to other countries.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CONDTN (1, 2, 3, 5).; [O]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information - text "); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; PICREP; [O]. 6.; SORDAT (attribute "reportedDate"); [C]. 7.; SCAMIN; [M]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; condition (1, 2, 3, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; status (1, 6, 8, 14).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.55 Gate (GATCON, gatcon)
A structure that may be swung, drawn, or lowered to block an entrance or passageway on a watercourse.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | point curve surface |
| Attributes | Attributes |
| 1.; CATGAT (2, 4).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 3.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; HORACC (complex attribute "horizontalClearanceOpen – horizontalDistanceUncertainty"); [O]. 7.; HORCLR (complex attribute "horizontalClearanceOpen – horizontalClearanceValue"); [M]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; locmrn; [C]. 11.; PICREP; [O]. 13.; SORDAT (attribute "reportedDate"); [C]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; unlocd; [C]. 18.; VERACC (complex attribute "verticalClearanceOpen – verticalUncertainty – uncertaintyFixed"); [O]. 18.; VERCLR (complex attribute "verticalClearanceOpen – verticalClearanceValue"); [O]. 19.; VERDAT (4, 23, 30).; [O]. 21.; vcrlev; [O]. 22.; vcrval; [O]. 1.; CATGAT (2, 4).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 3.; CONDTN (1, 2, 3, 5).; [O]. 5.; hunits (1, 2, 3, 4, 5, 6).; [C]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; HORACC (complex attribute "horizontalClearanceOpen – horizontalDistanceUncertainty"); [O]. 7.; HORCLR (complex attribute "horizontalClearanceOpen – horizontalClearanceValue"); [M]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; locmrn; [C]. 11.; PICREP; [O]. 13.; SORDAT (attribute "reportedDate"); [C]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; unlocd; [C]. 18.; VERACC (complex attribute "verticalClearanceOpen – verticalUncertainty – uncertaintyFixed"); [O]. 18.; VERCLR (complex attribute "verticalClearanceOpen – verticalClearanceValue"); [O]. 19.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [O]. 20.; wtwdis; [O]. 21.; vcrlev; [O]. 22.; vcrval; [O]. | 1.; categoryOfGate (2, 3, 4, 5, 6).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; depthRangeMinimumValue; [0,1]. 5.; distanceUnitOfMeasurement (1, 2, 3, 4, 5).; [1,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; horizontalClearanceOpen; [1,1]. horizontalClearanceValue; [1,1]. horizontalDistanceUncertainty; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; natureOfConstruction (1, 2, 6, 7).; [0,*]. 11.; pictorialRepresentation; [0,1]. 12.; qualityOfVerticalMeasurement (2, 3, 4, 6, 7).; [0,*]. 13.; reportedDate; [0,1]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 4, 6, 16, 17).; [0,*]. 17.; uNLocationCode; [0,1]. 18.; verticalClearanceOpen; [0,1]. verticalClearanceUnlimited; [1,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 19.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 20.; waterwayDistance; [1,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute HORACC shall be converted into the S-401 complex attribute HorizontalClearanceOpen sub-attribute horizontalDistanceUncertainty. - The S-57 attribute HORCLR shall be converted into the S-401 complex attribute horzizontalClearanceOpen sub-attribute horizontalClearanceValue. - The S-57 attribute VERACC shall be converted into the S-401 complex attribute verticalClearanceOpen sub-attribute verticalUncertainty sub-attribute uncertaintyFixed. - The S-57 attribute VERCLR shall be converted into the S-401 complex attribute verticalClearanceOpen sub-attribute verticalClearanceValue.

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.56 Gridiron (GRIDRN, Careening Grid)
A structure in the intertidal zone serving as a support for vessels at low stages of the tide to permit work on the exposed portion of the vessel's hull.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | surface |
| Attributes | Attributes |
| 1.; CONDTN (1, 2, 5).; [O]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; HORLEN; [O]. 4.; HORWID; [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; NATCON (2, 6).; [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; STATUS (4, 8, 14).; [O]. 13.; VERLEN; [O]. 14.; WATLEV (3, 4).; [O]. | 1.; condition (1, 2, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; horizontalLength; [0,1]. 4.; horizontalWidth; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; natureOfConstruction (1, 2, 6, 7, 11).; [0,*]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 4, 6, 8, 14, 28).; [0,*]. 13.; verticalLength; [0,1]. 14.; waterLevelEffect (1, 3, 4, 5).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

- For an existing S-57 object class GRIDRN with the primitive point, an S-401 gridiron feature has to be created as a surface.

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.57 Harbour Area (Administrative) (hrbare)
The area over which a harbour authority has jurisdiction. For IENCs the area of water and land with the works necessary for its formation, protection and maintenance.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; cathbr (1, 2, 3, 4, 5).; [O]. 2.; CONDTN (1, 2, 3, 5).; [C]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; locmrn; [C]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; STATUS (5).; [O]. 13.; unlocd; [C]. | 1.; categoryOfHarbourArea (1, 2, 3, 4, 5).; [0,*]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [1,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 4, 5, 6, 8, 14).; [0,*]. 13.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.58 Harbour Basin (hrbbsn)
An enclosed area of water surrounded by quay walls constructed to provide means for the transfer of cargos from and to ships.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CONDTN (1, 2, 3, 5).; [O]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; HORLEN; [O]. 5.; HORWID; [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (5).; [O]. 14.; unlocd; [C]. | 1.; condition (1, 2, 3, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; horizontalLength; [0,1]. 5.; horizontalWidth; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (5).; [0,*]. 14.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.59 Harbour Facility (HRBFAC, hrbfac)
A harbour installation with a service or commercial operation of public interest.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATHAF (5).; [M]. 3.; CONDTN (1, 2, 3, 5).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [M]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; STATUS (5).; [O]. 17.; unlocd; [C]. 1.; cathaf (4, 6, 9, 12, 13, 16, 17).; [M]. 3.; CONDTN (1, 2, 3, 5).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; STATUS (5).; [O]. 17.; unlocd; [C]. | 1.; categoryOfHarbourFacility (1, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17).; [1,*]. 2.; communicationChannel; [0,*]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; natureOfConstruction (1, 2, 3, 6, 7).; [0,*]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; product (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 25).; [0,1]. 12.; reportedDate; [0,1]. 13.; restriction (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18, 19, 20, 21, 23, 24, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 40, 41, 42, 43).; [0,*]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 4, 5, 6, 7, 8, 9, 12, 13, 14, 16, 17).; [0,*]. 17.; uNLocationCode; [0,1]. 18.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.60 Helipad (AIRARE (Point and CATAIR = 4))
A small landing surface for helicopters, with minimal or no supporting installations or facilities.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; CONDTN (1, 2, 3, 5).; [C]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information - text"); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PICREP; [O]. 8.; SORDAT ("reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication" - source); [C]. 11.; STATUS (5).; [O]. 12.; DATEND; [O]. 13.; CATAIR (1, 2, 4, 6).; [O]. 14.; DATSTA; [O]. 15.; PEREND; [O]. 16.; PERSTA; [O]. | 1.; condition (1, 2, 3, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; natureOfConstruction (1, 2, 4, 5, 6, 7).; [0,*]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 2, 4, 5, 6, 7, 8, 12, 14).; [0,*]. |

Conversion rules

Automatic conversion

- The S-57 object class AIRARE (Point) with the attribute CATAIR value 4 (civil heliport) shall be converted into the S-401 feature type Helipad with the corresponding attributes.

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.61 Hulk (HULKES, hulkes)
The hull of a wrecked or condemned ship, from which the fittings and superstructure have usually been removed, which is moored in a permanent position or grounded. It may be abandoned or put to some other use.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | point surface |
| Attributes | Attributes |
| 1.; CATHLK (1, 2, 3, 4, 5).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; HORACC; [O]. 11.; INFORM (complex attribute "information - text "); [O]. 11.; NINFOM (complex attribute "information - text"); [O]. 11.; TXTDSC (complex attribute "information - file reference"); [O]. 11.; NTXTDS (complex attribute "information - file reference "); [O]. 14.; PICREP; [O]. 16.; SORDAT (attribute "reportedDate"); [C]. 18.; SORIND (complex attribute "sourceIndication - source"); [C]. 21.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 1.; cathlk (1, 2, 3, 4, 5, 6).; [O]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; HORACC; [O]. 11.; INFORM (complex attribute "information - text "); [O]. 11.; NINFOM (complex attribute "information - text"); [O]. 11.; TXTDSC (complex attribute "information - file reference"); [O]. 11.; NTXTDS (complex attribute "information - file reference "); [O]. 12.; locmrn; [C]. 13.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 13.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 14.; PICREP; [O]. 16.; SORDAT (attribute "reportedDate"); [C]. 17.; SCAMIN; [M]. 18.; SORIND (complex attribute "sourceIndication - source"); [C]. 19.; unlocd; [C]. 21.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 23.; STATUS (5).; [O]. | 1.; categoryOfHulk (1, 2, 3, 4, 5, 6, 7).; [1,*]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; horizontalDistanceUncertainty; [0,1]. 9.; horizontalLength; [0,1]. 10.; horizontalWidth; [0,1]. 11.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 12.; interoperabilityIdentifier; [0,1]. 13.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 14.; pictorialRepresentation; [0,1]. 15.; radarConspicuous; [0,1]. 16.; reportedDate; [0,1]. 17.; scaleMinimum; [1,1]. 18.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 19.; uNLocationCode; [0,1]. 20.; verticalLength; [0,1]. 21.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 22.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

- If the S-57 object class HULKES is encoded, it must be ensured that the area beneath the pontoon is filled with the Group 1 object UnsurveyedArea.

Manual conversion

- If the S-57 object class HULKES, then encode the S-401 attribute scaleMiniumum with value 22000 for Europe, 30000 for US.

General rules:

### 2.1.5.2: Horizontal Accuracy (HORACC) ### 2.1.5.3: Vertical Accuracy (VERACC) ### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.62 Information Area (M_NPUB)
An area for which general information regarding navigation, but not directly related to safety of navigation, is available.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | point surface |
| Attributes | Attributes |
| 3.; TXTDSC (complex attribute "information - file reference"); [M]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 9.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

- If the S-57 object class is M_NPUB, than encode the S-401 attribute scaleMinimum with value 45000 for Europe, 60000 for US.

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.63 Inshore Traffic Zone (ISTZNE)
A routeing measure comprising a designated area between the landward boundary of a traffic separation scheme and the adjacent coast, to be used in accordance with the provisions of the International Regulations for Preventing Collisions at Sea.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface |
| Attributes | Attributes |
|  | 1.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; restriction (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 40, 41, 43).; [0,*]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; status (1, 3, 6, 9, 16, 17).; [0,*]. 10.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.64 Installation Buoy (BOYINB)
An installation buoy is a buoy used for loading tankers with gas or oil.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point |
| Attributes | Attributes |
|  | 1.; buoyShape (1, 2, 3, 4, 5, 6, 7, 8).; [1,1]. 2.; categoryOfInstallationBuoy (1, 2).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; natureOfConstruction (7, 11).; [0,*]. 10.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 11.; pictorialRepresentation; [0,1]. 12.; product (1, 2, 18, 19).; [0,*]. 13.; radarConspicuous; [0,1]. 14.; reportedDate; [0,1]. 15.; scaleMinimum; [1,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (1, 2, 4, 5, 7, 8, 18).; [0,*]. 18.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.65 Isolated Danger Beacon (BCNISD)
An isolated danger beacon is a beacon erected on an isolated danger of limited extent, which has navigable water all around it.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BCNSHP (1, 2, 3, 4, 5).; [M]. 2.; COLOUR (2, 3).; [M]. 3.; COLPAT (1).; [M]. 4.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; INFORM (complex attribute "information - text "); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 13.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 13.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 14.; PICREP; [O]. 15.; CONRAD (attribute "radarConspicuous") (3).; [O]. 16.; SORDAT (attribute "reportedDate"); [C]. 17.; SCAMIN; [M]. 18.; SORIND (complex attribute "sourceIndication - source"); [C]. 19.; STATUS (5).; [O]. 20. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation text"); [O]. NINFOM (complex attribute "topmark - shapeInformation text"); [O]. SCAMIN; [M]. SORDAT (attribute "sourceDate" or "reportedDate"); [C]. SORIND (attribute "sourceIndication; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; beaconShape (1, 2, 3, 4, 5, 6, 7).; [1,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; condition (1, 2, 5).; [0,1]. 5.; elevation; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 12.; natureOfConstruction (1, 2, 6, 7, 8).; [0,*]. 13.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 14.; pictorialRepresentation; [0,1]. 15.; radarConspicuous; [0,1]. 16.; reportedDate; [0,1]. 17.; scaleMinimum; [1,1]. 18.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 19.; status (1, 2, 4, 5, 7, 8, 12, 18).; [0,*]. 20.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 21.; verticalLength; [0,1]. 22.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR)

3.66 Isolated Danger Buoy (BOYISD)
An isolated danger buoy is a buoy moored on or above an isolated danger of limited extent, which has navigable water all around it.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BOYSHP (4, 5).; [M]. 2.; COLOUR (2, 3).; [M]. 3.; COLPAT (1).; [M]. 4.; OBJNAM (complex attribute "featureName - name"); [C]. 4.; NOBJNM (complex attribute "featureName - name"); [C]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - la file reference nguage"); [O]. 8.; MARSYS (1, 2).; [C]. 9.; mmsico; [O]. 11.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 11.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 12.; PICREP; [O]. 13.; CONRAD (attribute "radarConspicuous") (3).; [O]. 14.; SORDAT (attribute "reportedDate"); [C]. 15.; SCAMIN; [M]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; STATUS (5).; [O]. 18. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation text"); [O]. NINFOM (complex attribute "topmark - shapeInformation text"); [O]. SCAMIN; [M]. SORDAT (attribute "sourceDate" or "reportedDate"); [C]. SORIND (attribute "sourceIndication; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; buoyShape (1, 2, 3, 4, 5, 6, 7, 8).; [1,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 9.; mMSICode; [0,1]. 10.; natureOfConstruction (6, 7, 8, 11).; [0,*]. 11.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 12.; pictorialRepresentation; [0,1]. 13.; radarConspicuous; [0,1]. 14.; reportedDate; [0,1]. 15.; scaleMinimum; [1,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (1, 2, 5, 7, 8, 18).; [0,*]. 18.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 19.; verticalLength; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR)

3.67 Lake (LAKARE)
A large body of water entirely surrounded by land.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information" - text); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; PICREP; [O]. 6.; SORDAT (attribute "reportedDate"); [C]. 7.; SCAMIN; [M]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; elevation; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; status (5).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.68 Land Area (LNDARE)
The solid portion of the Earth's surface, as opposed to sea, water.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information - text "); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; PICREP; [O]. 6.; SORDAT (attribute "reportedDate"); [C]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; condition (1, 3, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [0,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; status (18).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

- Encode the S-401 attribute scaleMinimum with value 22000 for Europe, 30000 for US.

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.69 Land Elevation (LNDELV)
An elevation is the vertical distance of a point or a level, on, or affixed to, the surface of the earth, measured from a specified vertical datum.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point curve |
| Attributes | Attributes |
|  | 1.; elevation; [1,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 8.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.70 Landmark (LNDMRK)
Any prominent object at a fixed location on land which can be used in determining a location or a direction.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point curve surface |
| Attributes | Attributes |
| 1.; CATLMK (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21).; [M]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [O]. 5.; CONDTN (1, 2, 3, 4, 5).; [O]. 7.; OBJNAM (complex attribute "featureName - name"); [O]. 7.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; FUNCTN (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42).; [C]. 10.; INFORM (complex attribute "information - text "); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 10.; NTXTDS (complex attribute "information - file reference "); [O]. 15.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. 21.; VERLEN; [O]. 22.; CONVIS (attribute "visualProminence") (1).; [M]. | 1.; categoryOfLandmark (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 20, 21, 22, 23, 24, 25, 26, 27).; [1,*]. 2.; categoryOfSpecialPurposeMark (16, 17, 41).; [0,*]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 4, 5).; [0,1]. 6.; elevation; [0,1]. 7.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 8.; function (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 44, 45, 46, 47, 48).; [0,*]. 9.; height; [0,1]. 10.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 11.; interoperabilityIdentifier; [0,1]. 12.; inTheWater; [0,1]. 13.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 14.; natureOfConstruction (1, 2, 3, 6, 7, 8, 11, 12).; [0,*]. 15.; pictorialRepresentation; [0,1]. 16.; radarConspicuous; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; status (2, 4, 5, 7, 8, 12, 13, 14).; [0,*]. 21.; verticalLength; [0,1]. 22.; visualProminence (1, 2, 3).; [1,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.71 Land Region (LNDRGN)
An area of natural or cultivated scenery defined by its geographical characteristics and may be known by its proper name. (Adapted from S-57 Edition 3.1, Appendix A - Chapter 1, Page 1.92, November 2000). For IENCs: Land Areas adjacent to the waterway that are significant for navigation reference.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point curve surface |
| Attributes | Attributes |
| 1.; CATLND (2, 9, 11, 12).; [O]. 2.; CONDTN (1, 2, 3, 5).; [C]. 3.; OBJNAM (complex attribute "featureName - name"); [M]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfLandRegion (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21).; [0,*]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [1,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [0,*]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; waterLevelEffect (1, 6).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.72 Lateral Beacon (BCNLAT, bcnlat)
A lateral beacon is used to indicate the port or starboard hand side of the route to be followed. They are generally used for well defined channels and are used in conjunction with a conventional direction of buoyage. For IENCs also beacons that are used to mark specific “Landmarks” and submerged banks.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BCNSHP (1).; [M]. 2.; CATLAM (1, 2).; [M]. 3.; COLOUR (1, 2, 3, 4).; [M]. 4.; COLPAT (1).; [C]. 5.; CONDTN (1, 2, 3, 5).; [O]. 8.; OBJNAM (complex attribute "featureName - name"); [O]. 8.; NOBJNM (complex attribute "featureName - name"); [O]. 9.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 9.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 11.; INFORM (complex attribute "information - text "); [O]. 11.; NINFOM (complex attribute "information - text"); [O]. 11.; TXTDSC (complex attribute "information - file reference"); [O]. 11.; NTXTDS (complex attribute "information - file reference "); [O]. 16.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 16.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 17.; PICREP; [O]. 19.; SORDAT (attribute "reportedDate"); [C]. 20.; SCAMIN; [M]. 21.; SORIND (complex attribute "sourceIndication - source"); [C]. 22.; STATUS (5).; [O]. 1.; BCNSHP (1, 5).; [M]. 2.; catlam (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27).; [M]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 4.; COLPAT (1, 2, 3, 4, 5, 6).; [C]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; dirimp (1, 2, 3, 4).; [C]. 8.; OBJNAM (complex attribute "featureName - name"); [O]. 8.; NOBJNM (complex attribute "featureName - name"); [O]. 9.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 9.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 11.; INFORM (complex attribute "information - text "); [O]. 11.; NINFOM (complex attribute "information - text"); [O]. 11.; TXTDSC (complex attribute "information - file reference"); [O]. 11.; NTXTDS (complex attribute "information - file reference "); [O]. 14.; mmsico; [O]. 16.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 16.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 17.; PICREP; [O]. 19.; SORDAT (attribute "reportedDate"); [C]. 20.; SCAMIN; [M]. 21.; SORIND (complex attribute "sourceIndication - source"); [C]. 22.; STATUS (5).; [O]. 24.; typatn (1, 2, 3).; [O]. 23. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation text"); [O]. NINFOM (complex attribute "topmark - shapeInformation text"); [O]. SCAMIN; [M]. SORDAT (attribute "sourceDate" or "reportedDate"); [C]. SORIND (attribute "sourceIndication; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; beaconShape (1, 2, 3, 4, 5, 6, 7).; [1,1]. 2.; categoryOfLateralMark (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27).; [1,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; directionOfImpact (1, 2, 3, 4, 5).; [0,*]. 7.; elevation; [0,1]. 8.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 9.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 10.; height; [0,1]. 11.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 12.; interoperabilityIdentifier; [0,1]. 13.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 14.; mMSICode; [0,1]. 15.; natureOfConstruction (1, 2, 6, 7, 8).; [0,*]. 16.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 17.; pictorialRepresentation; [0,1]. 18.; radarConspicuous; [0,1]. 19.; reportedDate; [0,1]. 20.; scaleMinimum; [1,1]. 21.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 22.; status (1, 2, 4, 5, 7, 8, 12, 18).; [0,*]. 23.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 24.; typeOfAtoN (1, 2, 3).; [0,1]. 25.; verticalLength; [0,1]. 26.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.7: Topmark (TOPMAR)

3.73 Lateral Buoy (BOYLAT, boylat)
A lateral buoy is used to indicate the port or starboard hand side of the route to be followed. They are generally used for well-defined channels and are used in conjunction with a conventional direction of buoyage. For IENCs this includes: -Buoys at bifurcations of channels (A buoy at a fairway junction may indicate by its top mark on which side it is preferable to pass (main channel)).- Buoys at bridge pillars (A buoy at a bridge pillar may be used to improve the visibility of the pillar location on the radar.)- Buoys marking danger points (Buoys to indicate the presence of potentially dangerous obstructions such as groins, banks, or wrecks.)- Stalling buoys (The buoys (floating beacons) are used to mark stalling current which does not coincide with a direction of the fairway.)- Swinging lateral buoys (The buoys are used to mark swinging points at the edges of the extended rectilinear fairways, as well as at the fairway edges where the vision is limited.)

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BOYSHP (1, 2, 3, 4, 5, 6, 8).; [M]. 2.; CATLAM (1, 2, 3, 4).; [M]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 4.; COLPAT (1, 2, 3, 4, 5, 6).; [C]. 5.; OBJNAM (complex attribute "featureName - name"); [C]. 5.; NOBJNM (complex attribute "featureName - name"); [C]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information - text "); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; MARSYS (1, 2).; [C]. 10.; mmsico; [O]. 12.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 12.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 13.; PICREP; [O]. 14.; CONRAD (attribute "radarConspicuous") (3).; [O]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 18.; STATUS (5).; [O]. 20.; typatn (1, 2, 3).; [O]. 1.; BOYSHP (1, 2, 3, 4, 5, 6, 8).; [M]. 2.; catlam (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27).; [M]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 4.; COLPAT (1, 2, 3, 4, 5, 6).; [C]. 5.; OBJNAM (complex attribute "featureName - name"); [C]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information - text "); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; marsys (1, 2, 9, 10, 11, 12, 13, 15).; [C]. 10.; mmsico; [O]. 12.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 12.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 13.; PICREP; [O]. 14.; CONRAD (attribute "radarConspicuous") (3).; [O]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 18.; STATUS (5).; [O]. 20.; typatn (1, 2, 3).; [O]. 19. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation text"); [O]. NINFOM (complex attribute "topmark - shapeInformation text"); [O]. SCAMIN; [M]. SORDAT (attribute "sourceDate" or "reportedDate"); [C]. SORIND (attribute "sourceIndication; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; buoyShape (1, 2, 3, 4, 5, 6, 7, 8).; [1,1]. 2.; categoryOfLateralMark (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27).; [1,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 10.; mMSICode; [0,1]. 11.; natureOfConstruction (6, 7, 8, 11).; [0,*]. 12.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 13.; pictorialRepresentation; [0,1]. 14.; radarConspicuous; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (1, 2, 5, 7, 8, 18).; [0,*]. 19.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 20.; typeOfAtoN (1, 2, 3).; [0,1]. 21.; verticalLength; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR)

3.74 Light Air Obstruction (LIGHTS, CALIT=6)
An air obstruction light is a light marking an obstacle which constitutes a danger to air navigation.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 2.; CONDTN (1, 2, 3, 5).; [C]. 3.; EXCLIT (1, 2, 3, 4).; [C]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; HEIGHT; [O]. 8.; INFORM (complex attribute "information - text "); [C]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 10.; LITVIS (3, 4, 7, 8).; [C]. 11.; MLTYLT (complex attribute "multiplicityOfFeatures - numberOfFeatures"); [C]. 12.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 12.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 13.; PICREP; [O]. 14.; SORDAT (attribute "reportedDate"); [C]. 15.; LITCHR (complex attribute "rhythmOfLight - lightCharacteristic") (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 25, 26, 27, 28, 29).; [M]. 15.; SIGGRP (complex attribute "rhythmOfLight - signalGroup"); [C]. 15.; SIGPER (complex attribute "rhythmOfLight - signalPeriod"); [C]. 15.; SIGSEQ (complex attribute "rhythmOfLight - signalSequence"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 18.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [C]. 19.; VALNMR; [O]. 21.; CATLIT (1, 4, 5, 6, 12, 13, 14, 15).; [C]. 22.; ORIENT; [C]. 23.; SECTR1; [C]. 24.; SECTR2; [C]. | 1.; colour (1, 3, 4, 5, 6, 9, 10, 11).; [0,*]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; exhibitionConditionOfLight (1, 2, 3, 4).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; flareBearing; [0,1]. 7.; height; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; lightVisibility (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,*]. 11.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 12.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 13.; pictorialRepresentation; [0,1]. 14.; reportedDate; [0,1]. 15.; rhythmOfLight; [0,1]. lightCharacteristic (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 25, 26, 27, 28, 29).; [1,1]. signalGroup; [0,*]. signalPeriod; [0,1]. signalSequence; [0,*]. signalDuration; [1,1]. signalStatus (1, 2).; [1,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (1, 2, 4, 5, 6, 7, 8, 11, 14, 15, 16, 17).; [0,*]. 19.; valueOfNominalRange; [0,1]. 20.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute LITCHR into the S-401 complex attribute rhythmOfLight - lightCharacteristic if the S-57 attribute LITCHR is used, - The conversion shall copy the value of the S-57 attribute MLTYLT into the S-401 complex attribute multiplicityOfFeatures - numberOfFeatures if the S-57 attribute MLTYLT is used, - The conversion shall copy the value of the S-57 attribute SIGGRP into the S-401 complex attribute rhythmOfLight - signalGroup if the S-57 attribute SIGGRP is used, - The conversion shall copy the value of the S-57 attribute SIGPER into the S-401 complex attribute rhythmOfLight - signalPeriod if the S-57 attribute SIGPER is used, - The conversion shall copy the value of the S-57 attribute SIGSEQ into the S-401 complex attribute rhythmOfLight - signalSequence if the S-57 attribute SIGSEQ is used,

Manual conversion

General rules:

### 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.75 Light All Around (LIGHTS)
An all around light is a light that is visible over the whole horizon of interest to marine navigation and having no change in the characteristics of the light.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; CATLIT (1, 4, 5, 6, 12, 13, 14, 15).; [C]. 2.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 3.; CONDTN (1, 2, 3, 5).; [C]. 4.; EXCLIT (1, 2, 3, 4).; [C]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; HEIGHT; [O]. 9.; INFORM (complex attribute "information - text "); [C]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; LITVIS (3, 4, 7, 8).; [C]. 14.; MLTYLT (complex attribute "multiplicityOfFeatures - numberOfFeatures"); [C]. 15.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 15.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 16.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; LITCHR (complex attribute "rhythmOfLight - lightCharacteristic") (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 25, 26, 27, 28, 29).; [M]. 18.; SIGGRP (complex attribute "rhythmOfLight - signalGroup"); [C]. 18.; SIGPER (complex attribute "rhythmOfLight - signalPeriod"); [C]. 18.; SIGSEQ (complex attribute "rhythmOfLight - signalSequence"); [C]. 19.; SCAMIN; [M]. 21.; SORIND (complex attribute "sourceIndication - source"); [C]. 22.; STATUS (2, 3, 4, 5.8, 9, 12, 14, 16, 17).; [C]. 23.; VALNMR; [O]. 26.; ORIENT; [C]. 27.; SECTR1; [C]. 28.; SECTR2; [C]. | 1.; categoryOfLight (4, 5, 8, 9, 10, 11, 12, 13, 14, 15, 17, 18, 19, 20).; [0,*]. 2.; colour (1, 3, 4, 5, 6, 9, 10, 11).; [1,*]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; exhibitionConditionOfLight (1, 2, 3, 4).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; flareBearing; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; lightVisibility (1, 2, 3, 7, 8).; [0,1]. 12.; majorLight; [0,1]. 13.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 14.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 15.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 16.; pictorialRepresentation; [0,1]. 17.; reportedDate; [0,1]. 18.; rhythmOfLight; [1,1]. lightCharacteristic (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 25, 26, 27, 28, 29).; [1,1]. signalGroup; [0,*]. signalPeriod; [0,1]. signalSequence; [0,*]. signalDuration; [1,1]. signalStatus (1, 2).; [1,1]. 19.; scaleMinimum; [1,1]. 20.; signalGeneration (5, 6).; [0,1]. 21.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 22.; status (1, 2, 4, 5, 6, 7, 8, 11, 14, 15, 16, 17).; [0,*]. 23.; valueOfNominalRange; [0,1]. 24.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 25.; verticalLength; [0,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute LITCHR into the S-401 complex attribute rhythmOfLight - lightCharacteristic if the S-57 attribute LITCHR is used, - The conversion shall copy the value of the S-57 attribute MLTYLT into the S-401 complex attribute multiplicityOfFeatures - numberOfFeatures if the S-57 attribute MLTYLT is used, - The conversion shall copy the value of the S-57 attribute SIGGRP into the S-401 complex attribute rhythmOfLight - signalGroup if the S-57 attribute SIGGRP is used, - The conversion shall copy the value of the S-57 attribute SIGPER into the S-401 complex attribute rhythmOfLight - signalPeriod if the S-57 attribute SIGPER is used, - The conversion shall copy the value of the S-57 attribute SIGSEQ into the S-401 complex attribute rhythmOfLight - signalSequence if the S-57 attribute SIGSEQ is used, - The S-57 object class LIGHTS with the attribute value 1 for CATLIT and/or the attributes SECTR1 and SECTR2 are present, shall be converted into the S-401 feature type lightSectored. - The S-57 object class LIGHTS with the attribute value 6 for CATLIT shall be converted into the S-401 feature type lightAirObstruction.

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.76 Light Sectored (LIGHTS, CATLIT=1)
A light presenting different appearances (in particular, different colours) over various parts of the horizon of interest to maritime navigation.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; CATLIT (1, 4, 5, 6, 12, 13, 14, 15).; [C]. 2.; EXCLIT (1, 2, 3, 4).; [C]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; HEIGHT; [O]. 6.; INFORM (complex attribute "information - text "); [C]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; MLTYLT (complex attribute "multiplicityOfFeatures - numberOfFeatures"); [C]. 10.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 10.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 11.; PICREP; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; COLOUR(complex attribute "sectorCharacteristics - lightSector - colour") (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 14.; LITCHR (complex attribute "sectorCharacteristics - lightCharacteristic") (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 25, 26, 27, 28, 29).; [M]. 14.; LITVIS (complex attribute "sectorCharacteristics - lightSector - lightVisibility") (3, 4, 7, 8).; [C]. 14.; SECTR1 (complex attribute "sectorCharacteristics - lightSector - sectorLimit"); [C]. 14.; SECTR2 (complex attribute "sectorCharacteristics - lightSector - sectorLimit"); [C]. 14.; SIGGRP (complex attribute "sectorCharacteristics - signalGroup"); [C]. 14.; SIGPER (complex attribute "sectorCharacteristics - signalPeriod"); [C]. 14.; SIGSEQ (complex attribute "sectorCharacteristics - signalSequence"); [C]. 14.; VALNMR (complex attribute "sectorCharacteristics - lightSector - valueOfNominalRange"); [O]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [C]. 19.; CONDTN (1, 2, 3, 5).; [C]. 20.; ORIENT; [C]. | 1.; categoryOfLight (4, 5, 8, 9, 10, 11, 12, 13, 14, 15, 17, 18, 19, 20).; [0,*]. 2.; exhibitionConditionOfLight (1, 2, 3, 4).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; height; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 9.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 10.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 11.; pictorialRepresentation; [0,1]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sectorCharacteristics; [1,*]. lightCharacteristic (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 25, 26, 27, 28, 29).; [1,1]. lightSector; [1,*]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. directionalCharacter; [0,1]. moireEffect; [0,1]. orientation; [1,1]. orientationUncertainty; [0,1]. orientationValue; [1,1]. lightVisibility (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,*]. sectorArcExtension; [0,1]. sectorInformation; [0,*]. sectorLimit; [0,1]. sectorLimitOne; [1,1]. sectorBearing; [1,1]. sectorLineLength; [0,1]. sectorLimitTwo; [1,1]. sectorBearing; [1,1]. sectorLineLength; [0,1]. valueOfNominalRange; [0,1]. signalGroup; [0,*]. signalPeriod; [0,1]. signalSequence; [0,*]. signalDuration; [1,1]. signalStatus (1, 2).; [1,1]. 15.; signalGeneration (5, 6).; [0,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (1, 2, 4, 5, 6, 7, 8, 11, 14, 15, 16, 17).; [0,*]. 18.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute LITCHR into the S-401 complex attribute sectorCharacteristics - lightCharacteristic if the S-57 attribute LITCHR is used, - The conversion shall copy the value of the S-57 attribute MLTYLT into the S-401 complex attribute multiplicityOfFeatures - numberOfFeatures if the S-57 attribute MLTYLT is used, - The conversion shall copy the value of the S-57 attribute SIGGRP into the S-401 complex attribute sectorCharacteristics - signalGroup if the S-57 attribute SIGGRP is used, - The conversion shall copy the value of the S-57 attribute SIGPER into the S-401 complex attribute sectorCharacteristics - signalPeriod if the S-57 attribute SIGPER is used, - The conversion shall copy the value of the S-57 attribute SIGSEQ into the S-401 complex attribute sectorCharacteristics - signalSequence if the S-57 attribute SIGSEQ is used, - The conversion shall copy the value of the S-57 attribute COLOUR into the S-401 complex attribute sectorCharacteristics - lightSector - colour if the S-57 attribute COLOUR is used, - The conversion shall copy the value of the S-57 attribute LITVIS into the S-401 complex attribute sectorCharacteristics - lightSector - lightVisibility if the S-57 attribute LITVIS is used, - The conversion shall copy the value of the S-57 attribute SECTR1 into the S-401 complex attribute sectorCharacteristics - lightSector - sectorLimit - sectorLimitOne if the S-57 attribute SECTR1 is used, - The conversion shall copy the value of the S-57 attribute SECTR2 into the S-401 complex attribute sectorCharacteristics - lightSector - sectorLimit – sectorLimitTwo if the S-57 attribute SECTR2 is used, - The conversion shall copy the value of the S-57 attribute VALNMR into the S-401 complex attribute sectorCharacteristics - lightSector - valueOfNominalRange if the S-57 attribute VALNMR is used. - All instances of the S-57 Object class LIGHTS having: • attributes SECTR1 and SECTR2 present; and/or • attribute CATLIT = 1 (directional function) shall be converted to an instance of the S-401 feature type lightSectored with the corresponding attributes. However, the following exceptions apply: • LIGHTS having attribute LITVIS = 7 (obscured) will not be converted.

Manual conversion

- The S-57 attribute INFORM and NINFOM can be converted into the S-401 attribute information or in the complex attribute sectorCharacteristics – lightSector – sectorInformation.

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.77 Local Direction of Buoyage (m_nsys)
An area within which the navigational system of marks has been established in relation to a specific direction.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 3.; marsys (1, 2, 9, 10, 11, 12, 13, 15).; [M]. 4.; ORIENT; [C]. 5.; SORDAT (attribute "reportedDate"); [C]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [1,1]. 4.; orientationValue; [1,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.5: InteroperabilityIdentifier

3.78 Lock Basin (LOKBSN)
A wet dock in a waterway, permitting a ship to pass from one level to another.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; horclw; [C]. 6.; horcll; [C]. 7.; HORACC; [O]. 8.; HORLEN; [O]. 9.; HORWID; [O]. 10.; INFORM (complex attribute "information - text "); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 10.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; locmrn; [C]. 12.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 12.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 13.; PICREP; [O]. 14.; SORDAT (attribute "reportedDate"); [C]. 15.; SCAMIN; [M]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; STATUS (5).; [O]. 18.; unlocd; [C]. 19.; VERACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; horizontalClearanceFixed; [0,1]. horizontalClearanceValue; [1,1]. horizontalDistanceUncertainty; [0,1]. 6.; horizontalClearanceLength; [1,1]. 7.; horizontalDistanceUncertainty; [0,1]. 8.; horizontalLength; [0,1]. 9.; horizontalWidth; [0,1]. 10.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 11.; interoperabilityIdentifier; [0,1]. 12.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 13.; pictorialRepresentation; [0,1]. 14.; reportedDate; [0,1]. 15.; scaleMinimum; [1,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (1, 4, 5, 6, 8, 13, 14, 16, 17).; [0,*]. 18.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.1.5.2: Horizontal Accuracy (HORACC) - 2.1.5.3: Vertical Accuracy (VERACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.79 Lock Basin Part (lkbspt)
A lock basin is divided into several lock basin parts, if this lock basin has one ground level but several gates.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; horcll; [C]. 6.; horclw; [C]. 7.; HORACC; [O]. 8.; HORLEN; [O]. 9.; HORWID; [O]. 10.; INFORM (complex attribute "information - text "); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 10.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; locmrn; [C]. 12.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 12.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 13.; PICREP; [O]. 14.; SORDAT (attribute "reportedDate"); [C]. 15.; SCAMIN; [M]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; STATUS (5).; [O]. 18.; unlocd; [C]. 19.; VERACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; condition (1, 2, 3, 4, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; horizontalClearanceLength; [1,1]. 6.; horizontalClearanceWidth; [1,1]. 7.; horizontalDistanceUncertainty; [0,1]. 8.; horizontalLength; [0,1]. 9.; horizontalWidth; [0,1]. 10.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 11.; interoperabilityIdentifier; [0,1]. 12.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 13.; pictorialRepresentation; [0,1]. 14.; reportedDate; [0,1]. 15.; scaleMinimum; [1,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (5).; [0,*]. 18.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.1.5.2: Horizontal Accuracy (HORACC) - 2.1.5.3: Vertical Accuracy (VERACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.80 Magnetic Variation (MAGVAR, Magnetic Declination, Variation)
The angle between the magnetic and geographic meridians at any place, expressed in degrees east or west to indicate the direction of magnetic north from true north. Also called magnetic declination.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; INFORM (complex attribute "information - text "); [O]. 1.; NINFOM (complex attribute "information - text"); [O]. 1.; TXTDSC (complex attribute "information - file reference"); [O]. 1.; NTXTDS (complex attribute "information - file reference "); [O]. 3.; RYRMGV; [M]. 4.; SORDAT (attribute "reportedDate"); [C]. 5.; SCAMIN; [M]. 6.; SORIND (complex attribute "sourceIndication - source"); [C]. 7.; VALACM; [M]. 8.; VALMAG; [M]. 9.; DATEND; [O]. 10.; DATSTA; [O]. 11.; OBJNAM; [O]. 12.; NOBJNM; [O]. 13.; PICREP; [O]. | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; referenceYearForMagneticVariation; [1,1]. 4.; reportedDate; [0,1]. 5.; scaleMinimum; [1,1]. 6.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 7.; valueOfAnnualChangeInMagneticVariation; [1,1]. 8.; valueOfMagneticVariation; [1,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.81 Marine Farm/Culture (MARCUL)
An assemblage of cages, nets, rafts and floats or posts where fish, including shellfish, are artificially cultivated.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; CATMFA (1, 2, 3, 4).; [M]. 2.; EXPSOU (1, 2, 3).; [C]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; QUASOU (1, 2, 3, 4, 6, 7, 8, 9).; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; STATUS (2, 4, 5).; [C]. 16.; VALSOU; [C]. 18.; SOUACC (complex attribute "verticalUncertainty - uncertaintyFixed "); [C]. 20.; WATLEV (1, 2, 3, 4, 5, 7).; [C]. | 1.; categoryOfMarineFarmCulture (1, 2, 3, 4, 5).; [1,1]. 2.; expositionOfSounding (1, 2, 3).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; height; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; qualityOfVerticalMeasurement (1, 2, 3, 4, 6, 7, 8, 9).; [0,*]. 11.; reportedDate; [0,1]. 12.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43).; [0,*]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (1, 2, 4, 5, 6, 7, 8, 14, 16, 17, 28).; [0,*]. 16.; valueOfSounding; [0,1]. 17.; verticalLength; [0,1]. 18.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 19.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. 20.; waterLevelEffect (1, 2, 3, 4, 5, 7).; [1,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute SOUACC into the S-401 complex attribute verticalUncertainty - uncertaintyFixed if the S-57 attribute SOUACC is used,

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.82 Maximum Permitted Ship Dimensions (lg_sdm)
Waterway or waterway section for which a juridical regulation with respect to the maximum permitted vessel dimensions exists.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; lc_ase (1, 2, 3, 5, 6, 7, 8, 9, 10).; [O]. 2.; lc_asi (1, 2, 3, 5, 6, 7, 8, 9, 10).; [O]. 3.; lc_cce (1, 2, 4, 5, 6, 7, 8, 9).; [O]. 4.; lc_cci (1, 2, 4, 5, 6, 7, 8, 9).; [O]. 5.; lc_cse (1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32).; [O]. 6.; lc_csi (1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32).; [O]. 7.; lg_des; [O]. 8.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 8.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; lg_bme; [O]. 10.; lg_drt; [O]. 11.; lg_lgs; [O]. 12.; lg_wdp; [O]. 13.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 13.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 14.; lg_pbr; [O]. 15.; lg_rel (1, 2, 3, 4).; [O]. 16.; SORDAT (attribute "reportedDate"); [C]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 18.; lg_wdu (1, 2, 3).; [O]. | 1.; assembliesOfShipExcluding (1, 2, 3, 5, 6, 7, 8, 9, 10).; [0,*]. 2.; assembliesOfShipIncluding (1, 2, 3, 5, 6, 7, 8, 9, 10).; [0,*]. 3.; categoryOfCargoExcluding (1, 2, 4, 5, 6, 7, 8, 9).; [0,*]. 4.; categoryOfCargoIncluding (1, 2, 4, 5, 6, 7, 8, 9).; [0,*]. 5.; categoryOfShipExcluding (1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32).; [0,*]. 6.; categoryOfShipIncluding (1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32).; [0,*]. 7.; descriptionOfLegalConditions; [0,1]. 8.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 9.; maximalPermittedBeam; [0,1]. 10.; maximalPermittedDraught; [0,1]. 11.; maximalPermittedLength; [0,1]. 12.; maximalPermittedWaterDisplacement; [0,1]. 13.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 14.; publicationReference; [0,1]. 15.; relatedIssue (1, 2, 3, 4).; [0,1]. 16.; reportedDate; [0,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; waterDisplacementUnit (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND)

3.83 Maximum Permitted Vessel Speed (lg_vsp)
Waterway or waterway section for which a juridical regulation with respect to the maximum permitted vessel speed exists.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; lc_ase (1, 2, 3, 5, 6, 7, 8, 9, 10).; [O]. 2.; lc_asi (1, 2, 3, 5, 6, 7, 8, 9, 10).; [O]. 3.; lc_cce (1, 2, 4, 5, 6, 7, 8, 9).; [O]. 4.; lc_cci (1, 2, 4, 5, 6, 7, 8, 9).; [O]. 5.; lc_cse (1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32).; [O]. 6.; lc_csi (1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32).; [O]. 7.; lg_des; [O]. 8.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 8.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; lg_spd; [O]. 10.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 10.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 11.; lg_pbr; [O]. 12.; lg_rel (1, 2, 3, 4).; [O]. 13.; SORDAT (attribute "reportedDate"); [C]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; lg_spr (1, 2, 3).; [O]. 16.; lg_wdu (1, 2, 3).; [O]. | 1.; assembliesOfShipExcluding (1, 2, 3, 5, 6, 7, 8, 9, 10).; [0,*]. 2.; assembliesOfShipIncluding (1, 2, 3, 5, 6, 7, 8, 9, 10).; [0,*]. 3.; categoryOfCargoExcluding (1, 2, 4, 5, 6, 7, 8, 9).; [0,*]. 4.; categoryOfCargoIncluding (1, 2, 4, 5, 6, 7, 8, 9).; [0,*]. 5.; categoryOfShipExcluding (1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32).; [0,*]. 6.; categoryOfShipIncluding (1, 2, 3, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32).; [0,*]. 7.; descriptionOfLegalConditions; [0,1]. 8.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 9.; maximalPermittedSpeed; [0,1]. 10.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 11.; publicationReference; [0,1]. 12.; relatedIssue (1, 2, 3, 4).; [0,1]. 13.; reportedDate; [0,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; speedReference (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND)

3.84 Military Practice Area (MIPARE, Exercise Area)
An area within which naval, military or aerial exercises are carried out.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATMPA (2, 3, 4, 5, 6).; [O]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information" - text); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - v"); [O]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; RESTRN (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, , 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26).; [O]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (2, 4, 5).; [O]. | 1.; categoryOfMilitaryPracticeArea (2, 3, 4, 5, 6).; [0,*]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; nationality; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 39).; [0,*]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 2, 4, 5, 6, 7, 16, 17).; [0,*]. 14.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.85 Mooring Area (achare, catach=1, 2, 3)
An area in which vessels may be secured to mooring buoys.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; catach (1, 2, 3, 4, 5, 6, 7, 9, 10, 11, 12, 13).; [O]. 2.; clsdng (1, 2, 3, 4, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; locmrn; [C]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 12.; restrn (1, 2, 7, 8, 13, 14, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 38).; [O]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [O]. 17.; NATSUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [O]. 18.; unlocd; [C]. | 1.; categoryOfMooringArea (1, 2, 3).; [0,*]. 2.; classOfDangerousCargo (1, 2, 3, 4, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; maximumPermittedDraught; [0,1]. 8.; maximumPermittedVesselLength; [0,1]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; reportedDate; [0,1]. 12.; restriction (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18, 19, 20, 21, 23, 24, 25, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 42, 43).; [0,*]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (1, 2, 3, 5, 6, 7, 8, 9, 14).; [0,*]. 16.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 object class achare with the values 4-13 for the attribute catach, shall be converted into the S-401 feature type anchorageArea.

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.86 Mooring Buoy (MORFAC, CATMOR=7 and point)
A buoy secured to the bottom by permanent moorings with means for mooring a vessel by use of its anchor chain or mooring lines.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point |
| Attributes | Attributes |
| 1.; BOYSHP (2, 3, 7).; [C]. 2.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 10.; NATCON (1, 2, 3, 4, 5, 6, 7, 8, 9).; [O]. 11.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 11.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 12.; PICREP; [O]. 13.; SORDAT (attribute "reportedDate"); [C]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; STATUS (5).; [O]. 19.; CATMOR (1, 2, 3, 4, 5, 7).; [M]. 20.; CATTEV (4, 5, 6).; [O]. 21.; CONDTN (1, 2, 3, 5).; [O]. 22.; HORACC; [O]. 23.; VERACC; [O]. 24.; WATLEV (1, 2, 3, 4, 5).; [O]. | 1.; buoyShape (1, 2, 3, 4, 5, 6, 7, 8).; [1,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; maximumPermittedDraught; [0,1]. 9.; maximumPermittedVesselLength; [0,1]. 10.; natureOfConstruction (7, 8, 11).; [0,*]. 11.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 12.; pictorialRepresentation; [0,1]. 13.; reportedDate; [0,1]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 2, 4, 5, 7, 8, 18).; [0,*]. 17.; verticalLength; [0,1]. 18.; visitorsMooring; [0,1]. |

Conversion rules

Automatic conversion

- If the value of the S-57 attribute CATMOR is 7 and the primitive is point in the feature MORFAC, the feature must be converted into the S-401 feature mooringBuoy.

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.87 Mooring Trot (C_AGGR)
A mooring is a place where a vessel may be secured. A mooring trot is a mooring that is composed of ground tackle, mooring cables, buoys and mooring berths on junction cables.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface noGeometry |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [M]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information - text "); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; locmrn; [C]. 5.; PICREP; [O]. 6.; SORDAT (attribute "reportedDate"); [C]. 7.; SCAMIN; [O]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. 9.; unlocd; [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

- The use of a mooring trot aggregation has not been described in edition 2.6. If the mooring trots should be aggregated, it should be done manually.

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.88 Navigational System of Marks (m_nsys)
An area within which the navigational system of marks has been established in relation to a specific direction.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 3.; marsys (1, 2, 9, 10, 11, 12, 13, 15).; [M]. 4.; SORDAT (attribute "reportedDate"); [C]. 5.; SORIND (complex attribute "sourceIndication - source"); [C]. 6.; ORIENT; [C]. | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [1,1]. 4.; reportedDate; [0,1]. 5.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.5: InteroperabilityIdentifier

3.89 Navigation Line (NAVLNE)
A straight line extending towards an area of navigational interest and generally generated by two navigational aids or one navigational aid and a bearing. For IENCs a navigation line either defines a recommended track or marks the boundary between a safe and a dangerous area.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATNAV (1, 2, 3).; [C]. 2.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 2.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 3.; INFORM (complex attribute "information - text "); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; ORIENT (complex attribute "orientation -orientationValue "); [M]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; STATUS (5).; [O]. 13.; OBJNAM; [O]. 14.; NOBJNM; [O]. | 1.; categoryOfNavigationLine (1, 2, 3).; [1,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; measuredDistance; [0,1]. 6.; orientation; [1,1]. orientationUncertainty; [0,1]. orientationValue; [1,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 2, 5, 7, 8, 14).; [0,*]. |

Conversion rules

Automatic conversion

- The S-57 attribute ORIENT shall be converted into the S-401 complex attribute orientation sub-attribute orientationValue.

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.90 Notice Mark (notmrk)
A notice board or sign indicating information to the mariner.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; addmrk (1, 2, 3, 4, 5).; [O]. 2.; bnkwtw (1, 2).; [O]. 3.; catnmk (1 - 123).; [M]. 4.; CONDTN (1, 2, 3, 5).; [O]. 5.; dirimp (1, 2, 3, 4, 5).; [O]. 6.; disbk1; [O]. 7.; disbk2; [O]. 8.; disipd; [O]. 9.; disipu; [O]. 10.; OBJNAM (complex attribute "featureName - name"); [O]. 10.; NOBJNM (complex attribute "featureName - name"); [O]. 11.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 11.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 12.; fnctnm (1, 2, 3, 4, 5).; [M]. 13.; INFORM (complex attribute "information" - text); [O]. 13.; NINFOM (complex attribute "information - text"); [O]. 13.; TXTDSC (complex attribute "information - file reference"); [O]. 13.; NTXTDS (complex attribute "information - file reference "); [O]. 14.; marsys (1, 2, 9, 10, 11, 12, 13, 15).; [O]. 15.; mmsico; [O]. 16.; ORIENT; [C]. 17.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 17.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 18.; PICREP; [O]. 19.; SORDAT (attribute "reportedDate"); [C]. 20.; SCAMIN; [M]. 21.; SORIND (complex attribute "sourceIndication - source"); [C]. 22.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17).; [O]. | 1.; additionalMark (1, 2, 3, 4, 5).; [0,*]. 2.; bankOfTheWaterway (1, 2).; [0,1]. 3.; categoryOfNoticeMark (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 60, 61, 62, 63, 64, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 77, 78, 79, 80, 81, 82, 83, 84, 85, 86, 87, 88, 89, 90, 91, 92, 93, 94, 95, 96, 97, 98, 99, 100, 101, 102, 103, 104, 105, 106, 107, 108, 109, 110, 111, 112, 113, 114, 115, 116, 117, 118, 119, 120, 121, 122, 123, 124, 125, 126, 127, 128).; [1,1]. 4.; condition (1, 2, 5).; [0,1]. 5.; directionOfImpact (1, 2, 3, 4, 5).; [0,*]. 6.; distanceFromNoticeMarkFirst; [0,1]. 7.; distanceFromNoticeMarkSecond; [0,1]. 8.; distanceOfImpactDownstream; [0,1]. 9.; distanceOfImpactUpstream; [0,1]. 10.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 11.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 12.; functionOfNoticeMark (1, 2, 3, 4, 5).; [1,1]. 13.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 14.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 15.; mMSICode; [0,1]. 16.; orientationValue; [0,1]. 17.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 18.; pictorialRepresentation; [0,1]. 19.; reportedDate; [0,1]. 20.; scaleMinimum; [1,1]. 21.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 22.; status (2, 3, 4, 5, 8, 9, 12, 14, 16, 17, 18).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.91 Obstruction (OBSTRN)
In marine navigation, anything that hinders or prevents movement, particularly anything that endangers or prevents passage of a vessel. The term is usually used to refer to an isolated danger to navigation, such as a sunken rock or pinnacle. Examples of obstructions include: snags, stumps, wellheads, diffusers, cribs, fish havens, foul areas, foul grounds, booms, ice booms and ground tackle.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; CATOBS (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11).; [O]. 2.; CATTEV (4, 5, 6).; [O]. 5.; EXPSOU (1, 2, 3).; [C]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 9.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; NATCON (1, 2, 3, 4, 6, 7, 8).; [O]. 12.; NATSUR (9).; [C]. 13.; PICREP; [O]. 15.; QUASOU (2, 6, 7, 8).; [O]. 16.; SORDAT (attribute "reportedDate"); [C]. 17.; SCAMIN; [M]. 18.; SORIND (complex attribute "sourceIndication - source"); [C]. 22.; VALSOU; [C]. 24.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 25.; WATLEV (1, 2, 3, 4, 5).; [C]. 26.; HORACC; [O]. | 1.; categoryOfObstruction (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23).; [0,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; condition (1, 2, 5).; [0,1]. 4.; defaultClearanceDepth; [0,1]. 5.; expositionOfSounding (1, 2, 3).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; height; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; maximumPermittedDraught; [0,1]. 11.; natureOfConstruction (1, 2, 3, 4, 6, 7, 8).; [0,*]. 12.; natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [0,*]. 13.; pictorialRepresentation; [0,1]. 14.; product (1, 2, 3, 8, 23).; [0,*]. 15.; qualityOfVerticalMeasurement (1, 2, 3, 4, 6, 7, 8, 9).; [0,*]. 16.; reportedDate; [0,1]. 17.; scaleMinimum; [1,1]. 18.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 19.; status (1, 4, 5, 7, 8, 13, 18, 28).; [0,*]. 20.; surroundingDepth; [1,1]. 21.; techniqueOfVerticalMeasurement (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18).; [0,*]. 22.; valueOfSounding; [0,1]. 23.; verticalLength; [0,1]. 24.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 25.; waterLevelEffect (1, 2, 3, 4, 5, 7).; [1,1]. |

Conversion rules

Automatic conversion

- If the S-57 attribute CATOBS = 7, the object class OBSTRN shall be converted into the S-401 feature type foulground.

Manual conversion

General rules:

- 2.1.5.3: Vertical Accuracy (VERACC) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.92 Offshore Platform (OFSPLF)
A permanent offshore structure, either fixed or floating.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATOFP (1, 2, 3, 4, 5, 6, 7, 9).; [M]. 2.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 3.; COLPAT (1, 2, 3, 4, 5, 6).; [O]. 4.; CONDTN (1, 2, 5).; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; HEIGHT; [O]. 9.; INFORM (complex attribute "information" - text); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; PICREP; [O]. 12.; PRODCT (1, 2, 18, 19).; [O]. 13.; CONRAD (attribute "radarConspicuous") (1, 2, 3).; [O]. 14.; SORDAT (attribute "reportedDate"); [C]. 15.; SCAMIN; [M]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; STATUS (2, 4, 7, 12, 16, 17).; [O]. 18.; VERLEN; [O]. 19.; CONVIS (attribute "visualProminence") (1, 2).; [O]. 21.; NATCON (2, 7, 8).; [O]. | 1.; categoryOfOffshorePlatform (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11).; [1,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; condition (1, 2, 5).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; flareStack; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; pictorialRepresentation; [0,1]. 12.; product (1, 2, 3, 18, 19, 23).; [0,*]. 13.; radarConspicuous; [0,1]. 14.; reportedDate; [0,1]. 15.; scaleMinimum; [1,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (1, 2, 4, 7, 8, 12, 16, 17, 28).; [0,*]. 18.; verticalLength; [0,1]. 19.; visualProminence (1, 2, 3).; [0,1]. 20.; waterLevelEffect (1, 2, 3, 4, 5, 6).; [1,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD)

3.93 Offshore Production Area (OSPARE)
An area at sea within which there are production facilities.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATPRA (4, 9).; [M]. 2.; CONDTN (1, 2, 3, 4, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; HEIGHT; [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; NATCON (2, 7, 8).; [O]. 9.; PICREP; [O]. 11.; CONRAD (attribute "radarConspicuous") (1, 2, 3).; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; RESTRN (1, 2, 7, 8, 14).; [M]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; STATUS (2, 4, 7, 12, 16, 17).; [O]. 17.; VERLEN; [O]. 19.; CONVIS (attribute "visualProminence") (1, 2).; [O]. | 1.; categoryOfOffshoreProductionArea (1, 2, 3, 4, 5, 6).; [1,1]. 2.; condition (1, 2, 3, 4, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; height; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; natureOfConstruction (2, 7, 8).; [0,1]. 9.; pictorialRepresentation; [0,1]. 10.; product (1, 2, 4, 6, 10, 14, 23).; [0,*]. 11.; radarConspicuous; [0,1]. 12.; reportedDate; [0,1]. 13.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 39).; [0,*]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 2, 4, 7, 8, 12, 16, 17, 28).; [0,*]. 17.; verticalLength; [0,1]. 18.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. 19.; visualProminence (1, 2, 3).; [0,1]. 20.; waterLevelEffect (2, 3, 4, 7).; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute CATPRA value 9 shall be converted into the S-401 attribute categoryOfOffshoreProductionArea value 1.

Manual conversion

- The S-57 attribute CATPRA value 4 shall be converted into the S-401 attribute categoryOfOffshoreProductionArea value 2, 3 or 6.

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD)

3.94 Oil Barrier (OILBAR)
A floating barrier to stop and contain the spread of oil on a water body surface. For IENCs any construction to dam oil flow on water.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATOLB (1, 2).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfOilBarrier (1, 2).; [0,1]. 2.; condition (1, 2, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 2, 4, 5, 7, 8).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.95 Physical AIS Aid to Navigation ()
An Automatic Identification System (AIS) message 21 transmitted from a physical Aid to Navigation, or transmitted from an AIS station for an Aid to Navigation which physically exists.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point |
| Attributes | Attributes |
|  | 1.; estimatedRangeOfTransmission; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; mMSICode; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 5, 7).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.96 Pile (PILPNT)
A long heavy timber or section of steel, wood, concrete, etc., forced into the earth or seafloor to serve as a support, as for a pier, or to resist lateral pressure; or as a free standing pole within a marine environment.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point curve surface |
| Attributes | Attributes |
| 2.; CATTEV (4, 5, 6).; [O]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [O]. 4.; COLPAT (1, 2, 3, 4, 5, 6).; [C]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [C]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; HEIGHT; [O]. 9.; INFORM (complex attribute "information - text "); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; PICREP; [O]. 13.; SORDAT (attribute "reportedDate"); [C]. 14.; SCAMIN; [M]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; STATUS (1, 4, 6, 7, 8, 12, 14).; [O]. 17.; VERLEN; [O]. 18.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 20.; HORACC; [O]. | 1.; categoryOfPile (1, 3, 4, 5, 6, 7, 8).; [0,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; pictorialRepresentation; [0,1]. 12.; radarConspicuous; [0,1]. 13.; reportedDate; [0,1]. 14.; scaleMinimum; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 4, 6, 7, 8, 12, 14).; [0,*]. 17.; verticalLength; [0,1]. 18.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 19.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.1.5.3: Vertical Accuracy (VERACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.97 Pilotage District ()
An area within which a pilotage direction exists. Such directions are regulated by a competent harbour authority which dictates circumstances under which they apply.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface |
| Attributes | Attributes |
|  | 1.; communicationChannel; [0,*]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.98 Pilot Boarding Place (PILBOP)
A location offshore where a pilot may board a vessel in preparation to piloting it through local waters.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATPIL (1, 2, 3).; [O]. 3.; COMCHA; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information" - text); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; STATUS (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 12, 13, 14, 15, 16, 17, 18).; [C]. 16.; PILDST; [O]. 17.; NPLDST; [O]. | 1.; categoryOfPilotBoardingPlace (1, 2, 3).; [0,1]. 2.; categoryOfPreference (1, 2).; [0,1]. 3.; communicationChannel; [0,*]. 4.; destination; [0,*]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; pilotMovement (1, 2, 3).; [0,*]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (1, 2, 5, 6, 7, 9, 16, 17, 28).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.99 Pipeline Overhead (pipohd)
A string of interconnected pipes, supported by pylons and passing over or nearby navigable waters, used for the transport of matter, nowadays mainly oil or gas.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATPIP (2, 3, 4, 6).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 3.; CONDTN (1, 2, 3, 5).; [O]. 4.; hunits (1, 2, 3, 4, 5, 6).; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information - text "); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; locmrn; [C]. 10.; vcrlev; [O]. 11.; PICREP; [O]. 12.; PRODCT (1, 2, 3, 4, 5, 6, 7, 8, 14, 15, 17, 21, 22).; [M]. 14.; refgag; [C]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 19.; unlocd; [C]. 20.; VERACC (complex attribute "verticalClearanceFixed – verticalUncertainty – uncertaintyFixed"); [O]. 20.; VERCLR (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [M]. 21.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [O]. 22.; vcrval; [O]. 24.; wtwdis; [O]. 25.; HORACC; [O]. | 1.; categoryOfPipelinePipe (2, 3, 4, 6).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 7).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 10.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 11.; pictorialRepresentation; [0,1]. 12.; product (1, 2, 3, 7, 8, 9, 18, 19, 20, 22).; [0,*]. 13.; radarConspicuous; [0,1]. 14.; referenceGauge; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (1, 4, 7, 12).; [0,*]. 19.; uNLocationCode; [0,1]. 20.; verticalClearanceFixed; [1,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 21.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 22.; verticalRiverDatumReferenceLevelValue; [0,1]. 23.; visualProminence (1, 2, 3).; [0,1]. 24.; waterwayDistance; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute VERACC shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalUncertainty sub-attribute uncertaintyFixed. - The S-57 attribute VERCLR shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalClearanceValue.

Manual conversion

General rules:

- 2.1.4: Units (hunits) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.100 Pipeline Submarine/On Land (PIPSOL)
A connected set of pipes for conveying liquids, slurries, or gases.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line | curve |
| Attributes | Attributes |
| 2.; CATPIP (2, 3, 4, 6).; [O]. 3.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 11.; PICREP; [O]. 12.; PRODCT (1, 2, 3, 4, 5, 6, 7, 8, 14, 15, 17, 21, 22).; [O]. 13.; SORDAT (attribute "reportedDate"); [C]. 15.; SCAMIN; [M]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; STATUS (4, 18).; [C]. | 1.; buriedDepth; [0,1]. 2.; categoryOfPipelinePipe (2, 3, 4, 5, 6, 7).; [0,*]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; depthRangeMaximumValue; [0,1]. 5.; depthRangeMinimumValue; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 11.; pictorialRepresentation; [0,1]. 12.; product (1, 2, 3, 7, 8, 9, 18, 19).; [0,*]. 13.; reportedDate; [0,1]. 14.; restriction (1, 3, 4, 5, 8, 9, 11, 12, 13, 14, 16, 17, 18, 20, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43).; [0,*]. 15.; scaleMinimum; [1,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (1, 4, 7, 12, 18).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

- If the S-57 object class PIPSOL is encoded as a point, it shall be: - converted into the S-401 feature type pipelineSubmarineOnLand as a curve; - or if it is on land, into the S-401 feature type landmark with the primitive point; - or if it is in the water into the S-401 feature type obstruction with the primitive point.

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.101 Pontoon (PONTON, ponton)
A floating structure, usually rectangular in shape which serves as landing, pier head, bridge support, etc.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | curve surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (5).; [O]. 16.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 18.; HORACC; [O]. 1.; CATTEV (4, 5, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; locmrn; [C]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (5).; [O]. 14.; unlocd; [C]. 16.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 18.; HORACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; radarConspicuous; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 2, 4, 5, 6, 7, 8, 12, 14).; [0,*]. 14.; uNLocationCode; [0,1]. 15.; verticalLength; [0,1]. 16.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 17.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

- If the S-57 object class PONTON is encoded, it must be ensured that the area beneath the pontoon is filled with the Group 1 object UnsurveyedArea.

Manual conversion

- If the S-57 object class is PONTON, then encode the S-401 attribute scaleMinimum with value 12000 for Europe, 30000 for US.

General rules:

- 2.1.5.3: Vertical Accuracy (VERACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.102 Port Area (prtare)
Apart from harbours a port includes a city or borough with accommodation and facilities for landing passengers and goods and some amount of overseas trade. A port may possess a harbour but a harbour is not necessarily a port.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CONDTN (attribute "inDispute" and condition) (1, 2, 3, 5).; [O]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information" - text); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; locmrn; [C]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; STATUS (5).; [O]. 13.; unlocd; [C]. | 1.; condition (1, 2, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; inDispute; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (5).; [0,1]. 13.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

- The automatic conversion shall change the S-57 attribute CONDTN with value ‘3’ (under reclamation) into the S-401 attribute inDispute as a Boolean with value ‘true’.

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.103 Precautionary Area (PRCARE)
A routeing measure comprising an area within defined limits where ships must navigate with particular caution and within which the direction of traffic flow may be recommended.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point surface |
| Attributes | Attributes |
|  | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; iMOAdopted; [0,1]. 4.; information; [1,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; restriction (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 14, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 40, 41, 43).; [0,*]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 9, 28).; [0,*]. 12.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

3.104 Production/Storage Area (PRDARE)
An area on land for the exploitation or storage of natural resources.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | point surface |
| Attributes | Attributes |
| 1.; CATPRA (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - ame"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information - text "); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; PICREP; [O]. 10.; PRODCT (1, 2, 4, 5, 6, 7, 14, 15, 17, 21, 22).; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; STATUS (2, 12, 16, 17).; [O]. 17.; CONVIS (attribute "visualProminence") (1, 2).; [O]. | 1.; categoryOfProductionArea (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12).; [0,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; elevation; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; height; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; pictorialRepresentation; [0,1]. 10.; product (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 25).; [0,*]. 11.; radarConspicuous; [0,1]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (2, 4, 12, 16, 17).; [0,*]. 16.; verticalLength; [0,1]. 17.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.105 Pylon/Bridge Support (PYLONS)
A vertical construction consisting, for example, of a steel framework or pre-stressed concrete to carry cables, a bridge, etc.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATPYL (1, 2, 3, 4, 5).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; INFORM (complex attribute "information - text "); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 13.; PICREP; [O]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 20.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 22.; WATLEV (1, 2, 3, 4, 5).; [M]. 23.; HORACC; [O]. | 1.; categoryOfPylon (1, 2, 3, 4, 5, 6).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 12.; natureOfConstruction (1, 2, 6, 7, 11).; [0,*]. 13.; pictorialRepresentation; [0,1]. 14.; radarConspicuous; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (4, 12).; [0,*]. 19.; verticalLength; [0,1]. 20.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 21.; visualProminence (1, 2, 3).; [0,1]. 22.; waterLevelEffect (1, 2, 3, 4, 5, 6).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.1.5.3: Vertical Accuracy (VERACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.106 Quality of Bathymetric Data (M_QUAL)
An area within which a uniform assessment of the quality of the bathymetric data exists.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 4.; DRVAL1; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; SUREND; [O]. 11.; SURSTA; [O]. 12.; TECSOU (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15).; [C]. 13.; CATZOC (complex attribute "ZoneOfConfidence - categoryOfZoneOfConfidenceInData ") (1, 2, 3, 4, 5, 6).; [O]. 13.; POSACC (complex attribute "ZoneOfConfidence - horizontalPositionUncertainty"); [O]. 13.; SOUACC (complex attribute "ZoneOfConfidence - verticalUncertainty - uncertaintyFixed"); [O]. | 1.; categoryOfTemporalVariation (1, 2, 3, 4, 5, 6).; [0,1]. 2.; dataAssessment (1, 2, 3).; [0,1]. 3.; depthRangeMaximumValue; [0,1]. 4.; depthRangeMinimumValue; [0,1]. 5.; featuresDetected; [0,1]. leastDepthOfDetectedFeaturesMeasured; [1,1]. significantFeaturesDetected; [1,1]. sizeOfFeaturesDetected; [0,1]. 6.; fullSeafloorCoverageAchieved; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; reportedDate; [0,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; surveyDateRange; [0,1]. dateEnd; [1,1]. dateStart; [0,1]. 12.; techniqueOfVerticalMeasurement (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18).; [0,1]. 13.; zoneOfConfidence; [0,*]. categoryOfZoneOfConfidenceInData (1, 2, 3, 4, 5, 6).; [0,1]. fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. horizontalPositionUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute CATZOC into the S-401 complex attribute ZoneOfConfidence - categoryOfZoneOfConfidenceData if the S-57 attribute CATZOG is used. - The conversion shall copy the value of the S-57 attribute POSACC into the S-401 complex attribute ZoneOfConfidence - horizontalPositionUncertainty if the S-57 attribute POSACC is used. - The conversion shall copy the value of the S-57 attribute SOUACC into the S-401 complex attribute ZoneOfConfidence - verticalUncertainty - uncertaintyFixed if the S-57 attribute SOUACC is used.

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.5: InteroperabilityIdentifier

3.107 Quality of Non-Bathymetric Data (M_ACCY)
An area within which a uniform assessment of the quality of the non-bathymetric data exists.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [M]. 2.; HORACC; [O]. 3.; POSACC; [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; SOUACC; [O]. 10.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. | 1.; categoryOfTemporalVariation (1, 4, 5, 6).; [0,1]. 2.; horizontalDistanceUncertainty; [0,1]. 3.; horizontalPositionUncertainty; [1,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; orientationUncertainty; [0,1]. 7.; reportedDate; [0,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; surveyDateRange; [0,1]. dateEnd; [1,1]. dateStart; [0,1]. 10.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute VERACC into the S-401 complex attribute verticalUncertainty - uncertaintyFixed if the S-57 attribute VERACC is used, - The conversion shall copy the value of the S-57 attribute POSACC into the S-401 complex attribute horizontalPositionUncertainty - uncertaintyFixed if the S-57 attribute POSACC is used,

Manual conversion

General rules:

- 2.1.5.2: Horizontal Accuracy (HORACC) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.108 Quality of Survey (M_SREL)
An area within which a uniform assessment of the reliability of source survey information exists.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | curve surface |
| Attributes | Attributes |
| 11.; QUAPOS (4, 10).; [C]. 12.; QUASOU (1, 2, 8, 10, 11).; [M]. 13.; SORDAT (attribute "reportedDate"); [C]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; SURATH; [M]. 18.; SUREND; [M]. 18.; SURSTA; [M]. 19.; SURTYP (2).; [C]. | 1.; depthRangeMaximumValue; [0,1]. 2.; depthRangeMinimumValue; [0,1]. 3.; featuresDetected; [0,1]. leastDepthOfDetectedFeaturesMeasured; [1,1]. significantFeaturesDetected; [1,1]. sizeOfFeaturesDetected; [0,1]. 4.; fullSeafloorCoverageAchieved; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; lineSpacingMaximum; [0,1]. 8.; lineSpacingMinimum; [0,1]. 9.; measurementDistanceMaximum; [0,1]. 10.; measurementDistanceMinimum; [0,1]. 11.; qualityOfHorizontalMeasurement (4).; [0,1]. 12.; qualityOfVerticalMeasurement (1, 2, 3, 4, 6, 7, 8, 9, 10, 11).; [1,*]. 13.; reportedDate; [0,1]. 14.; scaleValueMaximum; [0,1]. 15.; scaleValueMinimum; [0,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; surveyAuthority; [1,1]. 18.; surveyDateRange; [1,1]. dateEnd; [1,1]. dateStart; [0,1]. 19.; surveyType (1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 20.; techniqueOfVerticalMeasurement (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.5: InteroperabilityIdentifier

3.109 Radar Line (RADLNE)
Recommended tracks along which ships can be guided by coastal radar stations in the event of bad visibility.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [O]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; ORIENT; [M]. 5.; PICREP; [O]. 6.; SORDAT (attribute "reportedDate"); [C]. 7.; SCAMIN; [M]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; orientationValue; [1,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; status (1, 2, 3, 4, 7).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.110 Radar Range (RADRNG)
Indicates the coverage of a sea area by a radar surveillance station. Inside this area a vessel may request shore-based radar assistance, particularly in poor visibility.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface |
| Attributes | Attributes |
|  | 1.; communicationChannel; [0,*]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 2, 3, 4, 7).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

3.111 Radar Reflector (RADRFL)
A device capable of, or intended for, reflecting radar signals.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point |
| Attributes | Attributes |
|  | 1.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 2.; height; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 4, 8).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.112 Radar Station (RADSTA)
A station with a transmitter emitting pulses of ultra-high frequency radio waves which are reflected by solid objects and are detected upon their return to the sending station. For IENCs the radar station of a VTS or a lock to locate vessels and/ or monitor the traffic.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 2.; CATRAS (1).; [M]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (5).; [O]. 15.; DATEND; [O]. 16.; DATSTA; [O]. | 1.; callSign; [0,1]. 2.; categoryOfRadarStation (1, 2).; [0,*]. 3.; communicationChannel; [0,*]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; height; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 2, 4, 5, 7, 8).; [0,*]. 14.; valueOfMaximumRange; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.113 Radar Transponder Beacon (RTPBCN)
A transponder beacon transmitting a coded signal on radar frequency, permitting an interrogating craft to determine the bearing and range of the transponder. For IENCs a radar transponder beacon (racon) may be used to indicate an entrance of a canal or a bridge passage.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; CATRTB (1, 2, 3).; [M]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; RADWAL (complex attribute "radarWaveLength - waveLengthValue"); [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 13.; SIGGRP; [O]. 15.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; STATUS (5).; [O]. | 1.; categoryOfRadarTransponderBeacon (1, 2, 3).; [1,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; radarWaveLength; [0,2]. radarBand; [1,1]. waveLengthValue; [1,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sectorLimit; [0,1]. sectorLimitOne; [1,1]. sectorBearing; [1,1]. sectorLineLength; [0,1]. sectorLimitTwo; [1,1]. sectorBearing; [1,1]. sectorLineLength; [0,1]. 13.; signalGroup; [0,1]. 14.; signalSequence; [0,*]. signalDuration; [1,1]. signalStatus (1, 2).; [1,1]. 15.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 16.; status (1, 2, 4, 5, 7, 8).; [0,*]. 17.; valueOfMaximumRange; [0,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute RADWAL into the S-401 complex attribute radarWaveLength - waveLengthValue if the S-57 attribute RADWAL is used,

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.114 Radio Calling-In Point (RDOCAL)
A designated position at which vessels are required to report to a traffic control centre. Also called reporting point or radio reporting point.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line | point curve |
| Attributes | Attributes |
| 1.; catcom (1, 2, 3, 4, 5, 6, 7, 8).; [M]. 2.; COMCHA; [M]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [C]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; locmrn; [C]. 7.; ORIENT; [M]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (5).; [O]. 14.; TRAFIC (1, 2, 3, 4).; [M]. 15.; unlocd; [C]. | 1.; categoryOfCommunication (1, 2, 3, 4, 5, 6, 7, 8).; [1,*]. 2.; communicationChannel; [1,*]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; orientationValue; [1,2]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 3, 4, 5, 6, 7).; [0,*]. 14.; trafficFlow (1, 2, 3, 4).; [1,1]. 15.; uNLocationCode; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.115 Radio Station (RDOSTA)
A place equipped to transmit radio waves. Such a station may be either stationary or mobile, and may also be provided with a radio receiver.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point |
| Attributes | Attributes |
|  | 1.; callSign; [0,1]. 2.; categoryOfRadioStation (5, 10, 11, 14, 19, 20).; [0,*]. 3.; communicationChannel; [0,*]. 4.; estimatedRangeOfTransmission; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; frequencyPair; [0,1]. frequencyShoreStationReceives; [0,1]. frequencyShoreStationTransmits; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 11.; pictorialRepresentation; [0,1]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (1, 2, 4, 5, 7, 8).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.116 Railway (RAILWY)
A rail or set of parallel rails on which a train, tram, or rail wagon runs.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CONDTN (1, 2, 3, 5).; [O]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PICREP; [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 8.; SCAMIN; [M]. 9.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; condition (1, 2, 3, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; height; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 4, 6, 12, 13, 14).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.117 Recommended Route Centreline (RCRTCL)
The recommended route centreline indicates the 'centreline' of a recommended route.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | curve |
| Attributes | Attributes |
|  | 1.; basedOnFixedMarks; [1,1]. 2.; depthRangeMinimumValue; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; orientationValue; [0,1]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; qualityOfVerticalMeasurement (1, 2, 3, 4, 6).; [0,*]. 11.; reportedDate; [0,1]. 12.; scaleMinimum; [1,1]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; status (1, 5, 6, 9).; [0,*]. 15.; techniqueOfVerticalMeasurement (1, 3, 6, 8, 9, 13, 15, 16, 17, 18).; [0,*]. 16.; trafficFlow (1, 2, 3, 4).; [0,1]. 17.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.118 Recommended Track (RECTRC)
A route which has been specially examined to ensure so far as possible that it is free of dangers and along which ships are advised to navigate. For IENCs it is a recommended sailing route for all or certain vessels.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATTRK (1, 2).; [M]. 2.; DRVAL1; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; ORIENT; [M]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; STATUS (5).; [O]. 17.; TRAFIC (1, 2, 3, 4).; [M]. 19.; DRVAL2; [O]. | 1.; basedOnFixedMarks; [1,1]. 2.; depthRangeMinimumValue; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; maximumPermittedDraught; [0,1]. 8.; orientationValue; [1,1]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; qualityOfVerticalMeasurement (1, 2, 6).; [0,*]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (1, 2, 5, 6, 8, 9, 14).; [0,*]. 16.; techniqueOfVerticalMeasurement (1, 2, 3, 6, 8, 9, 13, 15, 16, 17, 18).; [0,*]. 17.; trafficFlow (1, 2, 3, 4).; [1,1]. 18.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute CATTRK value 1 shall be converted into the S-401 attribute basedOnFixedMarks value true. - The S-57 attribute CATTRK value 2 shall be converted into the S-401 attribute basedOnFixedMarks value false.

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.119 Recommended Traffic Lane Part (RCTLPT)
A traffic flow pattern indicating a recommended directional movement of traffic where it is impractical or unnecessary to adopt an established direction of traffic flow.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 1.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; ORIENT; [M]. 5.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 5.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 6.; PICREP; [O]. 8.; SCAMIN; [M]. 7.; SORDAT (attribute "reportedDate"); [C]. 9.; SORIND (complex attribute "sourceIndication - source"); [C]. 10.; STATUS (5).; [O]. 11.; OBJNAM; [O]. 12.; NOBJNM; [O]. | 1.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; orientationValue; [1,1]. 5.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 5, 6, 9).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.120 Refuse Dump (refdmp)
At a refuse dump ships are able to unload their refuse like waste oil or black water.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; catrfd (1, 2, 3, 4).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information" - text); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; locmrn; [C]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; STATUS (5).; [O]. 13.; unlocd; [C]. | 1.; categoryOfRefuseDump (1, 2, 3, 4).; [0,*]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (5).; [0,1]. 13.; uNLocationCode; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.121 Rescue Station (RSCSTA, Life Saving Station)
A place where equipment for saving life at sea is maintained.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point surface |
| Attributes | Attributes |
| 1.; catrsc (1, 2, 4, 5, 6, 7, 8, 9).; [M]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; STATUS (2, 4, 5).; [O]. | 1.; categoryOfRescueStation (1, 2, 4, 5, 6, 7, 8, 9).; [1,*]. 2.; communicationChannel; [0,*]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 2, 4, 5, 7, 8, 14, 16, 17).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.122 Restricted Area (RESARE, resare)
A specified area designated by an appropriate authority within which navigation is restricted in accordance with certain specified conditions. For IENCs restricted areas typically surround dams; see 8.12 Dams.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; CATREA (1, 12).; [C]. 2.; CATTEV (4, 5, 6).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; RESTRN (1, 7, 8, 38).; [M]. 12.; SCAMIN; [M]. 13.; SORIND (complex attribute "sourceIndication - source"); [C]. 14.; STATUS (5).; [O]. 16.; HORACC; [O]. 17.; VERACC; [O]. 1.; CATREA (1, 4, 5, 6, 7, 8, 9, 10, 12, 14, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [C]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [C]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; NATSUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [O]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; restrn (1, 2, 7, 8, 13, 14, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 40, 41).; [M]. 12.; SCAMIN; [M]. 13.; SORIND (complex attribute "sourceIndication - source"); [C]. 14.; STATUS (5).; [O]. | 1.; categoryOfRestrictedArea (1, 4, 5, 6, 7, 8, 9, 10, 12, 14, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33).; [0,*]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [0,1]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43).; [1,*]. 12.; scaleMinimum; [1,1]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; status (1, 2, 3, 4, 5, 6, 7, 9, 18, 28).; [0,*]. 15.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.123 River (RIVERS)
A relatively large natural stream of water. (IHO Dictionary - S-32) This feature is used for non-navigable rivers.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [O]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - v"); [O]. 4.; PICREP; [O]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SCAMIN; [M]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 8.; status (5).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.124 Road (ROADWY)
A route with a specially prepared surface that is intended for use by wheeled vehicles or pedestrians.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 1.; CATROD (1, 2, 3, 4).; [M]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information" - text); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; NATCON (4, 5).; [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfRoad (1, 2, 3, 4, 5, 6).; [1,1]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; natureOfConstruction (4, 5).; [0,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 4, 6, 7, 8, 12, 13, 14).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.125 Runway (RUNWAY)
A defined area, on a land aerodrome, prepared for the landing and take-off run of aircraft.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | curve surface |
| Attributes | Attributes |
| 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information - text "); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; NATCON (4, 5).; [O]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; STATUS (5).; [O]. 12.; CONVIS (attribute "visualProminence") (1, 2).; [O]. 13.; CATRUN (1, 2).; [O]. | 1.; condition (1, 2, 3, 5).; [0,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; natureOfConstruction (1, 2, 4, 5, 6, 7).; [0,*]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 2, 4, 5, 6, 7, 8, 12, 14).; [0,*]. 12.; visualProminence (1, 2).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

- If the S-57 RUNWAY is encoded as a point, the curve geometry has to be encoded manually.

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.126 Safe Water Beacon (BCNSAW)
A safe water beacon is used to indicate that there is navigable water around the mark.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BCNSHP (1, 2, 3, 4, 5, 6, 7).; [M]. 2.; COLOUR (1, 3).; [M]. 3.; COLPAT (2).; [M]. 4.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute “featureName – name”); [O]. 6.; NOBJNM (complex attribute “featureName – name”); [O]. 7.; DATEND (complex attribute “fixedDateRange – dateEnd”); [O]. 7.; DATSTA (complex attribute “fixedDateRange – dateStart”); [O]. 8.; HEIGHT; [O]. 9.; INFORM (complex attribute “information – text “); [O]. 9.; NINFOM (complex attribute “information – text”); [O]. 9.; TXTDSC (complex attribute “information – file reference”); [O]. 9.; NTXTDS (complex attribute « information – file reference »); [O]. 11.; MARSYS (1, 2).; [C]. 13.; PEREND (complex attribute “periodicDateRange – dateEnd”); [O]. 13.; PERSTA (complex attribute “periodicDateRange – dateStart”); [O]. 14.; PICREP; [O]. 15.; CONRAD (attribute “radarConspicuous”) (3).; [O]. 16.; SORDAT (attribute ” “reportedDate”); [C]. 17.; SCAMIN; [M]. 18.; SORIND (complex attribute “sourceIndication – source”); [C]. 19.; STATUS (2, 3, 4, 5, 8, 9, 12, 14, 16, 17, 18).; [O]. 21.; VERLEN; [O]. 20. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation text"); [O]. NINFOM (complex attribute "topmark - shapeInformation text"); [O]. SCAMIN; [M]. SORDAT (attribute "sourceDate" or "reportedDate"); [C]. SORIND (attribute "sourceIndication; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; beaconShape (1, 2, 3, 4, 5, 6, 7).; [1,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [2,1]. 4.; condition (1, 2, 5).; [0,1]. 5.; elevation; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 12.; natureOfConstruction (1, 2, 6, 7, 8).; [0,*]. 13.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 14.; pictorialRepresentation; [0,1]. 15.; radarConspicuous; [0,1]. 16.; reportedDate; [0,1]. 17.; scaleMinimum; [1,1]. 18.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 19.; status (1, 2, 4, 5, 7, 8, 12, 18).; [0,*]. 20.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 21.; verticalLength; [0,1]. 22.; visualProminence (1, 2, 3).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR) |  |

3.127 Safe Water Buoy (BOYSAW)
A safe water buoy is used to indicate that there is navigable water around the mark. For IENCs a safe water buoy marking the axis or middle of a channel may be used as a centerline, mid-channel on lakes and broad waterways. The swinging axial buoys are used to mark swinging points of the fairway axis.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BOYSHP (1, 2, 3, 4, 5, 6, 8).; [M]. 2.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 3.; COLPAT (1, 2, 3, 4, 5, 6).; [M]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; mmsico; [O]. 8.; MARSYS (1, 2).; [C]. 11.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 11.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 12.; PICREP; [O]. 13.; CONRAD (attribute "radarConspicuous") (3).; [O]. 14.; SORDAT (attribute "reportedDate"); [C]. 15.; SCAMIN; [M]. 16.; SORIND (complex attribute "sourceIndication - source"); [C]. 17.; STATUS (5).; [O]. 18. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation text"); [O]. NINFOM (complex attribute "topmark - shapeInformation text"); [O]. SCAMIN; [M]. SORDAT (attribute "sourceDate" or "reportedDate"); [C]. SORIND (attribute "sourceIndication; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; buoyShape (1, 2, 3, 4, 5, 6, 7, 8).; [1,1]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 9.; mMSICode; [0,1]. 10.; natureOfConstruction (6, 7, 8, 11).; [0,*]. 11.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 12.; pictorialRepresentation; [0,1]. 13.; radarConspicuous; [0,1]. 14.; reportedDate; [0,1]. 15.; scaleMinimum; [1,1]. 16.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 17.; status (1, 2, 5, 7, 8, 18).; [0,*]. 18.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 19.; verticalLength; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR) - 2.8: Lights (LIGHTS) |  |

3.128 Sandwave (SNDWAV)
A large mobile wave-like sediment feature in shallow water and composed of sand. The wavelength may reach 100 metres, the amplitude may be up to 20 metres.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; INFORM (complex attribute "information - text "); [O]. 1.; NINFOM (complex attribute "information - text"); [O]. 1.; TXTDSC (complex attribute "information - file reference"); [O]. 1.; NTXTDS (complex attribute "information - file reference "); [O]. 3.; PICREP; [O]. 4.; SORDAT (attribute "reportedDate"); [O]. 5.; SCAMIN; [M]. 6.; SORIND (complex attribute "sourceIndication - source"); [O]. 7.; VERLEN; [O]. 8.; OBJNAM; [O]. 9.; NOBJNM; [O]. | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; pictorialRepresentation; [0,1]. 4.; reportedDate; [0,1]. 5.; scaleMinimum; [1,1]. 6.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 7.; verticalLength; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.129 Sea Area/Named Water Area (SEAARE)
A geographically defined part of the sea or other navigable waters. It may be specified within its limits by its proper name.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATSEA (5, 12, 13, 51, 52, 53, 54, 57, 58, 59).; [O]. 2.; CATTEV (4, 5, 6).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [M]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information - text "); [C]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [C]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PICREP; [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 8.; SCAMIN; [M]. 9.; SORIND (complex attribute "sourceIndication - source"); [C]. 10.; CONDTN (1, 2, 3, 5).; [C]. 11.; HORACC; [O]. 12.; VERACC; [O]. | 1.; categoryOfSeaArea (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 59).; [0,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; featureName; [1,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.130 Seabed Area (SBDARE)
A region of the seabed including the material of which it is composed and its physical characteristics. Also called nature of bottom, character (or characteristics) of the bottom, or quality of the bottom.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [O]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; PICREP; [O]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SCAMIN; [M]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. 8.; NATSUR (complex attribute "surfaceCharacteristics - natureOfSurface") (1, 2, 3, 4, 5, 6, 7, 8, 9, 18).; [C]. 8.; NATQUA (complex attribute " surfaceCharacteristics - natureOfSurfaceQualifyingTerms ") (1, 2, 3, 4).; [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 8.; surfaceCharacteristics; [1,*]. natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [0,1]. natureOfSurfaceQualifyingTerms (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [0,3]. underlyingLayer; [0,1]. 9.; waterLevelEffect (3, 4, 5).; [0,1]. |

Conversion rules

Automatic conversion

- The conversion shall copy the value of the S-57 attribute NATSUR into the S-401 complex attribute surfaceCharacteristics - natureOfSurface if the S-57 attribute NATUSR is used, - The conversion shall copy the value of the S-57 attribute NATQUA into the S-401 complex attribute surfaceCharacteristics - natureOfSurfaceQualifyingTerms if the S-57 attribute NATQUA is used,

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.131 Seagrass (WEDKLP)
Any of various submerged monocotyledonous plants (such as eelgrass, tape grass, and turtle grass) of tropical to temperate usually shallow coastal waters that have narrow grass-like leaves and often form dense underwater meadows.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [O]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; INFORM (complex attribute "information" - text); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; PICREP; [O]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SCAMIN; [M]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. 8.; CATWED (1, 2, 3, 4).; [M]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.132 Seaplane Landing Area (SPLARE)
A designated portion of water for the landing and take-off of seaplanes.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point surface |
| Attributes | Attributes |
|  | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43).; [0,*]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 2, 3, 4, 5, 6, 7, 8, 9, 14).; [0,*]. 11.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.133 Sensor (sensor)
A device that responds to a physical stimulus (as heat, light, sound, pressure, magnetism or a particular motion) and transmits a resulting impulse (as for measurement or operating a control).

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; catsen (1, 2, 3).; [M]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; fnctsn (1).; [M]. 5.; INFORM (complex attribute "information" - text); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; STATUS (5).; [O]. 12.; OBJNAM; [O]. 13.; NOBJNM; [O]. | 1.; categoryOfSensor (1, 2, 3).; [1,*]. 2.; communicationChannel; [0,*]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; functionOfSensor (1).; [1,*]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (5).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.134 Separation Zone or Line ()
A zone or line separating the traffic lanes in which ships are proceeding in opposite, or nearly opposite directions; or separating a traffic lane from the adjacent sea area; or separating traffic lanes designated for particular classes of ships proceeding in the same direction.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | curve surface |
| Attributes | Attributes |
|  | 1.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 8.; status (1, 3, 9, 28).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.135 Shoreline Construction (SLCONS, slcons)
A fixed artificial structure in the water and/or adjoining the land. It may also refer to features such as training walls, which are not necessarily connected to, nor form part of the shoreline. For IENCs Shoreline Construction includes groins, ground sills, revetments, training walls, boat ramps, slipways, docks, wharfs, lock walls, ice breakers, water intake structures, fenders, landing steps and ladders.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point line area | point curve surface |
| Attributes | Attributes |
| 1.; CATSLC (1, 2, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [C]. 6.; OBJNAM (complex attribute "featureName - name"); [C]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; HORACC (complex attribute "horizontalClearanceFixed - horizontalCleanranceUncertainty"); [O]. 12.; INFORM (complex attribute "information - text "); [O]. 12.; NINFOM (complex attribute "information - text"); [O]. 12.; TXTDSC (complex attribute "information - file reference"); [O]. 12.; NTXTDS (complex attribute "information - file reference "); [O]. 13.; locmrn; [C]. 14.; NATCON (1, 2, 3, 4, 5, 6, 7, 8, 9).; [C]. 15.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. 20.; STATUS (2, 3, 4, 8, 9, 12, 14, 16, 17).; [C]. 21.; unlocd; [C]. 23.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 25.; WATLEV (1, 2, 3, 4, 5).; [O]. 1.; catslc (2, 7, 8, 9, 18, 19, 20).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 3.; CONDTN (1, 2, 3, 5).; [O]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; HORACC (complex attribute "horizontalClearanceFixed - horizontalCleanranceUncertainty"); [O]. 12.; INFORM (complex attribute "information" - text); [O]. 12.; NINFOM (complex attribute "information - text"); [O]. 12.; TXTDSC (complex attribute "information - file reference"); [O]. 12.; NTXTDS (complex attribute "information - file reference "); [O]. 14.; NATCON (1, 2, 3, 4, 5, 6, 7, 8, 9).; [O]. 15.; PICREP; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. 23.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 25.; watlev (1, 2, 3, 4, 8, 9).; [C]. | 1.; categoryOfShorelineConstruction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; height; [0,1]. 9.; horizontalClearanceFixed; [0,1]. horizontalClearanceValue; [1,1]. horizontalDistanceUncertainty; [0,1]. 10.; horizontalLength; [0,1]. 11.; horizontalWidth; [0,1]. 12.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 13.; interoperabilityIdentifier; [0,1]. 14.; natureOfConstruction (1, 2, 3, 4, 5, 6, 7, 8, 9, 11).; [0,*]. 15.; pictorialRepresentation; [0,1]. 16.; radarConspicuous; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; status (1, 2, 3, 4, 6, 7, 8, 12, 13, 14, 28).; [0,*]. 21.; uNLocationCode; [0,1]. 22.; verticalLength; [0,1]. 23.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 24.; visualProminence (1, 2, 3).; [0,1]. 25.; waterLevelEffect (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.1.5.2: Horizontal Accuracy (HORACC) - 2.1.5.3: Vertical Accuracy (VERACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.136 Signal Station Traffic (SISTAT)
A traffic signal station is a place on shore from which signals are made to regulate the movement of traffic. For IENCs this includes: - Bridge passage signal station: a place on shore from which signals are made for the control of vessels wishing to pass under a bridge. - Lock signal station: a place on shore from which signals are made for the control of vessels entering or leaving a lock. - Oncoming traffic indicator: a place on shore from which signals are made to inform about oncoming traffic. - Port entry and departure: a place on shore from which signals are made for the control of vessels entering or leaving a port.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point surface |
| Attributes | Attributes |
| 1.; catsit (2, 6, 8, 10).; [M]. 3.; CONDTN (1, 2, 3, 5).; [O]. 4.; dirimp (1, 2, 3, 4).; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information - text "); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; locmrn; [C]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 12.; SCAMIN; [M]. 13.; SORIND (complex attribute "sourceIndication - source"); [C]. 14.; STATUS (5).; [O]. 15.; unlocd; [C]. | 1.; categoryOfSignalStationTraffic (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [1,*]. 2.; communicationChannel; [0,*]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; directionOfImpact (1, 2, 3, 4).; [0,*]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; reportedDate; [0,1]. 12.; scaleMinimum; [1,1]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; status (1, 2, 4, 5, 7, 8, 12, 14, 15, 16, 17).; [0,*]. 15.; uNLocationCode; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.137 Signal Station Warning (sistaw)
A warning signal station is a place on shore from which warning signals are made to ships at sea. For IENCs this includes: - Depth Indicator: Device that shows the real water depth between the actual water level and the bottom of the waterway or isolated dangers under water (e.g., ground sill). - The manner in which the device indicates this can either be analog (e.g., by a water level staff / pole - one can read the real water depth directly at the water level) or digital (e.g. by a display). - External indicator of a gauge, also if the indicator is not directly located at the gauge – this is not the same as a depth indicator (values at gauges are always referred to the zero point of the gauge). - High Water Mark: Device that shows if official high water levels are reached. This can be indicated either by analogue (e.g., by signs like a staff gauge) or digital (e.g., by a display).- A signal station showing a warning if an outlet of water (e.g. from a pumping station) causes a current.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point surface |
| Attributes | Attributes |
| 1.; catsiw (15, 16, 18).; [M]. 3.; CONDTN (1, 2, 3, 5).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [C]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (5).; [O]. | 1.; categoryOfSignalStationWarning (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 18).; [1,*]. 2.; communicationChannel; [0,*]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 2, 4, 5, 7, 8, 12, 14, 15, 16, 17).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.138 Silo/Tank (SILTNK)
A large storage structure used for storing loose materials, liquids and/or gases.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 2.; CATSIL (1, 2, 3, 4).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 7.; OBJNAM (complex attribute "featureName - name"); [O]. 7.; NOBJNM (complex attribute "featureName - name"); [O]. 9.; INFORM (complex attribute "information - text "); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 14.; PICREP; [O]. 15.; PRODCT (1, 2, 3, 4, 5, 6, 7, 8, 14, 15, 17, 21, 22).; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; buildingShape (5, 6, 7, 8, 9).; [0,1]. 2.; categoryOfSiloTank (1, 2, 3, 4).; [0,1]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 3, 5).; [0,1]. 6.; elevation; [0,1]. 7.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 8.; height; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; inTheWater; [0,1]. 12.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 13.; natureOfConstruction (1, 2, 6, 7, 8).; [0,*]. 14.; pictorialRepresentation; [0,1]. 15.; product (1, 2, 3, 5, 7, 8, 9, 13, 14, 16, 18, 19, 20, 21, 22, 24).; [0,*]. 16.; radarConspicuous; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; status (4, 12, 13).; [0,*]. 21.; verticalLength; [0,1]. 22.; visualProminence (1, 2, 3).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.139 Slope Topline (SLOTOP)
The upper marking of a slope, for example the ridge line or the separation line between two different gradients. (S-57 Edition 3.1, Appendix A - Chapter 1, Page 1.160, November 2000).

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATSLO (1, 2, 3, 6).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 4.; CONDTN (1, 2, 3, 5).; [C]. 6.; OBJNAM (complex attribute "featureName - name"); [O]. 6.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; INFORM (complex attribute "information - text "); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; NATSUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [O]. 10.; PICREP; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 17.; HORACC; [O]. | 1.; categoryOfSlope (1, 2, 3, 6).; [1,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; colour (1, 2, 3, 4, 6, 7, 8, 11, 13).; [0,*]. 4.; condition (1, 2, 3, 5).; [0,1]. 5.; elevation; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [0,*]. 10.; pictorialRepresentation; [0,1]. 11.; radarConspicuous; [0,1]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 16.; visualProminence (1, 2, 3).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.1.5.3: Vertical Accuracy (VERACC) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.140 Sloping Ground (SLOGRD)
An inclined surface. (S-57 Edition 3.1, Appendix A - Chapter 1, Page 1.161, November 2000).

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | point surface |
| Attributes | Attributes |
| 1.; CATSLO (1, 2, 3, 6).; [M]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; NATSUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [O]. 7.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfSlope (1, 2, 3, 4, 5, 6, 7).; [1,1]. 2.; colour (1, 2, 3, 4, 6, 7, 8, 11, 13).; [0,*]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; natureOfSurface (1, 2, 3, 4, 5, 6, 7, 8, 9, 11, 14, 17, 18).; [0,*]. 7.; pictorialRepresentation; [0,1]. 8.; radarConspicuous; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; visualProminence (1, 2, 3).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

- If the S-57 object class SLOGRD has been enocded as a line, it shall be converted into a surface.

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.141 Small Craft Facility (SMCFAC)
A place at which a service generally of interest to small craft or pleasure boats is available.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATSCF (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33).; [M]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; STATUS (5).; [O]. | 1.; categoryOfSmallCraftFacility (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33).; [1,*]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (1, 2, 3, 4, 5, 6, 7, 8, 9, 12, 14, 16, 17).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.142 Sounding (SOUNDG)
Measured or charted depth of water (may be a drying height), or the measurement of such a depth, which has been reduced to a vertical datum.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | pointSet |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [O]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SCAMIN; [M]. 7.; SOUACC; [O]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. 10.; TECSOU (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15).; [O]. 11.; PICREP; [O]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [1,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; qualityOfVerticalMeasurement (1, 3, 4, 8, 9).; [0,*]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; soundingAccuracy; [0,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; status (18).; [0,1]. 10.; techniqueOfVerticalMeasurement (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18).; [0,*]. |

Conversion rules

Automatic conversion

Manual conversion

- The S-57 object class SOUDNG can be converted into a pointSet with at least one point.

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.143 Sounding Datum (m_sdat)
The horizontal plane or tidal datum to which soundings have been reduced. Also called datum for sounding reduction. (Adapted from IHO Dictionary - S-32). The Sounding Datum for inland waterways is in most cases not a horizontal plane but a sloping plane.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 3.; SORDAT (attribute "reportedDate"); [C]. 4.; SORIND (complex attribute "sourceIndication - source"); [C]. 5.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [M]. | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; reportedDate; [0,1]. 4.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 5.; verticalDatum (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 19, 22, 23, 24, 25, 26, 27, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [1,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.5: InteroperabilityIdentifier |  |

3.144 Span Fixed (bridge, CATBRG=1, 9, 11, 12, 13)
A fixed component of the deck of a bridge spanning successive bridge piers.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | curve surface |
| Attributes | Attributes |
| 1.; CATBRG (1, 3, 4, 5, 7, 9, 10, 11, 12, 13).; [M]. 2.; CATTEV (4, 5, 6).; [O]. 3.; hunits (1, 2, 3, 4, 5, 6).; [C]. 4.; elevwl; [O]. 5.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 5.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information" - text); [C]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 8.; vcrlev; [O]. 9.; PICREP; [O]. 10.; refgag; [C]. 11.; reflev (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication – source"); [C]. 15.; unlocd; [C]. 16.; VERACC (complex attribute "verticalClearanceFixed – verticalUncertainty – uncertaintyFixed"); [O]. 16.; VERCLR (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [C]. 16.; VERCCL (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [C]. 17.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [C]. 18.; vcrval; [O]. 19.; wtwdis; [C]. 20.; CONDTN (1, 2, 3, 5).; [C]. 21.; HORACC; [O]. 22.; HORCLR; [O]. 23.; OBJNAM; [O]. 24.; NOBJNM; [O]. 25.; PEREND; [O]. 26.; PERSTA; [O]. 27.; STATUS (5).; [O]. 28.; VERCOP; [C]. | 1.; bridgeConstruction (1, 2, 3, 4, 5).; [0,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 6, 7).; [0,1]. 4.; elevationOfWaterLevel; [0,1]. 5.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 9.; pictorialRepresentation; [0,1]. 10.; referenceGauge; [0,1]. 11.; referenceGravitationalLevel (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [0,1]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; uNLocationCode; [0,1]. 16.; verticalClearanceFixed; [1,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 17.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 18.; verticalRiverDatumReferenceLevelValue; [0,1]. 19.; waterwayDistance; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute VERACC shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalUncertainty sub-attribute uncertaintyFixed. - The S-57 attribute VERCLR shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalClearanceValue. - The S-57 attribute CATBRG value 12 shall be converted into the S-401 attribute BridgeConstruction value 4 - The S-57 attribute CATBRG value 13 shall be converted into the S-401 attribute BridgeConstruction value 1 - If the S-57 attribute CATBRG value is 3, 4, 5 and/or 7, the bridge is not converted into a SpanFixed. - If an existing bridge feature is consisting multiple openings, it has to be split up in individual spans.

Manual conversion

General rules:

- 2.1.4: Units (hunits) - 2.1.5.2: Horizontal Accuracy (HORACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.145 Span Opening (bridge, CATBRG=3, 4, 5, 7, 9)
An opening component of the deck of a bridge spanning successive bridge piers.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | curve surface |
| Attributes | Attributes |
| 1.; CATBRG (1, 3, 4, 5, 7, 9, 10, 11, 12, 13).; [M]. 3.; CATTEV (4, 5, 6).; [O]. 4.; CONDTN (1, 2, 3, 5).; [C]. 5.; hunits (1, 2, 3, 4, 5, 6).; [C]. 6.; elevwl; [O]. 7.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 7.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 8.; HORACC (complex attribute "horizontalClearanceFixed - horizontalDistanceUncertainty"); [O]. 8.; HORCLR (complex attribute "horizontalClearanceFixed - horizontalClearanceValue"); [O]. 9.; INFORM (complex attribute "information" - text); [C]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 10.; locmrn; [C]. 11.; vcrlev; [O]. 12.; PICREP; [O]. 13.; refgag; [C]. 14.; reflev (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [O]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 18.; unlocd; [C]. 19.; VERACC (complex attribute "verticalClearanceClosed – verticalUncertainty – uncertaintyFixed"); [O]. 19.; VERCLR (complex attribute "verticalClearanceClosed – verticalClearanceValue"); [C]. 19.; VERCCL (complex attribute "verticalClearanceClosed – verticalClearanceValue"); [C]. 20.; VERCOP (complex attribute "verticalClearanceOpen – verticalClearanceValue"); [C]. 21.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [C]. 22.; vcrval; [O]. 23.; wtwdis; [C]. 24.; OBJNAM; [O]. 25.; NOBJNM; [O]. 26.; PEREND; [O]. 27.; PERSTA; [O]. 28.; STATUS (5).; [O]. | 1.; bridgeConstruction (1, 2, 3, 4, 5).; [0,1]. 2.; categoryOfOpeningBridge (3, 4, 5, 7).; [0,1]. 3.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 4.; condition (1, 2, 3, 5).; [0,1]. 5.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 6, 7).; [0,1]. 6.; elevationOfWaterLevel; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; horizontalClearanceFixed; [0,1]. horizontalClearanceValue; [1,1]. horizontalDistanceUncertainty; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 12.; pictorialRepresentation; [0,1]. 13.; referenceGauge; [0,1]. 14.; referenceGravitationalLevel (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; uNLocationCode; [0,1]. 19.; verticalClearanceClosed; [1,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 20.; verticalClearanceOpen; [1,1]. verticalClearanceUnlimited; [1,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 21.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 22.; verticalRiverDatumReferenceLevelValue; [0,1]. 23.; waterwayDistance; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute VERACC shall be converted into the S-401 complex attribute verticalClearanceClosed sub-attribute verticalUncertainty sub-attribute uncertaintyFixed. - The S-57 attribute VERCLR shall be converted into the S-401 complex attribute verticalClearanceClosed sub-attribute verticalClearanceValue. - If the S-57 attribute CATBRG value is 3, 4, 5 and/or 7, the bridge is converted into a SpanOpening. - If an existing bridge feature is consisting multiple openings, it has to be split up in individual spans. - The S-57 attribute CATBRG values will be converted into the S-401 attributes bridgeConstruction and categoryOfOpeningBridge as described in the table below: S-57 CATBRG; S-401 bridgeConstruction; S-401 categoryOfOpeningBridge. 1 (fixed Bridge). 3 (swing bridge); 3. 4 (lifting bridge); 4. 5 (bascule bridge); 5. 7 (draw bridge); 7. 9 (footbridge). 10 ???. 11 (aqueduct). 12 suspension bridge); 4. 13 (bridge arch); 1.

Manual conversion

General rules:

- 2.1.4: Units (hunits) - 2.1.5.2: Horizontal Accuracy (HORACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.146 Special Purpose/General Beacon (BCNSPP)
A special purpose beacon is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notices to Mariners.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BCNSHP (1, 2, 3, 4, 5, 6, 7).; [M]. 2.; CATSPM (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56).; [M]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 4.; COLPAT (1, 2, 3, 4, 5, 6).; [O]. 5.; CONDTN (1, 2, 3, 5).; [O]. 7.; OBJNAM (complex attribute "featureName - name"); [O]. 7.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 8.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; HEIGHT; [O]. 10.; INFORM (complex attribute "information - text "); [O]. 10.; NINFOM (complex attribute "information - text"); [O]. 10.; TXTDSC (complex attribute "information - file reference"); [O]. 10.; NTXTDS (complex attribute "information - file reference "); [O]. 12.; MARSYS (1, 2).; [C]. 14.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 14.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 15.; PICREP; [O]. 16.; CONRAD (attribute "radarConspicuous") (3).; [O]. 17.; SORDAT (attribute "reportedDate"); [C]. 18.; SCAMIN; [M]. 19.; SORIND (complex attribute "sourceIndication - source"); [C]. 20.; STATUS (5).; [O]. 22.; VERLEN. 21. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation text"); [O]. NINFOM (complex attribute "topmark - shapeInformation text"); [O]. SCAMIN; [M]. SORDAT; [C]. SORIND; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; beaconShape (1, 2, 3, 4, 5, 6, 7).; [1,1]. 2.; categoryOfSpecialPurposeMark (1, 2, 3, 4, 5, 6, 7, 8, 10, 11, 12, 14, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 60, 61, 62, 63).; [1,*]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; condition (1, 2, 5).; [0,1]. 6.; elevation; [0,1]. 7.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 8.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 9.; height; [0,1]. 10.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 11.; interoperabilityIdentifier; [0,1]. 12.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 13.; natureOfConstruction (1, 2, 6, 7, 8).; [0,*]. 14.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 15.; pictorialRepresentation; [0,1]. 16.; radarConspicuous; [0,1]. 17.; reportedDate; [0,1]. 18.; scaleMinimum; [1,1]. 19.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 20.; status (1, 2, 4, 5, 7, 8, 12, 18).; [0,*]. 21.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 22.; verticalLength; [0,1]. 23.; visualProminence (1, 2, 3).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR) |  |

3.147 Special Purpose/General Buoy (BOYSPP)
A special purpose buoy is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notices to Mariners. For IENCs also buoys to indicate the presence of potentially dangerous obstructions such as groins, banks, or wrecks, buoys to be used as initial marking of a dangerous wreck and supporting pontoons of swinging wire ferries

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; BOYSHP (1, 2, 3, 4, 5, 6, 8).; [M]. 2.; CATSPM (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56).; [M]. 3.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [M]. 4.; COLPAT (1, 2, 3, 4, 5, 6).; [C]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 6.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 6.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 7.; INFORM (complex attribute "information - text "); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; MARSYS (1, 2).; [C]. 10.; mmsico; [O]. 12.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 12.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 13.; PICREP; [O]. 14.; CONRAD (attribute "radarConspicuous") (3).; [O]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 18.; STATUS (5).; [O]. 20.; typatn (1, 2, 3).; [O]. 19. Topmark (TOPMAR). COLOUR (1, 2, 3, 4, 6, 8); [M]. COLPAT (1, 2); [O]. CONDTN (1, 2, 3, 5); [O]. INFORM (complex attribute "topmark - shapeInformation text"); [O]. NINFOM (complex attribute "topmark - shapeInformation text"); [O]. SCAMIN; [M]. SORDAT; [C]. SORIND; [C]. TOPSHP (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33); [M]. | 1.; buoyShape (1, 2, 3, 4, 5, 6, 7, 8).; [1,1]. 2.; categoryOfSpecialPurposeMark (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 60, 61, 62, 63).; [1,*]. 3.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [1,*]. 4.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; marksNavigationalSystemOf (1, 2, 9, 10, 11, 12, 13, 15).; [0,1]. 10.; mMSICode; [0,1]. 11.; natureOfConstruction (6, 7, 8, 11).; [0,*]. 12.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 13.; pictorialRepresentation; [0,1]. 14.; radarConspicuous; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (1, 2, 5, 7, 8, 18).; [0,*]. 19.; topmark; [0,1]. colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. colourPattern (1, 2, 3, 4, 5, 6, 7, 8, 9).; [0,1]. shapeInformation; [0,*]. language; [0,1]. text; [1,1]. topmarkDaymarkShape (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34).; [1,1]. 20.; typeOfAtoN (1, 2, 3).; [0,1]. 21.; verticalLength; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier - 2.6: Radar conspicuous (CONRAD) - 2.7: Topmark (TOPMAR) - 2.8: Lights (LIGHTS) |  |

3.148 Straight Territorial Sea Baseline (STSLNE)
Straight baselines are a system of straight lines joining specified or discrete points on the low-water line, usually known as straight baseline turning points. Straight baselines are used in delimitation.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | curve |
| Attributes | Attributes |
|  | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; nationality; [1,1]. 4.; reportedDate; [0,1]. 5.; scaleMinimum; [1,1]. 6.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.149 Structure Over Navigable Water ()
A roofed structure erected, or partly erected, over a body of water, to provide protection for a vessel or its cargo.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface |
| Attributes | Attributes |
|  | 1.; categoryOfStructure (1, 2, 3, 4, 5).; [0,*]. 2.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 3.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 4.; condition (1, 2, 5).; [0,1]. 5.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 7).; [0,1]. 6.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 7.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 8.; height; [0,1]. 9.; horizontalClearanceFixed; [1,1]. horizontalClearanceValue; [1,1]. horizontalDistanceUncertainty; [0,1]. 10.; horizontalLength; [0,1]. 11.; horizontalWidth; [0,1]. 12.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 13.; interoperabilityIdentifier; [0,1]. 14.; natureOfConstruction (1, 2, 6, 7, 8, 11, 12).; [0,*]. 15.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 16.; pictorialRepresentation; [0,1]. 17.; product (7, 12, 13, 21, 22, 25).; [0,1]. 18.; radarConspicuous; [0,1]. 19.; reportedDate; [0,1]. 20.; scaleMinimum; [1,1]. 21.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 22.; status (1, 4, 5, 7, 8, 12, 14).; [0,*]. 23.; verticalClearanceFixed; [1,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 24.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 25.; verticalLength; [0,1]. 26.; visualProminence (1, 2, 3).; [0,1]. 27.; waterwayDistance; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.150 Submarine Pipeline Area (PIPARE)
An area containing one or more submarine pipelines.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | point surface |
| Attributes | Attributes |
| 1.; CATPIP (2, 3, 4, 6).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information" - text); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PICREP; [O]. 8.; PRODCT (1, 2, 3, 4, 5, 6, 7, 8, 14, 15, 17, 21, 22).; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; RESTRN (1, 38).; [M]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (18).; [C]. | 1.; categoryOfPipelinePipe (2, 3, 4, 5, 6).; [0,*]. 2.; condition (1, 2, 3, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; pictorialRepresentation; [0,1]. 8.; product (1, 2, 3, 7, 8, 18, 19).; [0,*]. 9.; reportedDate; [0,1]. 10.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 38, 39).; [1,*]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 4, 7, 18).; [0,*]. 14.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.151 Terminal (termnl)
A terminal covers that area on shore which provides buildings and constructions for the transfer of cargo or passengers from and to ships.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; cathaf (1, 3, 7, 8, 10, 11).; [M]. 2.; CONDTN (attribute "inDispute" and condition) (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 13.; STATUS (5).; [O]. 14.; trshgd (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [O]. 15.; unlocd; [C]. | 1.; categoryOfHarbourFacility (1, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18).; [1,*]. 2.; condition (1, 2, 5).; [0,*]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; inDispute; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 4, 5, 6, 7, 8, 9, 12, 13, 14, 16, 17).; [0,1]. 14.; transshippingGoods (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11).; [0,*]. 15.; uNLocationCode; [0,1]. |

Conversion rules

Automatic conversion

- The automatic conversion shall change the S-57 attribute CONDTN with value ‘3’ (under reclamation) into the S-401 attribute inDispute as a Boolean with value ‘true’.

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.152 Text Placement ()
The Text Placement feature is used in association with the Feature Name attribute or a light description to optimize text positioning in ECDIS.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point |
| Attributes | Attributes |
|  | 1.; scaleMinimum; [1,1]. 2.; textOffsetBearing; [1,1]. 3.; textOffsetDistance; [1,1]. 4.; textRotation; [0,1]. 5.; textType (1, 2).; [1,2]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.153 Tidal Stream - Flood/Ebb (TS_FEB)
Approximate tidal stream rates given as discrete rate values for flood and ebb flow during springs. For IENCs tThe alternating horizontal movement of water associated with the rise and fall of the tide caused by tide-producing forces. Also called tidal current.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CAT_TS (1, 2, 3).; [M]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; ORIENT (complex attribute "orientation -orientationValue "); [O]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 14.; CURVEL; [O]. | 1.; categoryOfTidalStream (1, 2, 3).; [1,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; orientation; [0,1]. orientationUncertainty; [0,1]. orientationValue; [1,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; speed; [0,1]. speedMaximum; [1,1]. speedMinimum; [0,1]. 13.; speedUnits (1, 2, 3, 4).; [1,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute ORIENT shall be converted into the S-401 complex attribute orientation sub-attribute orientationValue.

Manual conversion

General rules:

- 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.154 Tidal Stream Panel Data (TS_PAD)
Approximate tidal stream characteristics given as discrete value sets at a specified interval before and/or after a high or low water.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point surface |
| Attributes | Attributes |
|  | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 8.; speedUnits (1, 2, 3, 4).; [1,1]. 9.; stationName; [1,1]. 10.; stationNumber; [0,1]. 11.; tidalStreamPanelValues; [1,*]. referenceTide (1, 2).; [1,1]. referenceTideType (1, 2, 3).; [1,1]. streamDepth; [0,1]. tidalStreamValue; [1,*]. orientation; [1,1]. speedMaximum; [1,1]. timeRelativeToTide; [1,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.155 Tideway (TIDEWY)
A channel through which a tidal current runs. A natural water course in intertidal areas where water flows during the ebb or flood can also be encoded as Tideway (TIDEWY) in IENCs.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [O]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; INFORM (complex attribute "information" - text); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; PICREP; [O]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SCAMIN; [M]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.156 Traffic Separation Scheme (C_AGGR)
A routeing measure aimed at the separation of opposing streams of traffic by appropriate means and by the establishment of traffic lanes.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface noGeometry |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [M]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; locmrn; [C]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [O]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; unlocd; [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; iMOAdopted; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; maximumPermittedDraught; [0,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

Manual conversion

- The use of a traffic separation scheme aggregation has not been described in edition 2.6. If the traffic separation schemes should be aggregated, it should be done manually.

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.157 Traffic Separation Scheme Boundary (TSSBND)
The outer limit of a traffic lane part or a traffic separation scheme roundabout.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 1.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; PICREP; [O]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SCAMIN; [M]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. 8.; STATUS (1, 2, 3, 4, 6, 7, 8, 9, 11, 12, 13, 14, 15, 16, 17, 18).; [O]. 9.; CATTSS (1, 2, ).; [O]. 10.; OBJNAM; [O]. 11.; NOBJNM; [O]. | 1.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 8.; status (1, 3, 9, 28).; [0,*]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.158 Traffic Separation Scheme Crossing (TSSCRS)
A defined area where traffic lanes cross.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 1.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 2.; INFORM (complex attribute "information" - text); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; PICREP; [O]. 5.; SORDAT (attribute "reportedDate"); [O]. 6.; RESTRN (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27).; [C]. 7.; SCAMIN; [M]. 8.; SORIND (complex attribute "sourceIndication - source"); [O]. 9.; STATUS (1, 3, 6, 9).; [C]. 11.; CATTSS (1, 2).; [O]. 12.; OBJNAM; [O]. 13.; NOBJNM; [O]. | 1.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; restriction (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27).; [0,*]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 9.; status (1, 3, 6, 9).; [0,*]. 10.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.159 Traffic Separation Scheme Lane Part (TSSLPT)
An area within defined limits in which one-way traffic is established. Natural obstacles, including those forming separation zones, may constitute a boundary.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 1.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "); [O]. 4.; ORIENT; [C]. 5.; PICREP; [O]. 6.; SORDAT (attribute "reportedDate"); [O]. 7.; RESTRN (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27).; [C]. 8.; SCAMIN; [M]. 9.; SORIND (complex attribute "sourceIndication - source"); [O]. 12.; CATTSS (1, 2).; [O]. 13.; OBJNAM; [O]. 14.; NOBJNM; [O]. | 1.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; orientationValue; [0,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; restriction (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 27).; [0,*]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (1, 3, 6, 9, 28).; [0,*]. 11.; vesselSpeedLimit; [0,*]. speedLimit; [1,1]. speedUnits (1, 2, 3, 4).; [1,1]. vesselClass; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.160 Tunnel (TUNNEL)
A passage that is open to the atmosphere at both ends, buried under the seabed or laid over the seafloor or bored under the ground or through mountains.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line area | curve surface |
| Attributes | Attributes |
| 1.; BURDEP; [O]. 2.; CATTEV (4, 5, 6).; [O]. 3.; CONDTN (1, 2, 3, 5).; [O]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; HORACC (complex attribute "horizontalClearanceFixed - horizontalDistanceUncertainty"); [O]. 5.; HORCLR (complex attribute "horizontalClearanceFixed - horizontalClearanceValue"); [C]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 8.; vcrlev; [O]. 9.; PICREP; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 14.; unlocd; [O]. 15.; VERACC (complex attribute "verticalClearanceFixed – verticalUncertainty – uncertaintyFixed"); [O]. 15.; VERCLR (complex attribute "verticalClearanceFixed – verticalClearanceValue"); [C]. 17.; vcrval; [O]. 18.; PEREND; [O]. | 1.; buriedDepth; [0,1]. 2.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 3.; condition (1, 2, 3, 5).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; horizontalClearanceFixed; [0,1]. horizontalClearanceValue; [1,1]. horizontalDistanceUncertainty; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 9.; pictorialRepresentation; [0,1]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (1, 3, 4, 6, 8, 14).; [0,*]. 14.; uNLocationCode; [0,1]. 15.; verticalClearanceFixed; [0,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 16.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 17.; verticalRiverDatumReferenceLevelValue; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 attribute VERACC shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalUncertainty sub-attribute uncertaintyFixed. - The S-57 attribute VERCLR shall be converted into the S-401 complex attribute verticalClearanceFixed sub-attribute verticalClearanceValue.

Manual conversion

General rules:

- 2.1.5.2: Horizontal Accuracy (HORACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.161 Turning Basin (trnbsn)
An area of water or enlargement of a channel used for turning vessels.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [O]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 2.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 3.; HORCLR; [O]. 4.; INFORM (complex attribute "information" - text); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; locmrn; [C]. 6.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 6.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 11.; STATUS (5).; [O]. 12.; unlocd; [C]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; horizontalClearanceValue; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; status (5).; [0,1]. 12.; uNLocationCode; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.162 Two-Way Route ()
A route within defined limits inside which two way traffic is established, aimed at providing safe passage of ships through waters where navigation is difficult or dangerous.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | surface noGeometry |
| Attributes | Attributes |
|  | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; maximumPermittedDraught; [0,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.163 Two-Way Route Part (TWRTPT)
An area of a two-way route within which traffic flow is generally along one bearing (and possibly its reciprocal). For IENCs a two-way route part is either for the entire area, or a part of an area where the traffic flow is restricted to one-way.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; ORIENT; [M]. 7.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; STATUS (1, 3, 6, 9).; [O]. 14.; TRAFIC (1, 2, 3, 4).; [M]. 16.; OBJNAM; [O]. 17.; NOBJNM; [O]. | 1.; basedOnFixedMarks; [0,1]. 2.; depthRangeMinimumValue; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; interoperabilityIdentifier; [0,1]. 6.; orientationValue; [1,1]. 7.; pictorialRepresentation; [0,1]. 8.; qualityOfVerticalMeasurement (1, 2, 3, 4, 6).; [0,*]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (1, 3, 6, 9).; [0,*]. 13.; techniqueOfVerticalMeasurement (1, 2, 3, 5, 6, 8, 9, 10, 11, 12, 13, 13, 15, 16, 17, 18).; [0,*]. 14.; trafficFlow (1, 2, 3, 4).; [1,1]. 15.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.164 Underwater/Awash Rock (UWTROC, uwtroc)
A concreted mass of stony material or coral which dries, is awash or is below the water surface.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 3.; EXPSOU (1, 2, 3).; [C]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; NATSUR (5, 9, 11, 14, 18).; [O]. 8.; PICREP; [O]. 9.; QUASOU (1, 2, 8, 10, 11).; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; VALSOU; [M]. 17.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 18.; WATLEV (1, 2, 3, 4, 5).; [M]. 19.; HORACC; [O]. 1.; CATTEV (4, 5, 6).; [O]. 3.; EXPSOU (1, 2, 3).; [C]. 4.; OBJNAM (complex attribute "featureName - name"); [O]. 4.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; NATSUR (5, 9, 11, 14, 18).; [O]. 8.; PICREP; [O]. 9.; QUASOU (1, 2, 8, 10, 11).; [O]. 10.; SORDAT (attribute "reportedDate"); [C]. 11.; SCAMIN; [M]. 12.; SORIND (complex attribute "sourceIndication - source"); [C]. 16.; VALSOU; [M]. 17.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 18.; watlev (1, 2, 3, 4, 8, 9).; [M]. 19.; HORACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; defaultClearanceDepth; [0,1]. 3.; expositionOfSounding (1, 2, 3).; [0,1]. 4.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; natureOfSurface (5, 9, 11, 14, 18).; [0,1]. 8.; pictorialRepresentation; [0,1]. 9.; qualityOfVerticalMeasurement (1, 2, 3, 4, 6, 7, 8, 9, 10, 11).; [0,*]. 10.; reportedDate; [0,1]. 11.; scaleMinimum; [1,1]. 12.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 13.; status (18).; [0,1]. 14.; surroundingDepth; [1,1]. 15.; techniqueOfVerticalMeasurement (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18).; [0,*]. 16.; valueOfSounding; [1,1]. 17.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 18.; waterLevelEffect (1, 2, 3, 4, 5, 8, 9).; [1,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.1.5.3: Vertical Accuracy (VERACC) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.165 Unsurveyed Area (UNSARE)
An area where hydrographic survey data is non-existent.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; INFORM (complex attribute "information - text "); [O]. 1.; NINFOM (complex attribute "information - text"); [O]. 1.; TXTDSC (complex attribute "information - file reference"); [O]. 1.; NTXTDS (complex attribute "information - file reference "); [O]. 3.; PICREP; [O]. 4.; QUASOU (2, 8).; [C]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SORIND (complex attribute "sourceIndication - source"); [C]. 7.; OBJNAM; [O]. 8.; NOBJNM; [O]. | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; pictorialRepresentation; [0,1]. 4.; qualityOfVerticalMeasurement (2, 8).; [0,*]. 5.; reportedDate; [0,1]. 6.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.166 Update Information ()
The Update Information metadata feature is used to represent a change to the information shown.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
|  | point curve surface noGeometry |
| Attributes | Attributes |
|  | 1.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; scaleMinimum; [0,1]. 5.; source; [0,1]. 6.; updateNumber; [1,1]. 7.; updateType (1, 2, 3, 4).; [1,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |

3.167 Vegetation (VEGATN)
Plants collectively or individually, especially those dominating a particular area or habitat.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point curve surface |
| Attributes | Attributes |
| 1.; CATVEG (6, 11, 13).; [M]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; PICREP; [O]. 8.; SORDAT (attribute "reportedDate"); [C]. 9.; SCAMIN; [M]. 10.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; CONVIS (attribute "visualProminence") (1, 2).; [O]. | 1.; categoryOfVegetation (3, 4, 5, 6, 11, 13, 14, 15, 16, 17, 18, 19, 20, 22).; [1,1]. 2.; elevation; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; height; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; pictorialRepresentation; [0,1]. 8.; reportedDate; [0,1]. 9.; scaleMinimum; [1,1]. 10.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 11.; verticalLength; [0,1]. 12.; visualProminence (1, 2, 3).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.168 Vehicle Transfer (vehtrf)
A place where vehicles can be loaded or unloaded from the inland vessel with onboard or on-shore facilities.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; catvtr (1, 2, 3, 4, 5, 6).; [M]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; HEIGHT; [M]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; locmrn; [C]. 8.; vcrlev; [O]. 9.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 9.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 10.; PICREP; [O]. 11.; SORDAT (attribute "reportedDate"); [C]. 12.; SCAMIN; [M]. 13.; SORIND (complex attribute "sourceIndication - source"); [C]. 14.; STATUS (5).; [O]. 15.; unlocd; [C]. 16.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [O]. 17.; vcrval; [O]. | 1.; categoryOfVehicleTransfer (1, 2, 3, 4, 5, 6).; [1,*]. 2.; condition (1, 2, 5).; [0,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; height; [1,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; interoperabilityIdentifier; [0,1]. 8.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 9.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 10.; pictorialRepresentation; [0,1]. 11.; reportedDate; [0,1]. 12.; scaleMinimum; [1,1]. 13.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 14.; status (5, 18).; [0,1]. 15.; uNLocationCode; [0,1]. 16.; verticalDatum (10, 12, 23, 24, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45).; [0,1]. 17.; verticalRiverDatumReferenceLevelValue; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.169 Vertical Datum of Data (m_vdat)
Any level surface (for example Mean Sea Level) taken as a surface of reference to which the elevations within a data set are reduced. Also called datum level, reference level, reference plane, levelling datum, datum for heights. (Adapted from IHO Dictionary - S-32). The Vertical Datum of Data for inland waterways is in most cases not a horizontal plane but a sloping plane.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 3.; SORDAT (attribute "reportedDate"); [C]. 4.; SORIND (complex attribute "sourceIndication - source"); [C]. 5.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [M]. | 1.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 2.; interoperabilityIdentifier; [0,1]. 3.; reportedDate; [0,1]. 4.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 5.; verticalDatum (3, 10, 12, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [1,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.5: InteroperabilityIdentifier |  |

3.170 Vessel Traffic Service Area (ADMARE, VTS Area)
The area of any service implemented by a relevant authority primarily designed to improve safety and efficiency of traffic flow and the protection of the environment. It may range from simple information messages, to extensive organisation of the traffic involving national or regional schemes.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; OBJNAM (complex attribute "featureName - name"); [M]. 1.; NOBJNM (complex attribute "featureName - name"); [O]. 2.; INFORM (complex attribute "information - text "); [O]. 2.; NINFOM (complex attribute "information - text"); [O]. 2.; TXTDSC (complex attribute "information - file reference"); [O]. 2.; NTXTDS (complex attribute "information - file reference "). 4.; PICREP; [O]. 5.; SORDAT (attribute "reportedDate"); [C]. 6.; SCAMIN; [M]. 7.; SORIND (complex attribute "sourceIndication - source"); [C]. 8.; CONDTN (3).; [C]. 9.; JRSDTN (1, 2, 3).; [M]. 10.; NATION; [M]. | 1.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 2.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 3.; interoperabilityIdentifier; [0,1]. 4.; pictorialRepresentation; [0,1]. 5.; reportedDate; [0,1]. 6.; scaleMinimum; [1,1]. 7.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

Conversion rules

Automatic conversion

- The S-401 Feature type Vessel Traffic Service Area has been introduced in S-401 as a dedicated feature to encode such areas. This information is encoded in S-57 using the Object class ADMARE. In order for this information to be converted across to S-401, the text string encoded in INFORM on the ADMARE should be in a standardised format, such as Vessel traffic service area. Data Producers are advised to examine any Vessel Traffic Service Area features created during the automated conversion process and confirm the attribution for these features as required, including any values populated for the complex attribute information. If INFORM is encoded in this way, ADMARE must be converted according into a vessel traffic service area.

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.171 Water Turbulence (WATTUR)
The disturbance of water caused by the interaction of any combination of waves, currents, tidal streams, wind, shoal patches and obstructions.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point curve surface |
| Attributes | Attributes |
| 1.; CATWAT (6).; [M]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information - text "); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 3.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; PICREP; [O]. 6.; SORDAT (attribute "reportedDate"); [C]. 7.; SCAMIN; [M]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfWaterTurbulence (1, 2, 3, 4, 5, 6).; [1,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.172 Waterway Area (wtware)
An area in which uniform general information of the waterway exists.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| area | surface |
| Attributes | Attributes |
| 1.; catccl (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12).; [M]. 2.; dirimp (1, 2, 3, 4).; [M]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; INFORM (complex attribute "information - text "); [O]. 5.; NINFOM (complex attribute "information - text"); [O]. 5.; TXTDSC (complex attribute "information - file reference"); [O]. 5.; NTXTDS (complex attribute "information - file reference "); [O]. 6.; locmrn; [C]. 7.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 7.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 8.; PICREP; [O]. 9.; SORDAT (attribute "reportedDate"); [C]. 10.; SCAMIN; [M]. 11.; SORIND (complex attribute "sourceIndication - source"); [C]. 12.; STATUS (5).; [O]. 13.; unlocd; [C]. | 1.; categoryOfCEMTClass (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12).; [1,*]. 2.; directionOfImpact (1, 2, 3, 4, 5).; [1,*]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 6.; interoperabilityIdentifier; [0,1]. 7.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 8.; pictorialRepresentation; [0,1]. 9.; reportedDate; [0,1]. 10.; scaleMinimum; [1,1]. 11.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 12.; status (5).; [0,1]. 13.; uNLocationCode; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.173 Waterway Axis (wtwaxs)
The waterway axis can be defined as, for example: - The middle line of a fairway, (Definition of fairway: That part of a river, harbour; etc. where the main navigable channel for vessels of larger size lies. It is also the usual course followed by vessels entering or leaving harbours, called "ship channel". (International Maritime Dictionary, 2nd Ed.)). - The middle line of a water way (Definition of waterway: The waterway covers the entire area of a river or canal).

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; catccl (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12).; [O]. 2.; OBJNAM (complex attribute "featureName - name"); [M]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 3.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 4.; INFORM (complex attribute "information - text "); [O]. 4.; NINFOM (complex attribute "information - text"); [O]. 4.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 5.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 6.; PICREP; [O]. 7.; SORDAT (attribute "reportedDate"); [C]. 8.; SCAMIN; [M]. 9.; SORIND (complex attribute "sourceIndication - source"); [C]. 10.; STATUS (5).; [O]. | 1.; categoryOfCEMTClass (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12).; [0,*]. 2.; featureName; [1,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 4.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 5.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 6.; pictorialRepresentation; [0,1]. 7.; reportedDate; [0,1]. 8.; scaleMinimum; [1,1]. 9.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 10.; status (5).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.174 Waterway Gauge (wtwgag)
An instrument for measuring water levels.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; catgag (1, 2, 3, 4, 5).; [O]. 2.; CONDTN (1, 2, 3, 5).; [O]. 3.; disipd; [O]. 4.; disipu; [O]. 5.; hunits (1, 2, 3, 4, 5, 6).; [O]. 6.; ELEVAT; [O]. 7.; OBJNAM (complex attribute "featureName - name"); [C]. 7.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 8.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 9.; INFORM (complex attribute "information - text "); [O]. 9.; NINFOM (complex attribute "information - text"); [O]. 9.; TXTDSC (complex attribute "information - file reference"); [O]. 9.; NTXTDS (complex attribute "information - file reference "); [O]. 10.; locmrn; [C]. 11.; othnam; [O]. 12.; hignam; [O]. 13.; lownam; [O]. 14.; meanam; [O]. 15.; sdrlev; [O]. 16.; vcrlev; [O]. 17.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 17.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 18.; PICREP; [O]. 19.; reflev (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [O]. 20.; SORDAT (attribute "reportedDate"); [C]. 21.; SCAMIN; [M]. 22.; sdrval; [O]. 23.; SORIND (complex attribute "sourceIndication - source"); [C]. 24.; STATUS (5).; [O]. 25.; unlocd; [C]. 26.; othwat; [O]. 27.; higwat; [O]. 28.; lowwat; [O]. 29.; meawat; [O]. 30.; verdat (12, 23, 24, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [O]. 31.; vcrval; [O]. 32.; wtwdis; [O]. | 1.; categoryOfWaterwayGauge (1, 2, 3, 4, 5).; [0,1]. 2.; condition (1, 2, 5).; [0,1]. 3.; distanceOfImpactDownstream; [0,1]. 4.; distanceOfImpactUpstream; [0,1]. 5.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 6, 7).; [0,1]. 6.; elevation; [0,1]. 7.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 8.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 9.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 10.; interoperabilityIdentifier; [0,1]. 11.; nameOfOtherLocallyRelevantWaterLevel; [0,1]. 12.; nameOfRelevantHighWaterLevel; [0,1]. 13.; nameOfRelevantLowWaterLevel; [0,1]. 14.; nameOfRelevantMeanWaterLevel; [0,1]. 15.; nameOfSoundingDatumReferenceLevel; [0,1]. 16.; nameOfVerticalRiverDatumReferenceLevel; [0,1]. 17.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 18.; pictorialRepresentation; [0,1]. 19.; referenceGravitationalLevel (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [0,1]. 20.; reportedDate; [0,1]. 21.; scaleMinimum; [1,1]. 22.; soundingDatumReferenceLevelValue; [0,1]. 23.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 24.; status (5, 18).; [0,1]. 25.; uNLocationCode; [0,1]. 26.; valueAtOtherLocallyRelevantWaterLevel; [0,1]. 27.; valueAtRelevantHighWaterLevel; [0,1]. 28.; valueAtRelevantLowWaterLevel; [0,1]. 29.; valueAtRelevantMeanWaterLevel; [0,1]. 30.; verticalDatum (10, 12, 23, 24, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45).; [0,1]. 31.; verticalRiverDatumReferenceLevelValue; [0,1]. 32.; waterwayDistance; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.175 Waterway Profile (wtwprf)
A physically non-existent line which is normally the connection of two opposite distance marks. Waterway profiles can be used to define a special water level.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| line | curve |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; hunits (1, 2, 3, 5, 6).; [M]. 3.; OBJNAM (complex attribute "featureName - name"); [O]. 3.; NOBJNM (complex attribute "featureName - name"); [O]. 4.; DATEND (complex attribute "fixedDateRange - dateEnd"); [O]. 4.; DATSTA (complex attribute "fixedDateRange - dateStart"); [O]. 5.; HEIGHT; [C]. 6.; INFORM (complex attribute "information - text "); [O]. 6.; NINFOM (complex attribute "information - text"); [O]. 6.; TXTDSC (complex attribute "information - file reference"); [O]. 6.; NTXTDS (complex attribute "information - file reference "); [O]. 7.; sdrlev; [O]. 8.; PEREND (complex attribute "periodicDateRange - dateEnd"); [O]. 8.; PERSTA (complex attribute "periodicDateRange - dateStart"); [O]. 9.; PICREP; [O]. 10.; reflev (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [C]. 11.; SORDAT (attribute "reportedDate"); [C]. 12.; SCAMIN; [M]. 13.; sdrval; [O]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; STATUS (5).; [O]. 16.; verdat (12, 23, 24, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 45).; [C]. 17.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 18.; wtwdis; [M]. 19.; HORACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; distanceUnitOfMeasurement (1, 2, 3, 4, 5, 7).; [1,1]. 3.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 4.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 5.; height; [0,1]. 6.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 7.; nameOfSoundingDatumReferenceLevel; [0,1]. 8.; periodicDateRange; [0,*]. dateEnd; [1,1]. dateStart; [1,1]. 9.; pictorialRepresentation; [0,1]. 10.; referenceGravitationalLevel (1, 2, 3, 4, 5, 6, 7, 8, 9, 10).; [0,1]. 11.; reportedDate; [0,1]. 12.; scaleMinimum; [1,1]. 13.; soundingDatumReferenceLevelValue; [0,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (5).; [0,1]. 16.; verticalDatum (10, 12, 23, 24, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 45).; [0,1]. 17.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 18.; waterwayDistance; [1,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.1.5.3: Vertical Accuracy (VERACC) - 2.2.2: Periodic date range, fixed date range, survey date range (DATSTA, DATEND, PERSTA, PEREND) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.176 Weed/Kelp (WEDKLP)
Any macroscopic marine alga.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATWED (1, 2, 3, 4).; [M]. 2.; OBJNAM (complex attribute "featureName - name"); [O]. 2.; NOBJNM (complex attribute "featureName - name"); [O]. 3.; INFORM (complex attribute "information" - text); [O]. 3.; NINFOM (complex attribute "information - text"); [O]. 3.; TXTDSC (complex attribute "information - file reference"); [O]. 4.; NTXTDS (complex attribute "information - file reference "); [O]. 5.; PICREP; [O]. 6.; SORDAT (attribute "reportedDate"); [C]. 7.; SCAMIN; [M]. 8.; SORIND (complex attribute "sourceIndication - source"); [C]. | 1.; categoryOfWeedKelp (1, 2, 4).; [1,1]. 2.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 3.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 4.; interoperabilityIdentifier; [0,1]. 5.; pictorialRepresentation; [0,1]. 6.; reportedDate; [0,1]. 7.; scaleMinimum; [1,1]. 8.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |

3.177 Wind Turbine (LNDMRK (CATLMK = 19))
A tower and associated equipment that generates electrical power from wind. They can be sited offshore and may be either fixed or floating.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point | point |
| Attributes | Attributes |
| 1.; COLOUR (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [O]. 3.; CONDTN (1, 2, 3, 4, 5).; [O]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 8.; INFORM (complex attribute "information - text "); [O]. 8.; NINFOM (complex attribute "information - text"); [O]. 8.; TXTDSC (complex attribute "information - file reference"); [O]. 8.; NTXTDS (complex attribute "information - file reference "); [O]. 13.; PICREP; [O]. 15.; SORDAT (attribute "reportedDate"); [C]. 16.; SCAMIN; [M]. 17.; SORIND (complex attribute "sourceIndication - source"); [C]. 21.; VERLEN; [O]. 22.; CONVIS (attribute "visualProminence") (1).; [M]. 24.; CATLMK (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21).; [M]. 25.; FUNCTN (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42).; [C]. | 1.; colour (1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13).; [0,*]. 2.; colourPattern (1, 2, 3, 4, 5, 6).; [0,1]. 3.; condition (1, 2, 3, 4, 5).; [0,1]. 4.; elevation; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; fixedDateRange; [0,1]. dateEnd; [0,1]. dateStart; [0,1]. 7.; height; [0,1]. 8.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 9.; interoperabilityIdentifier; [0,1]. 10.; inTheWater; [0,1]. 11.; multiplicityOfFeatures; [0,1]. multiplicityKnown; [1,1]. numberOfFeatures; [0,1]. 12.; natureOfConstruction (2, 6, 7, 8, 11).; [0,*]. 13.; pictorialRepresentation; [0,1]. 14.; radarConspicuous; [0,1]. 15.; reportedDate; [0,1]. 16.; scaleMinimum; [1,1]. 17.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 18.; status (1, 2, 4, 5, 7, 8, 12, 13, 14, 28).; [0,*]. 19.; verticalClearanceFixed; [0,1]. verticalClearanceValue; [1,1]. verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 20.; verticalDatum (3, 10, 12, 13, 16, 17, 18, 19, 20, 21, 23, 24, 25, 26, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 43, 44, 45).; [0,1]. 21.; verticalLength; [0,1]. 22.; visualProminence (1, 2, 3).; [0,1]. 23.; waterLevelEffect (2, 7).; [0,1]. |

Conversion rules

Automatic conversion

- The S-57 object class LANDMRK with the attribute CATLMK value 19 (windmotor) shall be converted into the S-401 feature type Wind Turbine with the corresponding attributes.

Manual conversion

General rules:

- 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier

3.178 Wreck (WRECKS)
The ruined remains of a stranded or sunken vessel which has been rendered useless.

| S-57 Edition 2.6 | S-401 Edition 1.2.0 |
| --- | --- |
| Primitives | Primitives |
| point area | point surface |
| Attributes | Attributes |
| 1.; CATTEV (4, 5, 6).; [O]. 2.; CATWRK (1, 2, 3, 4, 5).; [M]. 4.; EXPSOU (1, 2, 3).; [C]. 5.; OBJNAM (complex attribute "featureName - name"); [O]. 5.; NOBJNM (complex attribute "featureName - name"); [O]. 7.; INFORM (complex attribute "information - text "); [O]. 7.; NINFOM (complex attribute "information - text"); [O]. 7.; TXTDSC (complex attribute "information - file reference"); [O]. 7.; NTXTDS (complex attribute "information - file reference "); [O]. 9.; PICREP; [O]. 10.; QUASOU (1, 2, 8, 10, 11).; [O]. 12.; SORDAT (attribute "reportedDate"); [C]. 13.; SCAMIN; [M]. 14.; SORIND (complex attribute "sourceIndication - source"); [C]. 15.; STATUS (12, 16, 17, 18).; [O]. 17.; TECSOU (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15).; [O]. 18.; VALSOU; [C]. 19.; VERACC (complex attribute "verticalUncertainty – uncertaintyFixed"); [O]. 21.; WATLEV (1, 2, 3, 4, 5).; [O]. 22.; HORACC; [O]. | 1.; categoryOfTemporalVariation (4, 5, 6).; [0,1]. 2.; categoryOfWreck (1, 2, 3, 4, 5).; [1,1]. 3.; defaultClearanceDepth; [0,1]. 4.; expositionOfSounding (1, 2, 3).; [0,1]. 5.; featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. 6.; height; [0,1]. 7.; information; [0,*]. fileLocator; [0,1]. fileReference; [0,1]. headline; [0,1]. language; [1,1]. text; [0,1]. 8.; interoperabilityIdentifier; [0,1]. 9.; pictorialRepresentation; [0,1]. 10.; qualityOfVerticalMeasurement (1, 2, 3, 4, 6, 7, 8, 9, 10, 11).; [0,*]. 11.; radarConspicuous; [0,1]. 12.; reportedDate; [0,1]. 13.; scaleMinimum; [1,1]. 14.; sourceIndication; [0,1]. categoryOfAuthority (2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16).; [0,1]. countryName; [0,1]. featureName; [0,*]. language; [1,1]. name; [1,1]. nameUsage (1, 2).; [0,1]. reportedDate; [0,1]. source; [1,*]. sourceType (1, 2, 7, 8, 9, 10, 11, 12, 13, 14).; [0,1]. 15.; status (7, 12, 13, 16, 17, 18).; [0,*]. 16.; surroundingDepth; [1,1]. 17.; techniqueOfVerticalMeasurement (1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 15, 16, 17, 18).; [0,*]. 18.; valueOfSounding; [0,1]. 19.; verticalUncertainty; [0,1]. uncertaintyFixed; [1,1]. uncertaintyVariableFactor; [0,1]. 20.; visualProminence (1, 2, 3).; [0,1]. 21.; waterLevelEffect (1, 2, 3, 4, 5).; [1,1]. |

| Conversion rules |  |
| --- | --- |
| Automatic conversion |  |
| Manual conversion |  |
| General rules: |  |
| - 2.1.5.3: Vertical Accuracy (VERACC) - 2.3: Source information (SORDAT, SORIND) - 2.4: Textual information (INFORM, NINFOM, OBJNAM, NOBJNM, TXTDSC, NTXTDS) - 2.5: InteroperabilityIdentifier |  |