## Options regarding the transition from the S-57 based IENC Product Specifications to S-401

This document describes the theoretical possibilities for the transition from S-57 based Inland ENC Product Specifications (all up to 2.6) to the S-100 based Product Specification for Inland ENCs S-401.

All scenarios have to take two parts into account:

a)	the transition of the chart production from 2.4, 2.5 or 2.6 to S-401;

b)	the transition from S-57 based Inland ECDIS or ECS applications to S-401 based applications or dual fuel applications.

### The transition in the maritime area

IHO and IMO have agreed on a dual‑fuel period, during which both S‑57 and S‑101 ENCs may be used in parallel. This ensures that:

- Hydrographic offices can continue producing S‑57 while building S‑101 capability.

- ECDIS manufacturers can update systems without disrupting the use by mariners.

- Mariners can safely operate with mixed fleets and mixed chart types.

a)	Transition in the maritime chart production

Hydrographic Offices

- Continue producing S‑57 ENCs

- Begin producing S‑101 ENCs in 2026 (often via automated conversion + manual refinement)

- Validate S‑101 through RENCs

- Maintain both datasets until S‑57 retirement (date not set, but at least for 10 years)

b)	Transition for maritime ECDIS applications

ECDIS Manufacturers

- Release S‑100‑capabl ECDIS starting in 2026

- Support both S‑57 and S‑101 simultaneously

Mariners

- Use S‑57 or S‑101 depending on vessel equipment in the dual fuel period

- Receive training on new S‑101 features

- Experience improved data richness and consistency over time

Summary of the transition in the maritime area

During a dual fuel period of at least 10 years

- Chart producers are encouraged to provide S-57 based ENCs and S-101 based ENCs from 2026 onward. Chart production tools and conversion tools are already available. There is no fixed date until which a full coverage with S-101 ENCs has to be provided. This will be decided later by IHO and IMO.

- Application providers and users can still use S-57 based ECDIS until an end date for the provision of S-57 ENCs which is not fixed. They can also use dual fuel ECDIS, but dual fuel ECDIS is not yet on the market. An S-101 based ECDIS without dual fuel capacity can only be used when all Hydrographic Offices have converted all their S-57 ENCs and published a complete coverage with S-101 ENCs.

- There is a dual fuel period for both sides.
Advantage: smooth transition with a lot of flexibility
Disadvantage: chart producers have to produce S-57 and S-101 in the dual fuel period and application providers have to provide dual fuel ECDIS in the dual fuel period, but the dual fuel ECDIS will not be needed any more after the end of the dual fuel period.

### Scenarios for inland navigation

### 1.	Hard cut

a)	Effects of a hard cut on chart producers

Chart producers have to provide S-57 based IENCs until a fixed date. Before that date, they have to produce a complete coverage with S-401 IENCs that is published on the fixed date. There is no period in which they have to produce updates for both versions.

b)	Effects of a hard cut on Inland ECDIS or ECS applications

Boatmasters have to use S-57 based Inland ECDIS or ECS until the fixed end date. Ideally, they would switch to S-401 based Inland ECDIS or ECS at the fixed date. It will however not be possible that the whole inland fleet replaces their Inland ECDIS or ECS at the same date. S-57 based Inland ECDIS or ECS would still work after the fixed date, but would not receive any updates any more.

This could also cause juridical problems, because users who do not obtain a S-401 based Inland ECDIS or ECS in time would be obliged to use outdated IENCs.

### 2.	Dual fuel period only for Inland ENCs

a)	Effects of a dual fuel period for Inland ENCs on the chart producers

Chart producers have to provide IENCs and their updates in S-57 based format and in S-401 format during a transitional period. This causes a higher workload for the chart producers.

The suppliers of chart production software recommend to amend the base data in the GIS system to be in conformity with the requirements of 2.6 and S-401. The two IENCs can then be produced with the push of a button. This approach would only cause minimal additional workload for the publication, because the manual addition of data is only done once (as in the hard cut scenario).

Chart producers that do not use such a chart production tool, would have to convert their IENCs 2.6 to S-401 with a conversion tool after each update (or the other way round). This will however not be possible without manual interaction and would create a lot of additional workload during the dual fuel period.

b)	Effects of a dual fuel period for Inland ENCs on Inland ECDIS applications

Users could either use S-57 based Inland ECDIS or ECS or S-401 based Inland ECDIS or ECS in the dual fuel period. Manufacturers would not be forced to provide dual fuel applications. But at the end of the dual fuel period all vessels have to be equipped with S-401 based Inland ECDIS or ECS.

This scenario would be the most convenient and cheapest for the users.

### 3.	Dual fuel period only for the Inland ECDIS or ECS applications

a)	Effects of a dual fuel period for Inland ECDIS or ECS applications on chart producers

Chart producers would be allowed to switch from S-57 based IENCs to S-401 IENCs during the dual fuel period whenever they want. They could gradually convert their charts and would not have to provide updates in two formats.

This scenario would be the most convenient and cheapest for the chart producers.

b)	Effects of a dual fuel period for Inland ECDIS or ECS applications on applications

All users would have to use dual fuel Inland ECDIS or ECS during the whole dual fuel period. Before the start of the dual fuel period the Inland ECDIS or ECS would only use S-57 based IENCs, in the dual fuel period it would use S-57 based IENCs and S-401 IENCs depending on their availability and after the dual fuel period they would only use S-401 IENCs.

### 4.	Dual fuel for both sides (as in the maritime area)

a)	Effects of a dual fuel period for both sides on chart producers

Chart producers have to provide IENCs and their updates in S-57 based format and in S-401 format during a transitional period. This causes a higher workload for the chart producers.

The suppliers of chart production software recommend to amend the base data in the GIS system to be in conformity with the requirements of 2.6 and S-401. The two IENCs can then be produced with the push of a button. This approach would only cause minimal additional workload for the publication, because the manual addition of data is only done once (as in the hard cut scenario).

Chart producers that do not use such a chart production tool, would have to convert their IENCs 2.6 to S-401 with a conversion tool after each update (or the other way round). This will however not be possible without manual interaction and would create a lot of additional workload during the dual fuel period.

b)	Effects of a dual fuel period for both sides on applications

All users would have to use dual fuel Inland ECDIS or ECS during the whole dual fuel period. Before the start of the dual fuel period the Inland ECDIS or ECS would only use S-57 based IENCs, in the dual fuel period it would use S-57 based IENCs and S-401 IENCs depending on their availability and after the dual fuel period they would only use S-401 IENCs.

A dual fuel period for both sides has negative effects on both sides, but provides the highest flexibility and does not require a political decision that the load of the transition has to be carried mainly by the chart producers or mainly by the users.

### Overall summary and comparison of the options

| Option | Pros | Cons |
| --- | --- | --- |
| Hard cut | Cheapest scenario for chart producers No dual fuel Inland ECDIS needed | Replacement of all existing S-57 based Inland ECDIS or ECS with S-401 based Inland ECDIS or ECS on a specific date is not possible. Users would not have access to updates until they have replaced their Inland ECDIS or ECS. |
| Dual fuel period only for Inland ENCs | Most convenient and cheapest scenario for the users | More workload for chart producers, especially for those who do not use a GIS based IENC production software |
| Dual fuel period only for the Inland ECDIS or ECS applications | Cheapest scenario for chart producers | All users need dual fuel Inland ECDIS or ECS |
| Dual fuel for both sides | Same approach as in maritime area, flexible. No political decision for one side that has to bear the costs. | More workload for chart producers, especially for those who do not use a GIS based IENC production software All users need dual fuel Inland ECDIS or ECS |